| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IEEE 802.16m Rapporteur Group for DL MIMO – Final Report |  |
| Date Submitted | 2008-07-11 |  |
| Source(s) | Minnie Ho, Jerry (Zhouyue) Pi, Yong Sun DL MIMO Rapporteur Group Chairs | minnie.ho@intel.com zpi@sta.samsung.com sun@toshiba-trel.com |
| Re: | IEEE 802.16m-08/023, Charter and Scope of New TGm Rapporteur Groups (DL MIMO Rapporteur Group ) |  |
| Abstract | Final Report – TGm DL MIMO Rapporteur Group activity |  |
| Purpose | For discussion in TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

IEEE 802.16m Rapporteur Group for DL MIMO – Final Report

Minnie Ho, Jerry Pi, Yong Sun

DL MIMO Rapporteur Group Chairs

# Objectives

During TGm session#55, hosted in Macau, the DL MIMO Rapporteur Group was created, and its scope together with its process was defined in [6] C80216m-DL MIMO-08/008.doc (3rd draft) as the following:

“The DL MIMO [“DL MIMO”] Rapporteur Group is chartered to develop proposed baseline content regarding the Downlink MIMO structure suitable for use in the 802.16m System Description Document (SDD). It shall be submitted by the Rapporteur Group as a Task Group m contribution by 7 July 2008, with the expectation that it could be accepted by Task Group m at Session #56. The Rapporteur Group shall also submit a report of its activities as a TGm contribution by the same deadline. Rapporteur Group Chairs are Minnie Ho, Jerry Pi, and Yong Sun.”

The DL MIMO RG Chairs thus decided to propose recommendations for evaluation criteria to facilitate narrowing down options, and thus develop harmonized SDD text for Project 802.16m DL MIMO functions initially based on proposals submitted in Session #55.

Since DL MIMO Pilot and control aspects were being handled by other e-mail threads resulting from the DL_PHY, DL_CTL, and UL_CTL groups, the focus of the DL MIMO RG was on MIMO for data, while pilot, downlink signaling, and some control aspects were not included in the final report.

# Scope

Identify areas where consensus can be built through discussions in the Rapporteur Group and outline potential options and leave unresolved issues to TGm for consideration. Final down selection of proposed text will be made by TGm in Session #56.

# Summary of Activities

Following session #55, the Rapporteur Group chairs generated an initial draft SDD text for DL MIMO functions, together with shared a clear and detailed work plan document Error: Reference source not found.

The Rapporteur Group chairs provided an excel-sheet Error: Reference source not found containing the list of contributions that were submitted in Session #55 with content related to DL MIMO and their key features on 5/16/2008.

- The Rapporteur Group chairs provided a separate template Error: Reference source not found to capture major impacts of the proposed schemes on certain functionalities and features such as antenna configuration, diversity scheme, etc., to be filled by the proponents on 5/16/2008.

- Authors were requested to send the Rapporteur Group chairs the latest revision of the submitted contributions, if proposed text and associated figures were not submitted in MS Word or PowerPoint format.

- The contributors were required to update the excel sheet Error: Reference source not found and return their updates by 5/21/2008. The files were uploaded to the TGm member upload facility at http://dot16.org/memberupload/TGm_DL MIMO. Inputs from all the proponents were consolidated and provided in Error: Reference source not found at the DL MIMO upload facility.

- The Rapporteur Group chairs provided the first working draft Error: Reference source not found on 5/26/08 based on the member contributions and updates.

- Members were encouraged to express their views and to suggest changes to the proposed draft through discussions on the 802.16m Google group email reflector between 5/27/2008 and 6/2/2008.

- An updated working draft was created where proposed text from contributions was merged by Rapporteur Group chairs wherever consensus was observed. The updated draft [3] was available to the Rapporteur Group on 6/6/2008.

- Members were encouraged to express their views and to suggest changes to the proposed draft through discussions on the 802.16m Google group email reflector between 6/7/2008 and 6/16/2008.

- An updated working draft was created where proposed text from contributions was merged by Rapporteur Group chairs wherever consensus was observed. The updated draft [3] was available to the Rapporteur Group on 6/20/2008.

- Members were encouraged to continue discussions and to express their views on the reflector between 6/21/2008 to 6/30/2008 with the goal of reducing the options for the purpose of decision making of TGm in the July meeting and consensus building on DL MIMO design criteria.

- The Rapporteur Group chairs submitted the final draft SDD text Error: Reference source not found as a contribution to TGm on Monday 7/7/2008.

- A call for comments on the contribution from the Rapporteur Group was issued with a deadline of 12:00 PM US Mountain Daylight Saving Time (Denver time) on Monday 07/14/2008 and where the comments would be resolved in TGm Session #56.

Timeline of major milestones during DL MIMO Rapporteur Group

# References

[1]	C80216m-DL MIMO-08/001.doc

[2]	C80216m-DL MIMO-08/002r1.doc

[3]	C80216m-DL MIMO-08/003r2.xls

[4]	C80216m-DL MIMO-08/004.doc (1st draft)

[5]	C80216m-DL MIMO-08/005.doc (2nd draft)

[6]	C80216m-DL MIMO-08/008.doc (3rd draft)

[7]	IEEE 802.16m-08/023

[8]	C80216m-08/657r2

# Appendix A

This appendix gives the table containing full list of DL MIMO contributions from Session#55, taken into account for initiating the SDD text discussions. Further details can be found in the reference Error: Reference source not found.

| Contribution Number | 1st Author |
| --- | --- |
| C80216m-08_273 | Fred Vook |
| C80216m-08_281 | Ryohei Kimura |
| C80216m-08_282 | Takaaki Kishigami |
| C80216m-08_285r2 | David Mazzarese |
| C80216m-08_302r3 | Chung-Lien Ho |
| C80216m-08_309r2 | Yu-Tao Hsieh |
| C80216m-08_312r2 | Chien-Yu Kao |
| C80216m-08_314r2 | Jen-Yuan Hsu |
| C80216m-08_326r1 | Chih-Yuan Lin |
| C80216m-08_331r1 | Vinosh James |
| C80216m-08_333 | Mohammad Khojastepour |
| C80216m-08_337 | Jeff Tao |
| C80216m-08_342r1 | Sophie Vrzic |
| C80216m-08_343 | Dongsheng Yu |
| C80216m-08_345r2 | Robert Novak |
| C80216m-08_346 | Sophie Vrzic |
| C80216m-08_347 | Jun Yuan |
| C80216m-08_349 | Dongsheng Yu |
| C80216m-08_361 | Kaushik Josiam |
| C80216m-08_366 | Tsuguhide Aoki |
| C80216m-08_370 | Jerry Pi |
| C80216m-08_372r1 | Ron Porat |
| C80216m-08_374r1 | Choong Il Yeh |
| C802.16m-08_379 | Dongjun Lee |
| C80216m-08_387 | Hua Zhou |
| C80216m-08_388 | Yuantao Zhang |
| C80216m-08_390 | Jie Zhang |
| C80216m-08_392 | Hua Zhou |
| C80216m-08_397 | Li Shaohua |
| C80216m-08_399 | Li Shaohua |
| C80216m-08_405r1 | Guangjie Li |

| Contribution Number | 1st Author |
| --- | --- |
| C80216m-08_419r1 | Young Kwon |
| C80216m-08_420r2 | Young Kwon |
| C80216m-08_421r1 | Yang Tang |
| C80216m-08_422r1 | Tao Wu |
| C80216m-08_423 | Keying Wu |
| C80216m-08_424 | Wookbong Lee |
| C80216m-08_426r1 | Moon-il Lee |
| C80216m-08_428 | Wookbong Lee |
| C80216m-08_429 | Sunam Kim |
| C80216m-08_430 | Dongguk Lim |
| C80216m-08_431 | Jaewan Kim |
| C80216m-08_435 | Keying Wu |
| C80216m-08_449 | Mythri Hunukumbure |
| C80216m-08_451r4 | Wooram Shin |
| C80216m-08_452r1 | Hao Donglai |
| C80216m-08_457 | Hongwei Yang |
| C80216m-08_458r1 | Hao Donglai |
| C80216m-08_461r1 | Hongwei Yang |
| C80216m-08_462r1 | Yinggang Du |
| C80216m-08_463 | Yinggang Du |
| C80216m-08_464 | Yinggang Du |
| C80216m-08_465 | Yinggang Du |
| C80216m-08_466 | Martin Liu |
| C80216m-08_467 | Mattias Wennstroem |
| C80216m-08_468 | Martin Liu |
| C80216m-08_481r2 | Young Seog Song |
| C80216m-08_484 | Seung Joon Lee |
| C80216m-08_487 | Stephan Saur |
| C80216m-08_488 | Joerg Schaepperle |

Table 1: List of DL MIMO contributions from Session #55