| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on Uplink Ranging Channel |  |
| Date Submitted | 2008-07-14 |  |
| Source(s) | Xin Chang, Hongjie Si, Juejun Liu, Yunsong Yang Huawei HyunWoo Lee, Jin Sam Kawk, Young-Hyoun Kwon, Sungho Moon LG Electronics Chie Ming Chou Industrial Technology Research Institute | E-mail: xin.chang@huawei.com yunsongyang@huawei.com camille@lge.com samji@lge.com chieming@itri.org.tw |
| Re: | IEEE C802.16m-08/725, Proposed Baseline Content on the Uplink Control Structure for the 802.16m SDD |  |
| Abstract | This contribution proposes modified text for the separation of uplink ranging channel for non-synchronized MS and synchronized MS. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Uplink Ranging Channel

Xin Chang, Hongjie Si, Juejun Liu, Yunsong Yang

Huawei

HyunWoo Lee, Jin Sam Kwak, Young-Hyoun Kwon, and Sungho Moon

LG Electronics

Chie Ming Chou

Industrial Technology Research Institute

# Motivation

The physical control channel should be defined due to unique transmission structure. The physical structure of uplink ranging channel for non-synchronized MS and synchronized MS isn’t same due to different allowable time offset. Therefore, this contribution proposes the separation of uplink ranging channel for non-synchronized MS and synchronized MS as shown in following text proposal.

# Text Proposal

--------------------------------------------Start of the modified text--------------------------------------------

[Replace subsection 11.x.2.4 by following text]

### UL Ranging Channel

The UL ranging channel is used for UL synchronization. The UL ranging channel can be further classified into ranging channel for non-synchronized MS and synchronized MS. A random access procedure, which can be contention based or non-contention based is used for ranging. Contention-based random access is used for initial ranging, periodic ranging and handover. Non-contention based random access is used for periodic ranging and handover.

# Ranging Channel for Non-Synchronized MS

The ranging channel for non-synchronized MS is used for initial access and handover.

# Multiplexing with other control channels and data channels

The UL ranging channel ranging channel for non-synchronized MS starts at a pre-determined location with the configuration defined in a DL broadcast control message.

The UL ranging channel ranging channel for non-synchronized MS is FDM with other UL control channels and data channels.

# PHY structure

The ranging sequence design and mapping to subcarriers are TBD.

# Ranging Channel for Synchronized MS

The ranging channel for synchronized MS is used for periodic ranging.

# Multiplexing with other control channels and data channels

The UL ranging channel ranging channel for synchronized MS starts at a pre-determined location with the configuration defined in a DL broadcast control message.

The UL ranging channel ranging channel for synchronized MS is FDM with other UL control and data channels.

# PHY structure

The structure of UL ranging channel resource blocks, pilots and resource mapping are TBD.

The ranging sequence design and mapping to subcarriers are TBD.

--------------------------------------------End of the modified text--------------------------------------------