| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Structure of SU and MU-MIMO codebooks |  |
| Date Submitted | 2008-07-14 |  |
| Source(s) | Bruno Clerckx, David Mazzarese, Jerry Pi, Hokyu Choi, Heewon Kang Samsung Electronics | E-mail: bruno.clerckx@samsung.com, d.mazzarese@samsung.com, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for comments on final DL MIMO SDD text (IEEE C80216m-08_657r2) |  |
| Abstract | Proposed for SU-MIMO codebook structure |  |
| Purpose | To discuss and adopt the proposal into the IEEE 802.16m SDD (IEEE C80216m-08_657r2) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Structure of SU and MU-MIMO codebooks

Bruno Clerckx, David Mazzarese et al.

Samsung Electronics

# Introduction

The codebook(s) for 2 and 4 transmit antennas should be designed based on performance and complexity. Some desirable properties of the codebook are:

- Low complexity codebook design can be attained by choosing the elements of each matrix/vector from a small set. An example of the small set is the 4-alphabet size {±1, ±j}. This avoids the need for computing matrix multiplication. Additionally a nested property can further reduce the complexity of CQI calculation when rank adaptation is performed.

- Rank overriding: Base station may perform rank overriding which will result in significant CQI mismatch if the codebook structure cannot cope with this. A nested property w.r.t. rank overriding can be exploited to limit the impact of such issues.

- Power amplifier balance: Codebook design with constant modulus (CM) property is beneficial for avoiding unnecessary increase in PAPR.

- Good performance for a wide range of propagation scenarios: uncorrelated, correlated, and dual-polarized channels. A DFT-like codebook is optimal for linear array with small antenna spacing since the vectors match with the structure of the transmit array response. Additionally, with a clever choice of the matrices and the entries of the codebook (rotated block diagonal structure), significant gains can be obtained in dual-polarized scenarios.

- Low feedback and signaling overhead. We believe that a 3 bit codebook for 2Tx and 4bit codebook for 4Tx is a reasonable compromise. Additional gain can be obtained by the use of a differential codebook.

- Low memory requirement

16e codebooks have been optimized exclusively for uncorrelated scenarios and the performance loss in other scenarios (correlated antennas, dual-polarized antennas) is quite severe. In order to achieve significant performance gain in many deployment scenarios and for several CL MIMO schemes, we propose not to use 16e codebooks in 16m but recommend the use of a unique codebook composed of a combination of DFT-based matrices and precoding matrices optimized for cross-polarized channels. Our rationale is explained in the following text.

# Performance Analysis

In the following figures, we compare performance of 16e vs. DFT-based codebook. The 2Tx codebook is a DFT codebook with 3bits for rank 1 and 2 bits for rank 2. The 4-bit 4Tx codebook is a DFT-based codebook (which has been optimized to perform well in all propagation scenarios) composed of 6 matrices. The 6-bit 4Tx codebook is an un-optimized DFT codebook. Results are given for an average SNR of 0 and 10dB.

In SU-MIMO, these DFT codebooks offer nearly the same performance as GLP and Householder based codebooks (i.e. 802.16e codebook) in uncorrelated channels. They are however more robust to transmit correlations even with a smaller number of bits. In cross-polarized channels, 802.16e codebook performance doesn't increase from 3 to 6 bits, whereas the 4-bits DFT-based codebook with some matrices optimized for the cross-polarized channel significantly outperforms both of them.

In MU-MIMO, a DFT codebook significantly outperforms a GLP/16e codebook for uncorrelated and correlated scenarios for both unitary and non-unitary precoding.

## SU-MIMO

#### 2x2

Figure 1. Performance of 16e (3bits) and DFT (3bits) codebook-based CL SU MIMO in 2x2 uncorrelated and correlated scenarios at 10dB.

Figure 2. Performance of 16e (3bits) and DFT (3bits) codebook-based CL SU MIMO in 2x2 uncorrelated and correlated scenarios at 0dB.

Figure 3. Performance of 16e (3bits) and DFT (3bits) codebook-based CL SU MIMO in 2x2 uncorrelated and correlated scenarios.

#### 4x2 16e 6bits vs DFT 6bits in correlated and uncorrelated

Figure 4. Performance of 16e (6bits) and DFT (6bits) codebook-based CL SU MIMO in 4x2 uncorrelated and correlated scenarios at 10dB.

Figure 5. Performance of 16e (6bits) and DFT (6bits) codebook-based CL SU MIMO in 4x2 uncorrelated and correlated scenarios at 0dB.

#### 4x2 16e 6bits vs. DFT 4bits in correlated and uncorrelated

Figure 6. Performance of 16e (6bits) and DFT (4bits) codebook-based CL SU MIMO in 4x2 uncorrelated and correlated scenarios at 10dB.

Figure 7. Performance of 16e (6bits) and DFT (4bits) codebook-based CL SU MIMO in 4x2 uncorrelated and correlated scenarios at 0dB.

#### 4x2 16e 6bits vs. DFT 4bits in dual-pol

Figure 8. Performance of 16e (3/6bits) and DFT (4bits) codebook-based CL SU MIMO in 4x2 dual-polarized scenarios at 10dB.

#### 4x4 16e 6bits vs. DFT 4bits in dual-pol

Figure 9. Performance of 16e (3/6bits) and DFT (4bits) codebook-based CL SU MIMO in 4x4 dual-polarized scenarios at 10dB.

#### 4x4 16e 6bits vs. DFT 4bits in uncorrelated

Figure 10. Performance of 16e (6bits) and DFT (4bits) codebook-based CL SU MIMO in 4x4 uncorrelated scenarios at 10dB.

## MU-MIMO

#### ZFBF in 4x4 with 16e 6bits vs. DFT 4bits and DFT 6bits in uncorrelated channels 4x4

Figure 11. Performance of 16e (6bits) and DFT (4/6bits) codebook-based CL MU MIMO with ZFBF in 4x4 uncorrelated scenarios.

#### ZFBF in 4x4 with 16e 6bits vs. DFT 4bits and DFT 6bits in correlated channels 4x4

Figure 12. Performance of 16e (6bits) and DFT (4bits) codebook-based CL MU MIMO with ZFBF in 4x4 correlated scenarios.

#### Unitary precoding with 16e vs. DFT (2bits) in uncorrelated 2x2

Figure 13. Performance of 16e (2bits) and DFT (2bits) codebook-based CL MU MIMO with unitary precoding in 2x2 uncorrelated scenarios.

#### Unitary precoding with 16e vs. DFT (2bits) in correlated 2x2

Figure 14. Performance of 16e (2bits) and DFT (2bits) codebook-based CL MU MIMO with unitary precoding in 2x2 correlated scenarios.

#### Unitary precoding with 16e vs. DFT (2bits) in correlated 4x2

Figure 15. Performance of 16e (3bits) and DFT (3bits) codebook-based CL MU MIMO with unitary precoding in 4x2 correlated scenarios.

Simulation conditions

| Transmission bandwidth | 20 MHz |
| --- | --- |
| Centre frequency | 2.5 GHz |
| Subframe duration | 0.6171 ms |
| Subcarrier spacing | 10. 938 kHz |
| FFT size | 1024 |
| Number of occupied subcarriers | 1008 |
| Number of OFDM symbols per subframe | 6 |
| Number of subcarriers per Resource Unit | 18 |
| Spatial channel environment | Modified PedB channel, 3 km/h, uncorrelated at MS - Uncorrelated case: 4 wavelengths spacing and 15 degree angular spread at the base station - Correlated case: ½ wavelength spacing and 3 degree angular spread at the base station - Dual polarized case: +/-45  V-H, ½ wavelength spacing and 3 degree angular spread at the base station |
| CQI feedback | 6 subframes delay, error-free |
| Uplink sounding | 18 dB power difference between DL and UL |
| Feedback load | Full feedback (for every resource unit), 10 users |
| Downlink pilot overhead | 2 Tx: 11.11%, 4 Tx: 22.22% |
| Channel estimation | Ideal |
| MIMO detection method | Linear MMSE |
| Modulation and coding | 10 MCS levels |
| HARQ | Chase Combining, non-adaptive, 8 subframes retransmission delay, maximum 4 retransmissions |

# Complexity Analysis

Here we make a coarse analysis on the implementation complexity of various codebooks.  For simplicity, we just consider the total scalar multiplication operations for UE CQI calculation. The reason being that multiplication operation is much more complex than addition operation.

The following LMMSE SINR metric needs to be computed for each pre-coder matrix W and the k-th spatial stream

where H is the channel matrix and W is an element of the codebook.

# We investigate the impact of the codebook structure on the number of complex multiplication operations involved in the calculation of the quantity. We investigate four codebook structures: no structure, constrained alphabet (QPSK)-based codebook, nested property-based codebook and constrained alphabet and nested property-based codebook. The nested property w.r.t. the CQI calculation is defined as follows:

Definition 1: A codebook has the nested property w.r.t. the CQI calculation when the following is true:

- Calculation results could be re-used for a larger rank if the lower rank precoder is part of the precoder used for a larger rank.

- Calculations for the larger rank could be based on the operations performed for a lower rank and with a precoder that is a subset of the precoder used for the larger rank.

We assume that when  is calculated, the product  is first evaluated. Once calculated, it is used in the product .

### No Structure

For a given rank r and codeword W in the codebook, the calculation of  involves  complex multiplications. Calculations of  involves . Denoting as  the number of codewords W in rank-r codebook, the total number of multiplications involved in calculating the quantityfor all codewords in all ranks is given by

### Constrained alphabet (e.g. QPSK)-based codebook

For a given rank r and codeword W in the codebook, the calculation of  involves 0 complex multiplications since the codeword entries are chosen in the QPSK set [1,-1,j,-j]. Calculations of  involves . Denoting as  the number of codewords W in rank-r codebook, the total number of multiplications involved in calculating the quantityfor all codewords in all ranks is given by

### Nested property (NP)-based codebook

For rank 1 and codeword W in the codebook, the calculation of involves  complex multiplications. For a given rank r and codeword W in the codebook, the calculation of  doean’t involve any additional complex multiplications thanks to the nested property. Calculations of  involve  complex multiplications for rank 1 and maximum  complex multiplications for rank r>1. Denoting as  the number of codewords W in rank-r codebook, the total number of multiplications involved in calculating the quantityfor all codewords in all ranks is given by

### Constrained alphabet (e.g. QPSK) and nested property-based codebook

For rank 1 and codeword W in the codebook, the calculation of involves 0 complex multiplications. For a given rank r and codeword W in the codebook, the calculation of  doesn’t involve any additional complex multiplications thanks to the nested property. Calculations of  involve  complex multiplications for rank 1 and maximum  complex multiplications for rank r>1. Denoting as  the number of codewords W in rank-r codebook, the total number of multiplications involved in calculating the quantityfor all codewords in all ranks is given by

Here is the relative complexity (w.r.t. a codebook with no structure) in terms of the required number of complex multiplications for a 4x4 CL SU MIMO using 4 bits codebook per rank.

Figure 16. Relative complexity of different kind of codebook structures

The 2Tx 3-bit codebook and 4Tx 4-bit codebook, whose performance have been shown in previous sections, satisfy the nested property w.r.t. CQI calculation and have a large number of QPSK matrices.

Figure 17 shows the relative complexity of 16e 6bits codebook vs. a NP+QPSK DFT-based 4bits codebook. The complexity of the DFT codebook is about 1/20 the complexity of 16e codebook.

Figure 17. Relative complexity of different kind of codebook structures

In MU-MIMO, the user is limited to rank 1 transmission. Hence the nested property cannot be exploited. However the constrained alphabet may still be very beneficial. Since some schemes rely on very large codebook sizes, we show hereafter the relative simulation time as a measure of complexity. We assume ZFBF with 4, 6, 8 and 11 bits in 4x2 channels. The codebook is GLP-like codebook. The only simulation parameters that have changed are the codebook sizes. This clearly shows that the complexity of the best PMI search is not negligible for large codebook sizes, which should be avoided in 16m.

Figure 18. Relative simulation time for various codebook sizes

# PAPR

Denoting the entries of the precoder W over subband s as Wij,s, the PAPR measure used in the following table is given as

with  where Es denotes expectation over all subbands.

The following table gives an estimate of the PAPR (dB) of 16e codebooks in uncorrelated and correlated scenarios (same simulation parameters as in the aforementioned table). As we can see, the PAPR is not negligible. The DFT codebooks on the other hand have a PAPR=0dB since all precoders are constant modulus

Uncorrelated channels, CL SU MIMO

|  | 0 | 5 | 10 | 15 | 20 | 25 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2x2 3bits |  |  |  |  |  |  |  |
| 4x2 3bits | 2.0512 | 2.6117 | 2.9979 | 3.1443 | 2.9887 | 3.2426 | 3.2779 |
| 4x2 6bits | 1.2269 | 0.9126 | 0.8381 | 1.0038 | 0.9417 | 0.9161 | 0.8055 |
| 4x4 3bits | 0.9978 | 0.5149 | 0.4721 | 0.3985 | 0.1684 | 0.0727 | 0.0156 |
| 4x4 6bits | 0.7108 | 0.5469 | 0.4548 | 0.2431 | 0.1395 | 0.0618 | 0.0128 |

Correlated channels, CL SU MIMO

|  | 0 | 5 | 10 | 15 | 20 | 25 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2x2 3bits |  |  |  |  |  |  |  |
| 4x2 3bits | 4.8588 | 4.2392 | 4.2981 | 4.1037 | 2.8271 | 1.9459 | 2.9210 |
| 4x2 6bits | 2.5515 | 2.8177 | 2.0229 | 1.7632 | 1.9476 | 1.7066 | 2.6161 |
| 4x4 3bits | 4.3576 | 4.3263 | 4.3126 | 3.0632 | 2.4117 | 3.3714 | 3.3001 |
| 4x4 6bits | 1.6583 | 2.0245 | 1.9140 | 2.0536 | 2.7772 | 2.8714 | 3.2693 |

The power imbalance in 4x2 CL SU MIMO with 6bits 16e codebook is illustrated in Figure 19

Figure 19. Power imbalance (PAPR) of 16e 6 bits codebook in 4x2 CL SU MIMO in correlated and uncorrelated channels.

# Rank overriding

The nested property can be used to easily perform rank overriding at the base station. The nested property w.r.t. rank overriding is defined as follows.

Definition 2: A codebook has the nested property for rank overriding when for each pre-coder matrix in a certain rank (2,3,...,min{nt,nr}), there exists at least one corresponding column subset in all the codebooks of the lower ranks.

Note that a codebook that satisfies the nested property w.r.t. CQI calculation doesn’t necessarily satisfy the nested property w.r.t. rank overriding and inversely.

The 2Tx 3-bit codebook and 4Tx 4-bit codebook, whose performance have been shown in previous sections, satisfy the nested property w.r.t. to rank overriding.

# Summary of Conclusions

| Scenario | Comparison | Remark |
| --- | --- | --- |
| SU CL MIMO in 2x2 channel | 10 dB: 16e > DFT-based by 3% (uncorr) 16e = DFT-based (corr) 0 dB: 16e > DFT-based by 0.5% (uncorr) 16e < DFT-based by 2% (corr) | 16e: 3 bits DFT-based: 3 bits |
| SU CL MIMO in 4x2 channel | 10 dB: 16e > DFT-based by 4% (uncorr) 16e = DFT-based (corr) 16e < DFT-based by 3.5% (dual-pol) 0 dB: 16e = DFT-based (uncorr) 16e < DFT-based by 5.5% (corr) | 16e: 6 bits DFT-based: 4 bits |
| SU CL MIMO in 4x4 channel | 16e > DFT-based by 3% (uncorr) 16e < DFT-based by 5% (dual-pol) | 16e: 6 bits DFT-based: 4 bits |
| Overall performance in SU MIMO | 16e 6-bits ≈ DFT-based 4-bits | Lower overhead for DFT-based codebook |
| MU CL MIMO in 2x2 channel (PU2RC) | 16e < DFT-based by 2.5% (uncorr) 16e < DFT-based by 16% (corr) | 16e: 2 bits DFT-based: 2 bits |
| MU CL MIMO in 4x2 channel (PU2RC) | 16e < DFT-based by 150% (corr) | 16e: 3 bits DFT-based: 3 bits |
| MU CL MIMO in 4x4 channel (ZFBF) | 16e < DFT-based by 5% (uncorr) 16e < DFT-based by 110% (corr) | 16e: 6 bits DFT-based: 4 bits |
| Overall performance in MU MIMO | 16e << DFT-based | For any overhead |
| MS computational complexity | 16e >> DFT-based |  |
| PAPR | 16e >> DFT-based |  |

Overall conclusion:

- In SU-MIMO, 16e codebook and DFT-based codebook have similar performance with a slight advantage of 16e over DFT-based codebook in uncorrelated and slight advantages of DFT-based codebook over 16e in correlated and dual-polarized scenarios.

- In MU-MIMO, DFT-based codebook very significantly outperforms 16e codebook.

- The overhead is lower for a DFT-based codebook compared to 16e codebook.

- The complexity of CQI calculation with DFT-based codebook is significantly lower than with 16e codebook. DFT is constant modulus and enables rank-overriding contrary to 16e codebook.

# Proposed Text

# 11.x.2.1.2. Closed-loop SU-MIMO

# 11.x.2.1.2.1. Precoding technique

In FDD and TDD systems, unitary codebook based precoding shall be supported.

[To add to current text]

The desired properties of the codebook are:

• Good performance in uncorrelated, correlated, and dual-polarized channels

• Low feedback and signaling overhead

• Low computational complexity

• Low memory requirement

• Low PAPR / Power balanced

A codebook composed of a combination of DFT-based matrices and precoding matrices optimized for cross-polarized channels is supported

…

11.x.2.2. Multi-user MIMO

11.x.2.2.3.2. CSI feedback

[To add to current text]

The MU MIMO codebook is composed of unitary DFT-based matrices.