| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on 08/657r2 |  |
| Date Submitted | 2008-07-14 |  |
| Source(s) | Adrian Boariu Shaohua Li Peter Wang Zexian Li Xin Qi | Shaohua.li@nsn.com |
| Re: | DL MIMO Rapporteur Group Discussions |  |
| Abstract | Comments on 80216m-08/657r2 |  |
| Purpose | For review and discussion in the Project 802.16m DL MIMO Rapporteur Group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Comments on 08/657r2

Adrian Boariu, et al.

NSN

# Introduction

Some features to be included in the MIMO RG document are proposed.

802.16e system allows rate 2 STC transmission for 2 TX, as matrix C in 8.4.8.3.3. The current draft for DL MIMO does not support this feature. We are proposing to include this feature in the draft (see Proposal 1)

Most of the transmit diversity currently proposed in the draft, will likely be used with the localized resource allocation. So it is important to achieve higher diversity where ever the number of antennas is increased. On p.7 line 11 it is proposed a 4TX rate 1 scheme, which however it has the diversity gain of 2 per RX antenna. We are proposing in Proposal 2, a diversity scheme that can achieve higher diversity gain. Also, we propose to remove the 8x2 diversity technique in the draft based on the same reasoning (see Proposal 3).

# Changes

========== Proposal 1==========

[Insert at the end of section 11.x.2.1.1.1  the following]

For 2Tx antennas, rate 2, the input to the MIMO encoder is represented a 4  1 vector

x=s1s2s2s4.

The output of the MIMO encoder is a 4  2 matrix

z=s1s2−s2s1s3s4−s4s3

The output of precoder is a 2x2 matrix

y=D×W×z

where D is a 2  2 diagonal phase angles matrix, and W is a precoder 2x4 matrix.

[In Table 1 insert a line as]

|2 | 2| 4 | 2|

========= end Proposal 1 ===============

=========== Proposal 2 ============

[Insert the end of section 11.x.2.1.1.1 the following]

For 4Tx antennas, rate 1, the input to the STBC/SFBC encoder is represented as a 4  1 vector

x=s1s2s2s4.

The output of the STBC/SFBC encoder is a 4  4 matrix

z=s1s2s3s4−s2s1−s4s3s3s4s1s2−s4s3−s2s1.

The output of precoder is a 4x4 matrix

y=D×W×z

where D is a 4  4 diagonal phase angles matrix and W is a precoder 4x4 matrix.

[In Table 1 insert a line as]

|4 | 1| 4 | 4|

============= end Proposal 2 ==========

=========== Proposal 3 =========

Delete on p.7 lines 15 to 18, related to 8 TX rate 1.

Delete the 3rd row in Table 1 that it is in the 08/657r2 draft.

========= end Proposal 3 =========