| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for MAC Addressing |  |
| Date Submitted | 2008-07-16 |  |
| Source(s) | TGm A (Upper MAC sessions) |  |
| Re: | Re: IEEE 802.16m-08/024 – Call for Contributions on Project 802.16m System Description Document (SDD), on the topic of “Upper MAC – MAC Addressing” |  |
| Abstract | This contribution defines MAC addresses for 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposal for MAC Addressing

Susan Hartman, Intel Corporation

Proposed SDD text

============================== Start of Proposed Text ===============================

10.X  MAC Addressing

The MS has a global address and logical addresses that identify the MS and connections during operation.

10.X.1  IEEE 802 MAC Address

The 48-bit globally-unique IEEE 802 MAC address uniquely identifies the MS.

10.X.2  MAC Logical Addresses

MAC logical addresses are assigned to the MS by management messages from the BS.  Logical addresses are used for resource allocation and management of the MS.

10.X.2.1  “Station Identifier”

The BS assigns a “Station Identifier” to the MS during network entry to uniquely identify the MS to the BS.  Each MS registered in the network has an assigned “Station Identifier”.  Some specific “Station Identifiers” are reserved, for example, for broadcast, multicast, and ranging.

10.X.2.2  “Flow Identifier”

Each MS connection is assigned a “Flow Identifier” that uniquely identifies the connection within the MS. “Flow Identifiers” identify management connections and active transport Service Flows.

============================== End of Proposed Text =================================