| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Session 56 UL PHY Rapp Group Chairs Report |  |
| Date Submitted | July 16, 2008 |  |
| Source(s) | Apostolos Papathanassiou Changhoi Koo Ron Murias | apostolos.papathanassiou@intel.com chkoo@samsung.com ron@murias.ca |
| Re: |  |  |
| Abstract | Chairs report on the development of the UL PHY Rapp Group SDD contribution for Session 56 |  |
| Purpose | Define UL PHY Structure for the 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Session 56 UL PHY Rapp Group Chairs Report

Apostolos Papathanassiou

Changhoi Koo

Ron Murias

Rapp Group Chairs

# Draft Preparation

Based on contributions submitted to Session #55, Draft 1 was released on May 27, 2008.  This draft consisted of a Table of Contents.  On June 9, contributions submitted for Session 55 were used to create baseline content for Draft 2.  New contributions and submissions were also accepted at this time.  Some content was merged to eliminate redundant text.

Several interim drafts were produced to reflect the submission of more contributions and feedback between draft releases.

Draft 3 was generated based on content received by June 30, 2008.  A PDF version of this draft was released along with a call for comments on the draft, the comments to be resolved during Session 56.

# Session 56 Activity

## Meeting Minutes (June 16 session)

Call to order: 08:06 the meeting was called to order with Apostolos Papathanassiou and Ron Murias chairing.  The Chairs detailed the agenda for the day and directed the participants to the download location of the files required for the meeting.  The main points of discussion revolved around the issues of tile size, resource unit mapping, and multiplexing mode for PUSC legacy systems.

Comment discussion (highlights)

The database contained 31 comments.  The comments were reviewed in order of page number referenced in the document.

During the discussion of comment #5, it became clear that there may be cases where the context of a resource unit may not be clear.  The Chairs were asked to highlight the issue to the SDD Editor and recommend that, where the (uplink or downlink) context is not obvious, a prefix denoting the transmission direction (i.e. UL- or DL-) should be used.

The group had a lengthy discussion on tile structure.  One major issue is that it is difficult to move forward with things like pilot comparisons when there is not tile structure defined.  On the other hand, pilot schemes have an impact on how to design the tile structure.  The group decided to start with a 6x Nsym tile (Nsym depending on the subframe type) and to leave other possible tile structures for further study.

Resource mapping reached an agreement by harmonization done during the meeting.

The PUSC legacy issue was not resolved.  We feel it is best to leave a “TBD” in the contribution to allow members to bring proper contributions to the Task Group for a thorough discussion.

The Group recessed at 12:06 to reconvene in the joint UL PHY/Control session.  During this session, the UL PHY group closed the remaining comments and discussed plans for the Chairs report and the TGm contribution.  A vote was taken to forward the contribution (as modified by the database).  There were no objections to forwarding the updated contribution to TGm. Following this, the two Rapp groups worked together to define comparison criteria based UL Control contributions.

# Remaining Issues

Legacy support with PUSC remains undecided between (essentially) TDM and FDM.  We felt that further discussion in the main group is most appropriate.  We expect that interested parties will bring detailed contributions that clearly show the issues and proposed solutions.

## TGm Contributions

C802.16m-08/872: UL PHY Rapp Group Chairs Report (this document)

C802.16m-08/873: UL PHY Rapp Group Contribution

C802.16m-08/874: UL PHY Rapp Group Commentary Database

# Motion

To adopt the text in C80216m-08_aaa as part of the SDD