| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modified Remedy Regarding “Comment #121” |  |
| Date Submitted | 2008-07-17 |  |
| Source(s) | Sassan Ahmadi Intel Corporation Jin Sam Kwak and Sungho Moon LG Electronics | E-mail: sassan.ahmadi@intel.com E-mail: {samji; msungho}@lge.com |
| Re: | IEEE 802.16m-08/024 “Call for Contributions on P802.16m SDD” - IEEE 802.16m-08/003r3 |  |
| Abstract | The contribution proposes a modified remedy on “Comment 121” for 802.16m SDD. |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Modified Remedy Regarding “Comment #121”

Sassan Ahmadi

Intel Corporation

Jin Sam Kwak and Sungho Moon

LG Electronics

Text Proposal for the Modified Remedy on “Comment #121”

============================= Start of Proposed Text =============================

## 11.4 Frame structure

### 11.4.1 Basic Frame structure

The IEEE 802.16m basic frame structure is illustrated in Figure 1. Each 20 ms superframe is divided into four equally-sized 5 ms radio frames. When using the same OFDMA parameters as in Table 1 with the channel size of 5 MHz, 10 MHz, or 20 MHz, each 5 ms radio frame further consists of eight subframes. A subframe is assigned for either DL or UL transmissions. There are two types of subframes depending on the size of cyclic prefix: 1) the type-1 subframe which consists of six OFDMA symbols (some of which may be idle symbols) and 2) the type-2 subframe that consists of seven OFDMA symbols.

The basic frame structure is applied to FDD and TDD duplexing schemes, including H-FDD MS operation. The number of switching points in each radio frame in TDD systems either two or four, where a switching point is defined as a change of directionality, i.e., from DL to UL or from UL to DL.

When H-FDD mobile stations are included in an FDD system, the frame structure from the point of view of the H-FDD mobile station is similar to the TDD frame structure; however, the DL and UL transmissions occur in two separate frequency bands. The transmission gaps between DL and UL (and vice versa) are required to allow switching the TX and RX circuitry.

Figure 1 Basic frame structure

### 11.4.1.1 Frame Structure for CP=1/8 Tu

Figure 2 illustrates an example TDD frame structure with DL to UL ratio of 5:3. Assuming OFDMA symbol duration of 102.82 µs and a CP length of 1/8 Tu, the length of type-1 subframe is 0.617 ms. In Figure 15, the last DL subframe, i.e., DL SF4, is an irregular subframe whose last OFDMA symbol is an idle symbol to accommodate the gap required to switch from DL to UL. Other numerologies may result in different number of subframes per frame and symbols within the subframes.  shows the frame structure in FDD mode.

DL

SF0

DL

SF1

DL

SF2

DL

SF3

DL

SF4

UL

SF5

UL

SF6

UL

SF7

UL/DL PHY Frame: 5 ms (8 subframes)

Subframe Length = 6 Full OFDM Symbols = 0.617 ms

Switching Points for DL:UL=5:3

Super Frame: 20 ms (4 frames, 32 subframes)

CP=1/8 Tu

F3

F2

F1

F0

Regular Subframe

S0

S1

S2

S3

S4

Irregular Subframe

S0

S1

S2

S3

S4

S5

Subframe Length = 6 Full OFDM Symbols = 0.617 ms

S5

Idle time

62.86 µs

Idle time

102.82 µs

Idle symbol

Figure 2 Frame structure with type-1 subframe in TDD duplex mode (CP=1/8 Tu)

S0

S1

F3

F2

F1

F0

S2

S3

S4

S5

SF0

SF1

SF2

UL/DL

SF3

SF4

SF5

SF6

SF7

UL/DL PHY Frame: 5 ms (8 subframes + Idle Time)

UL/DL PHY Subframe: 0.617 ms

Subframesubframesubframe

One OFDM symbol

Super Frame: 20 ms (4 frames, 32 subframes)

Idle Time = 62.86 µs

CP=1/8 Tu

Figure 3 Frame structure with type-1 subframe in FDD duplex mode (CP=1/8 Tu)

### 11.4.1.2 Frame Structure for CP=1/16 Tu

Figure 17 illustrates an example of TDD and FDD frame structure with a CP of 1/16 Tu. Assuming OFDMA symbol duration of 97.11 µs and a CP length of 1/16 Tu, the length of type-1 and type-2 subframes are 0.583 ms and 0.680 ms, respectively. Other numerologies may result in different number of subframes per frame and symbols within the subframes.

Figure 4 TDD and FDD Frame Structure with a CP of 1/16 Tu (DL to UL ratio of 5:3)

============================= End of Proposed Text ==============================