| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Elliptic Curve Cryptography Authorization and Key Agreement for IEEE 802.16m |  |
| Date Submitted | 2008-08-29 |  |
| Source(s) | Ranga Reddy US Army DJ Shyy MITRE Sheng Sun Nortel | E-mail: ranga.reddy@us.army.mil E-mail: djshyy@mitre.org E-mail: shengs@nortel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | "Call for Contributions and Comments on Project 802.16m System Description Document (SDD)", IEEE 802.16m-08/033, comments on Section 12 of SDD "Security" |  |
| Abstract | Elliptic Curve Cryptography (ECC)-based authorization is computationally and resource-wise more efficient that RSA-based authorization currently used in IEEE 802.16-based networks. In an effort to optimize system operation and resource utilization, it is suggested that ECC-based authorization be incorporated into IEEE 802.16m. |  |
| Purpose | Review contribution, discuss, and consider incorporation of text into IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Elliptic Curve Cryptography-based Authorization

# & Key Agreement for IEEE 802.16m

Ranga Reddy

US Army

DJ Shyy

MITRE

Sheng Sun

Nortel

# 1. Introduction

RSA cryptography generates keys by taking two large prime numbers.  The inherent security is in the difficultly of recovering this key via factorization of large integers.  It is generally accepted that RSA keys should be a minimum of 1024 bits.  Discrete logarithm cryptography (DLC) is another area of cryptography where security is provided by difficulty in solving logarithmic equations over large finite groups.  ECC is a subset of DLC, where the discrete logarithm solution over an equation (plane curve).  The elliptic curve equations can over finite groups on prime fields (Fp) or binary fields (F2m).  For prime fields, where p is a large/odd prime > 3, curves are of the form y2 = x3 + a*x + b.  For binary fields, the order of the field is a power of 2, the curves are of the form y2 + x*y = x3 + a*x2 + b.

ECC keys can be smaller than RSA keys, because it is believed that the solution to a discrete logarithm is fundamentally more complex than the factorization of large integers.  For example, the ECC key size equivalent of a 1024 bit RSA key is 160 bits [3].  Table 1 [6] and Table 2 [7] shows some (practical) performance data for ECC and RSA operations (ECDSA is the ECC equivalent of DSA).  The measurements were executed on 8bit Atmel Atmega128-series microcontrollers.

| Algorithm | Signature |  | Key Exchange |  |
| --- | --- | --- | --- | --- |
| RSA 1024 | 304 | 11.9 | 15.4 | 304 |
| ECDSA 160 | 22.82 | 45.09 | 22.3 | 22.3 |
| RSA 2048 | 2302.7 | 53.7 | 57.2 | 2302.7 |
| ECDSA 224 | 61.54 | 121.98 | 60.4 | 60.4 |

Table 1: Energy cost of digital signatures

and key exchange computations (mJ)

|  | Operation Time [s] | Speedup (ECC:RSA) |
| --- | --- | --- |
| RSA 1024 | 10.99 | 1 |
| ECC 160 | 0.81 | 13.6 |
| RSA 2048 | 83.26 | 1 |
| ECC 224 | 2.19 | 38 |

Table 2: Operation speedup

Currently RSA-based authorization, or RSA + EAP authorization/authentication is supported in 802.16 networks [1].  The X.509 certificates have RSA encryption keys, between 1024 and 2048 bits.  We propose to add elliptic curve cryptography (ECC)-based authorization to the suite of security protocols in IEEE 802.16m.  This requires the use of ECC-based X.509 certificates that support key lengths of 160 - 224 bits.  The structure of the certificates will have to be adjusted to match ECC requirements, and a new CA capable of supporting X.509-based ECC certificates will have to be created [8].  The ECC authorization should be added to the cryptographic suite of protocols and should be the preferred method, while maintaining use of RSA authorization for legacy MS.

# 2. Text Proposal

[---------------------------------------------------Start of Text Proposal--------------------------------------------------]

# 12 Security

## 12.x Authorization, Authentication Procedures

### 12.x.x Authorization via ECC/RSA-based Authentication

Manufacturer installed X.509 certificates can be formed from either RSA or ECC keys and signatures.  For RSA-based authentication, the rest of the authorization/authentication and key-agreement process will follow the procedures used in the legacy system.  For ECC-based authentication, procedures will be amended to make use of some form of Elliptic Curve Diffie Hellman key-agreement.

## 12.y Cryptographic Methods

### 12.y.y Public-key encryption of AK & Digital Signatures

AKs in Auth Reply messages shall be either RSA or ECC public-key encrypted, using the SS’s public key.  RSA-based certificates will follow the structure as defined in legacy system.

ECC will use curves over prime fields, where the order of the field is no less 160 bit prime and no greater than 224 bit prime.  Example curves are listed in Appendix J, Section J.5.1 thru J.5.3 in ANSI X9.63-2001.  These examples can be used, but it is recommended that when creating certificates manufacturers calculate their own base points.

[---------------------------------------------------End of Text Proposal--------------------------------------------------]

# 3. References

[1] "Draft Standard for Local and Metropolitan Area Networks, Part16: Air Interface for Broadband Wireless Access Systems", IEEE P802.16 Rev2/D6, July 2008.

[2] Hamiti, Shkumbin, "The Draft IEEE 802.16m System Description Document", IEEE 802.16m-08/003r4, July 2008.

[3] Barker, Elaine, et al., "Recommendation for Key Management - Part 1: General (Revised)", NIST Special Publication 800-57, March 2007.

[4] Barker, Elaine, et al., "Recommendation for Pair-Wise Key Establishment Schemes Using Discrete Logarithm Cryptography (Revised)", NIST Special Publication 800-56a, March 2007.

[5] American National Standards Institute, "American National Standard for Financial Services X9.63-2001: Public Key Cryptography for the Financial Services Industry, Key Agreement and Key Transport Using Elliptic Curve Cryptography", ANSI X9.63-2001, November 2001.

[6] Wander, A.S., et al., “Energy Analysis of public-key cryptopgraphy for wireless sensor networks”, Third IEEE Conference on Pervasive Computing and Communications (PerCom), pg's 324 – 328, March 2005.

[7] Eberle, Hans, "Accelerating Next-generation Public-key Cryptography on General-purpose CPUs", Hot Chips 16, http://www.hotchips.org/archives/hc16/3_Tue/2_HC16_Sess6_Pres2_bw.pdf, August 2004.

[8] Cano, M.-D., etc al., "A Certification Authority for Elliptic Curve X.509v3 Certificates", IEEE Third International Conference on Networking and Services, pg 49, June 2007.