| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on 802.16m SDD Subframe Structure with CP Length of 1/8 |  |
| Date Submitted | 2008-09-05 |  |
| Source(s) | Kanchei(Ken) Loa, Chiu-Wen Chen, Chun-Yen Hsu, Yi-Hsueh Tsai, Yung-Ting Lee, Youn-Tai Lee, Hua-Chiang Yin, Chih-Wei Su Yi-Ting Lin Institute for Information Industry Shiann-Tsong Sheu National Central University (NCU) Yih-Guang Jan, Yang-Han Lee Tamkang University (TKU) Whai-En Chen National Ilan University (NIU) | Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw |
| Re: | SDD Session 56 Cleanup; in response to the TGm Call for Contributions and Comments 802.16m-08/033 for Session 57 |  |
| Abstract | Comments on 802.16m SDD |  |
| Purpose | For discussion and approval by IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on 802.16m SDD Subframe Structure with CP Length of 1/8

Kanchei(Ken) Loa, Chiu-Wen Chen, Chun-Yen Hsu, Yi-Hsueh Tsai, Yung-Ting Lee, Youn-Tai Lee, 
Hua-Chiang Yin, Chih-Wei Su, Yi-Ting Lin
Institute for Information Industry

Shiann-Tsong Sheu
National Central University (NCU)

Yih-Guang Jan, Yang-Han Lee
Tamkang University (TKU)

Whai-En Chen
National Ilan University (NIU)

Introduction

In current SDD (IEEE 802.16m-08/003r4), Figure 15 and related text in section 11.4.1.1 and 11.5.3 are not consistent in subframe type and symbol duration with CP 1/8. This contribution proposes modifications to ensure consistency in section 11.4.1.1 and 11.5.3. The proposed modifications are based on the comment #133, which was accepted in Session #56, in the commentary file 80216m-08_025r3.cmt.

Proposed Changes:

[Modify Lines 6-11 in Page 32 as indicated:]

Figure 15 illustrates an example TDD frame structure with DL to UL ratio of 5:3. Assuming OFDMA symbol duration of 102.857µs and a CP length of 1/8 Tu, the length of type-1 subframe is 0.617 ms. In Figure 15, the last DL subframe, i.e., DL SF4, is an type-1 shorttype-2 subframe whose last OFDMA symbol is an idle symbol to accommodate the gap required to switch from DL to UL. Other numerologies may result in different number of subframes per frame and symbols within the subframes. Figure 16 shows an example of a frame structure in FDD mode, , and Figure 17 shows an example of a frame structure with CP of 1/16 Tu.

[Modify Figure 15 in Page 33 as indicated:]

DL

SF0

DL

SF1

DL

SF2

DL

SF3

DL

SF4

UL

SF5

UL

SF6

UL

SF7

UL/DL PHY Frame: 5 ms (8 subframes)

Subframe Length = 6 Full OFDM Symbols = 0.617 ms

Switching Points for DL:UL=5:3

Superframe: 20 ms (4 frames, 32 subframes)

CP=1/8 Tu

F3

F2

F1

F0

Tpye-1Regular Subframe

S0

S1

S2

S3

S4

Type-1 ShortIrregular Subframe

S0

S1

S2

S3

S4

S5

Subframe Length = 6 Full OFDM Symbols = 0.617 ms

S5

Idle time

62.86 µs

Idle time

102.857102.82 µs

Idle symbol

Figure 1 Frame structure with type-1 subframe in TDD duplex mode (CP=1/8 Tu)

[Modify Lines 20-24 in Page 48 as indicated:]

The pilots can be used for channel estimation, measurements (CQI and interference mitigation/cancellation), frequency offset estimation and time offset estimation. The pilots will be dedicated to localized and distributed subchannels. The pilots will be pre-coded, using the same pre-coding and/or boosting of the information burst(s) when such pre-coding is applied. The pilot patterns are optimized for both type-1regular and type-2 subframes.

[Modify Lines 3-6 in Page 49 as indicated:]

For type-1 short subframe or type-2 subframe of 5 or 7 OFDMA symbol, the first or last OFDMA symbols will be deleted or added. The interlaced pilot patterns can be generated by cyclic shifting the base pilot pattern. The interlaced pilot patterns are shown in Error: Reference source not found and can be optionally used by different BSs. The use of interlaced pilot pattern is FFS.