| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | RS Network Entry |  |
| Date Submitted | 2008-09-12 |  |
| Source(s) | Kanchei (Ken) Loa, Yi-Hsueh Tsai, Hua-Chiang Yin, Yung-Ting Lee, Chun-Yen Hsu, Chih-Wei Su, Youn-Tai Lee Institute for Information Industry Whai-En Chen National Ilan University Shiann-Tsong Sheu, Chih-Cheng Yang National Central University Yang-Han Lee, Yih Guang Jan Tamkang University | Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw |
| Re: | MAC: Network Entry; in response to the TGm Call for Contributions and Comments 802.16m-08/033 for Session 57 |  |
| Abstract | Comment on 802.16m System Description Document |  |
| Purpose | For review and discussion in the Project 802.16m UMAC Rapporteur Group (Idle AHG) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

RS Network Entry

Kanchei (Ken) Loa, Yi-Hsueh Tsai, Hua-Chiang Yin, Yung-Ting Lee,
Chun-Yen Hsu, Chih-Wei Su, Youn-Tai Lee

Institute for Information Industry (III)

Whai-En Chen

National Ilan University

Shiann-Tsong Sheu, Chih-Cheng Yang

National Central University

Yang-Han Lee, Yih Guang Jan

Tamkang University

Introduction

This contribution proposes text for the 16m RS network entry based on the draft standard IEEE P802.16j.

Proposed Changes:

3 Definition, Symbols, Abbreviation

[Add following text in subclause 3:]

Access station: A station that provides a point of access into the network for an MS or RS. An access station can be a BS or RS.

16 Support for multi-hop relay

[Add following subclauses in 16.x:]

## 16.x Relay Station State Diagram

The following Figure xxx illustrates the mobile station state transition diagram for IEEE802.16m RS. Relay Station state diagram for IEEE802.16m systems consists of 3 states, Initialization State, Access State, and Operational State.

Figure xxx System State Transition Diagram of IEEE802.16m

## 16.x.1 Initialization State

Initialization State is where an RS without any connection with BS performs the access station selection by scanning and synchronizing to a BS preamble, and acquiring the system configuration information through BCH before entering Access State. During this state, if the RS cannot properly perform the BCH information decoding and access station selection, it should return to perform scanning and DL synchronization. If the RS successfully decodes BCH information and selects at least one target access station (1st access station selection), e.g. BS or access RS, it goes to the Access State. The RS can transit from any state to the initialization state under abnormal conditions.

Figure yyy Initialization State Transition Diagram

## 16.x.1.1 Access State

Access State is where the RS performs network entry to the selected access stations by going through several processes. The RS performs the ranging process (initial ranging code and RNG-REQ/RSP MAC message is used in the IEEE P802.16j) in order to obtain UL synchronization. Next, the RS performs basic capability negotiation with the access station (SBC-REQ/RSP MAC message is used in the IEEE P802.16j). The RS then performs the authentication and authorization process. Next, the RS performs the registration process (REG-REQ/RSP MAC message is used in the IEEE P802.16j). Then, the RS will report neighbor station measurement in order to perform 2nd stage access station selection (RS_NBR-MEAS-REP/RS_AccessRS-REQ message is used in the IEEE P802.16j). Finally, the access station shall determine the RS's operation parameters and configure these parameters to the RS (RS_Config-CMD message is used in the IEEE P802.16j). RS receives the 802.16m specific IDs as part of Access State procedures.

Upon successfully performing the Access State operation, the RS goes to the Operational State. Otherwise, in case of abnormal operation, the RS goes back to the Initialization State.

Figure 1 Access State Transition Diagram

<Editor’s note: This figure identifies the access state transitions consistent with IEEE 802.16j. These procedures may be modified for IEEE 802.16m. Further updates are likely necessary. In addition the names of the messages in the parenthesis are the ones used in the Reference System>

## 10.x.1.2 Operational State

The Operational State consists of 2 modes; Active Mode and Neighbor Measurement Mode. During Operational State, the RS maintains at least one connection as established during Access State, while RS and access station may establish additional connections, e.g. tunnel connection or transport connection. And also, RS can scan neighbor’s signal to reconfigure it’s parameters which provides robust and reliable services.

Figure zzz Operational State Transition Diagram

### 10.x.1.2.1 Active mode

During Active Mode, the RS and the access Station perform normal operations to exchange the DL/UL traffic transaction between the RS and the access Station. The RS can perform the fast network re-entry procedures after handover: while in handover, RS maintains any 802.16m specific identifiers required for handover (FFS) and its connections with access station. Without going through Access State, the RS may remain in Operational State with target BS.

### 10.x.1.2.1 Neighbor measurement mode

During Neighbor measurement and Scanning mode, the RS performs neighbor measurement or scanning operation and may be temporarily unavailable to the access Station. While in active mode, RS transitions to neighbor measurement mode via explicit MAC signaling (RCD/RS_NBR-MEAS-REP message is used in the IEEE P802.16j) or implicitly without management messages generation.