| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Dynamically Locating User Specific Control Blocks of USCCH in IEEE 802.16m |  |
| Date Submitted | 2008-09-05 |  |
| Source(s) | Namgi Kim Kyonggi University Hyun Lee, Sungcheol Chang, Kwangjae Lim, Chulsik Yoon ETRI | Voice: +82-16-470-7414 E-mail: ngkim@kyonggi.ac.kr Voice: +82-42-860-5456 E-mail: {hyunlee, csyoon}@etri.re.kr |
| Re: | MAC: SDD Session56 Cleanup; in response to the TGm Call for Contributions and Comments 802.16m-08/033 for Session 57 |  |
| Abstract | This contribution provides a way to dynamically locate user specific control blocks of USCCH in IEEE 802.16m. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Dynamically Locating User Specific Control Blocks of USCCH in IEEE 802.16m

Namgi Kim

Kyonggi University

Hyun Lee, Sungcheol Chang, Kwangjae Lim, Chulsik Yoon

ETRI

# Introduction

The superframe concept has already been included in SDD [1]. In the superframe concept, the data burst is transmitted on a subframe basis. A data burst can be transmitted in a single subframe or transmitted over consecutive subframes depending on many reasons such as low latency, large burst size, and link budget [2]. Therefore, the data burst allocation also should be possible to be dynamically assigned over subframes. In order to fully support dynamic allocation of the data burst based on a subframe, the control blocks for user specific control information of USCCH needs to be dynamically located by the subframe. This is because the location of the control blocks limits the maximum number of subframes over which a data burst is encoded. Therefore, the ‘n’ value, which determines the location of the control blocks, needs to be dynamically changed by the subframe. However, the current SDD supports only statically fixed ’n’ value in a system. The location of the control blocks for user specific control information of USCCH cannot change by the subframe [1]. Accordingly, in this comment, we propose a simple method to dynamically change the ‘n’ value by the subframe. It dynamically locates the control blocks for user specific information of USCCH based on a subframe. The proposed method does not require changing the statically fixed operation of the ‘n’ value in the current SDD. It is just an add-on feature and well harmonized with the current SDD operation which statically locates the USCCH control blocks for user specific information and does not change it by the subframe.

# Proposal

In the SDD, control blocks for user specific control information of USCCH are located ‘n’ 802.16m subframes apart. However, there is no way to assign the ‘n’ value by the subframe. Therefore, we propose that the ‘n’ value is transmitted through the control block for user specific control information of USCCH (CB-USCI) if a system wants to change the ‘n’ value on a subframe basis. When the ‘n’ value is included in the CB-USCI, the ‘n’ value in the current CB-USCI indicates where the next CB-USCI will be appeared by the subframe. For example, if the current CB-USCI is appeared in subframe N and the ‘n’ value in the current CB-USCI is 2, then the current CB-USCI can point to resource allocation in subframe N or N+1 and next CB-USCI will be appeared in subframe N+2. Figure 1 shows the example of dynamically locating the CB-USCI by inserting the ‘n’ value in the CB-USCI. In this figure, the dimension of resource allocation for downlink bursts does not matter. This proposal allows both one and two dimensional resource allocations. To specify which dimensional allocation is used for the downlink is out of scope of this proposal. If a system wants to fix the ‘n’ value and does not want to change it on a subframe basis, the ‘n’ value in CB-USCI can be omitted and the fixed or default ‘n’ value may be used.

Figure 1. Example of dynamically locating user specific control blocks of USCCH

# Proposed Text

[Add new sentences at the end of line 18, page 62]

In order to dynamically change the ‘n’ value by the subframe, the ‘n’ value can be transmitted through the control blocks for user specific control information. If the ‘n’ value is appeared in the control blocks of the current subframe, then the next control blocks for user specific control information should be appeared after ‘n’ subframes of the same frame. If a system wants to statically fix the ‘n’ value and does not want to change it on a subframe basis, the ‘n’ value in the control blocks can be omitted and the fixed or default ‘n’ value may be used.

# References

[1] IEEE 80216m-08/003r4, “The Draft IEEE 802.16m System Description Document”

[2] IEEE C80216m-08/062r1, “Proposed 802.16m Frame Structure”