| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Handover Procedure with Multi-Carrier Operation in IEEE 802.16m |  |
| Date Submitted | 2008-09-05 |  |
| Source(s) | Namgi Kim Kyonggi University Hyun Lee, Sungcheol Chang, Kwangjae Lim, Chulsik Yoon ETRI | Voice: +82-16-470-7414 E-mail: ngkim@kyonggi.ac.kr Voice: +82-42-860-5456 E-mail: {hyunlee, csyoon}@etri.re.kr |
| Re: | MAC: Multi-Carrier Operation; in response to the TGm Call for Contributions and Comments 802.16m-08/033 for Session 57 |  |
| Abstract | This contribution provides the procedure for efficient handover with multi-carrier operation in IEEE 802.16m. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Handover Procedure with Multi-Carrier Operation in IEEE 802.16m

Namgi Kim

Kyonggi University

Hyun Lee, Sungcheol Chang, Kwangjae Lim, Chulsik Yoon

ETRI

# Introduction

According to the SDD [1], the multi-carrier operation has been considered as a feature of the IEEE 802.16m system. In the multi-carrier operation, a BS and MS may simultaneously exchange information through more than one carrier. Because there are one more than carriers, the handover scenarios also can be various and quite different from that of single carrier operation. However, there is no consideration for handover scenarios with the multi-carrier operation in the current SDD document [1]. Therefore, in this contribution, we propose a new handover procedure for the multi-carrier operation in the IEEE 802.16m system. The proposed procedure almost eliminates handover interruption time by starting handover the primary carrier while transferring data through the secondary carriers.

# Proposal

According to the SRD [2], the handover interruption time is the time duration that an MS cannot exchange user plane packets with any BS. In the single carrier operation, this handover interruption time cannot be eliminated because a MS cannot exchange packets when the only carrier is being used for handover. However, the situation is different in the multi-carrier operation. In the multi-carrier operation, there is at least more than one carrier. Thus, it may be possible to start handover through one carrier while transferring data through the others. Figure 1 shows the proposed handover procedure with multi-carrier operation. In the proposed multi-carrier handover (MCHO) procedure, the MS and the serving BS (S-BS) exchanges data through the secondary carriers while the MS and the target BS (T-BS) starts handover through the primary carrier. When a MS wants to perform handover, the MS negotiates with the S-BS and the T-BS whether they use the MCHO procedure or not. These MCHO request messages may be transferred by MOB_MSHO_REQ and HO-Request. As the same way, the MCHO response messages may be transferred by HO-Response and MOB_BSHO_RSP. After negotiating MCHO possibility, if the MCHO is possible, the MS starts to exchange data through only secondary carriers and starts to handover by sending MOB_HO_IND to the S-BS. The procedure up to sending RNG_REQ to the T-BS is the same as the procedure of the single carrier operation. After the T-BS gets the RNG_REQ from the MS, the T-BS sends HO-Disconnect-SC to the S-BS. The HO-Disconnect-SC indicates that the S-BS needs not to maintain the connection with the MS. So, the S-BS disconnects the connection with the MS through the secondary carriers and sends HO-Ack and the current context of the MS to the T-BS. After getting the HO-Ack and context, the T-BS sends RNG_RSP to the MS. After receiving the RNG_RSP, the MS needs not to maintain the second carrier connections with the S-BS any more. After that, the MS sends BW_REQ to T-BS and completely ends the MCHO procedure. If a fully configured secondary carrier exists, the proposed MCHO procedure is always supportable. On the other hand, if only partially configured secondary carriers exist, the proposed procedure may be supported with limited capability.

Figure 1. Handover procedure with multi-carrier operation

# Proposed Text

[Insert the following text into a suitable place of SDD]

-------- Start of the text proposal ---------------------------------------------------------------------------------------

### 19.x Handover Procedure for Multi-carrier Operation

When the MS, serving BS, and target BS is supportable multi-carrier operation, the MS can perform multi-carrier handover (MCHO). In the MCHO, the MS can start handover to the target BS through the primary carrier while transferring data with the serving BS through the secondary carriers. The usability of MCHO should be negotiated with the MS, serving BS, and target BS before the MS starts to send handover signaling messages to the target BS through the primary carrier. After the MS is connected with the target BS through the primary carrier, the target BS shall send a message to the serving BS for disconnecting the secondary carriers between the MS and the serving BS. After receiving the disconnection message, the serving BS may send the current context of the MS to the target BS.

-------- End of the text proposal ---------------------------------------------------------------------------------------

# References

[1] IEEE 80216m-08/003r4, “The Draft IEEE 802.16m System Description Document”

[2] IEEE 80216m-08/002r5, “IEEE 802.16m System Requirements”