| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Further Study on Non-linear Precoding with Guaranteed Gain over Linear Precoding |  |
| Date Submitted | 2008-09-05 |  |
| Source(s) | Tsuguhide Aoki, Hiroki Mori, Yong Sun Henning Vetter, Ngoc Dao (Toshiba) Marc de Courville, Fred Vook (Motorola) Ron Porat (Nextwave) Isamu Yoshii (Panasonic) Takaaki Kishigami(Panasonic) | tsuguhide.aoki@toshiba.co.jp Sun@toshiba-trel.com Marc.de.Courville@motorola.com Fred.Vook@motorola.com RPorat@nextwave.com yoshii.isamu@jp.panasonic.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-08/033: Call for Contributions and Comments on Project 802.16m System Description Document (SDD), 2008-08-01, on Call for Detailed Physical Layer Comments - Any parts of Section 11 (PHY) that are incomplete, inconsistent, empty, TBD, or FFS. |  |
| Abstract | This contribution presents further study on non-linear precoding with guaranteed gain over linear precoding to support nonlinear precoding in SDD. |  |
| Purpose | To be considered and accepted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Further Study on Non-linear Precoding 
with Guaranteed Gain over Linear Precoding

Tsuguhide Aoki, Yong Sun, Courville, Fred Vook, Ron Porat, Zheng Yan Xiu, Isamu Yoshii

Toshiba, Motorola, Nextwave, ITRI, MediaTech, NEC, Panasonic

# Introduction

This contribution is in correspondence to Call for Contributions and Comments on Project 802.16m System Description Document (SDD) issued on 2008-08-01 [1], specifically on Call for Detailed Physical Layer Comments – Any parts of Section 11 (PHY) that are incomplete, inconsistent, empty, TBD, or FFS.

In section 11.8.2.2.1 of the current version of SDD [2], linear precoding has been specified and supported for multiuser MIMO, and non-linear precoding is FFS. This contribution presents further study on nonlinear precoding over linear precoding. With our comprehensive study of non-linear precoding in different applications, it shows clearly that system with employing non-linear precoding can achieve significant guaranteed gain on performance and capacity over linear precoding. Consequently, we propose that non-linear precoding is supported for multiuser MIMO in SDD.

# Overview of linear precoding and non-linear precoding

Multiuser MIMO (MU-MIMO) advocates for the use of spatial sharing of the channel by the users. In spatial multiple access, the resulting multiuser interference is handled by the multiple antennas which in addition to providing per-link diversity also give the degrees of freedom necessary for spatial separation of the users. Consequently, in multiuser MIMO networks, the spatial degrees of freedom offered by multiple antennas can be advantageously exploited to enhance the system capacity, by scheduling multiple users to simultaneously share the spatial channel.

It is well-understood that there are several major advantages of MU-MIMO. The most important aspect of MU-MIMO schemes is to allow for a direct gain in multiple access capacity, which directly improves the system capacity at its design stage. Secondly, MU-MIMO appears more immune to most of propagation limitations plaguing single user MIMO communications such as channel rank loss or antenna correlation. Furthermore, MU-MIMO allows the spatial multiplexing gain at the base station to be obtained without the need for multiple antenna terminals, which significantly simplifies the design and lower the complexity of mobile terminals.

Because of the potential of MU-MIMO, there have been many proposals and contributions to the SDD to support MU-MIMO in the IEEE 802.16m system, including linear precoding and non-linear precoding. The linear precoding has been well-accepted and supported in SDD, which is a generalization of traditional SDMA, where users are assigned different precoding matrices at the transmitter. The precoders are designed jointly based on channel state information (CSI) of all the users, based on any number of designs. Even though the linear precoding provides reasonable throughput performance, it remains far from non-linear precoding in practical system deployment.

Fundamentally, non-linear precoding can achieve near optimal system capacity. With non-linear precoding approach, any known interference at the transmitter can be subtracted without the penalty of radio resources if the optimal precoding scheme can be applied on the transmit signal. Besides the much higher throughput performance over linear precoding, with our study in various application environments, it also shows clearly that non-linear precoding is more robust to impairments of CSI feedback, interference in cellular application and mobility support.

It is stated clearly in IMT-Advanced requirements [3] that one of the key features of IMT-Advanced is enhanced peak data rates to support advanced services and applications (100 Mbit/s for high and 1 Gbit/s for low mobility). Even though the higher MIMO configurations, such as 4x4 and 8x8, might provide the basis of the system to achieve the requirements, the advanced non-linear precoding provides the guaranteed capacity gain.

So it is essential to support both linear precoding and non-linear precoding in SDD for 802.16m system.

# Summary of contributions to 802.16m on non-linear precoding

There are several contributions proposed to TGm on non-linear precoding techniques, which also presented advantages and high performance on applications. In [7, 5], VP and THP have been proposed and discussed…

[4]  IEEE C802.16m-08/842r1, “MU-MIMO: Non-Linear Precoding for DL-MIMO”, 2008-07-21

[5]  IEEE C802.16m-08/366, “Proposal on Multi-user MIMO Precoding Considerations of IEEE 802.16m”, 2008-05-05

[6]  IEEE C802.16m-08/205r2, “Pilot design for precoding in Multiuser MIMO on IEEE802.16m downlink”, 2008-03-12

[7]  IEEE C802.16m-08/058r1, “Proposal on Multi-user MIMO Precoding Considerations of IEEE 802.16m”, 2008-01-18

In this contribution, we present a comprehensive study of non-linear precoding over linear precoding. The detailed comparisons on various applications are demonstrated in the following section.

# Demonstration of system performance on applications

Simulation parameters

| Parameters | Values |
| --- | --- |
| Bandwidth | 10MHz |
| FFT size | 1024 |
| Carrier Frequency | 2.5GHz |
| Subframe structure | 16m |
| Channel Model | SCME |
| Radio environment | Urban Micro |
| Mobile speed | 0km/h |
| Linear precoding schemes | ZF, MMSE |
| Non-linear precoding schemes | - Tomlinson-Harashima Precoding (ZF, MMSE) - Vector Precoding (ZF, MMSE) |
| CQI feedback frequency, error, and delay | Assume perfect CSI |
| Antenna configuration | 4x4 |
| BS antenna spacing | 10l |
| Receiver algorithm | 2 antenna-MMSE |
| Channelization | Localized |
| Resource allocation size | 3RB per user |
| Channel Coding | 16e CTC |
| Modulation | QPSK |
| Code rate | 1/2 |
| Channel Estimation | Perfect channel estimation |

# System gain under ideal conditions

Although the several contributions about NLP don’t obey the simulation parameters provided from 16m, we’ll show the results which fit the simulation parameters above this time.

Figure 1 Performance with regard to the normalized SNR

Figure 1 illustrates the Frame Error Rate (FER) performance with regard to the normalized SNR. The normalized SNR is defined as  where  and  is a transmit power and noise power, respectively. The ZF/MMSE shows the linear ZF/MMSE performance and THP shows the ZF or MMZE-based Tomlinson-Hiroshima Precoding [9]. The VP shows the ZF or MMSE-based Vector Precoding [10]. This figure shows that nonlinear precoding gives large gain over linear precoding. When BS has knowledge of the SINR at MS, then BS can utilize it and MMSE-based processing is possible. When MMSE-based processing is used, simple MMSE-based THP with ordering achieves near the VP performance.

# Low SNR Performance

Figure 1 indicates that the nonlinear precoding still provides significant gain over linear precoding. This means nonlinear precoding is superior to linear precoding in cellular environment with interference.

# Performance with channel impairments

We’ve already evaluated how CSI error affects the MU-MIMO performance. The results in [4] shows that the effect of the CSI error is severe for linear precoding and not for nonlinear precoding.

# Advantages and disadvantages on user selection and scheduling

There are a couple of other concerns we have dealt with. When user selection algorithm is used with MU-MIMO techniques, the linear precoding might achieve extra gain when the number of users increases and there are semi-orthogonal users. This is well-known multiuser-diversity. However, these semi-orthogonal users don't always require a connection or transmission in real system so multiuser-diversity gain cannot always be achieved. On the other hand, the nonlinear precoding is not affected by the multiuser-diversity and always provides better performance.

# Extra signaling, feedback, complexity and latency

Nonlinear precoding needs channel information at the transmitter. The information can be obtained from sounding, analog-feedback or codebook-based feedback without extra overhead. The results in Figure 1 assumes sounding or analog-feedback. Note that in [8] it has already indicated that, for the same UL overhead, analog is better than one PMI feedback. The channel feedback and process might cause a certain latency. However, The latency problem depends on the vender implementation. VP needs to search the optimum vector based on the transmit symbols and provides the best performance. MMSE-based THP has a good trade-offs about the complexly and performance.

.

# MCS selection

The MCS selection algorithm for the nonlinear precoding is the same as the one used for the linear precoding which is based on the sounding, analog-feedback or codebook-based feedback.

# Suggestions and recommendations

Nonlinear precoding provides significant gain compared with linear precoding under the low SNR cellular environment and is very robust against the channel state information at the transmitter. Therefore, nonlineare precoding should be supported in IEEE802.16m.

# ~.~.~.~.~.  Text input start  ~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.

##### 11.8.2.2.1 Precoding technique

The precoding for MU-MIMO can be either standardized or vendor-specific. Up to four MSs can be assigned to each resource allocation.

In MU-MIMO systems, the received signal of the f-th subcarrier in the i-th MS (without considering co-channel interference) can be described as:

yi,f=Hi,fj=1Kvj,fxj,f+ni,f, Equation 1

where K is the number of the allocated users, Vj,f is the precoding vector of the f-th subcarrier for the transmit signal to the j-th MS, xj,fis the transmit signal of the f-th subcarrier to the j-th MS and ni,fis the noise of the f-th subcarrier in the j-th MS.

If dedicated pilots are used, the form and derivation of  the assembled precoding matrix, , can be either standardized or vendor-specific. If the columns of the assembled precoding matrix are orthogonal to each other,  it is defined as unitary MU-MIMO. Otherwise, it is defined as non-unitary MU-MIMO. Note that beamforming is enabled with this precoding mechanism. Non-linear precoding is supported..

# ~.~.~.~.~.  End of text input~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.~.

# References

[1]   IEEE 802.16m-08/033, “Call for Contributions and Comments on Project 802.16m System Description Document (SDD)”, 2008-08-04.

[2]   IEEE 802.16m-08/003r4, “Draft IEEE 802.16m System Description Document”, 2008-07-29.

[3]   ITU-R Working Party 5D Sub-Working Group Radio Aspects, “DRAFT REPORT ON REQUIREMENTS RELATED TO TECHNICAL PERFORMANCE FOR IMT-ADVANCED RADIO INTERFACE(S) IMT.TECH]”, 1 June 2008

[4]   IEEE C802.16m-08/842r1, “MU-MIMO: Non-Linear Precoding for DL-MIMO”, 2008-07-21

[5]   IEEE C802.16m-08/366, “Proposal on Multi-user MIMO Precoding Considerations of IEEE 802.16m”, 2008-05-05

[6]   IEEE C802.16m-08/205r2, “Pilot design for precoding in Multiuser MIMO on IEEE802.16m downlink”, 2008-03-12

[7]   IEEE C802.16m-08/058r1, “Proposal on Multi-user MIMO Precoding Considerations of IEEE 802.16m”, 2008-01-18

[8]   IEEE C802.16m-08/529r1, “Analog vs. Codebook Feedback Performance Comparison”, 2008-07-10

[9]   J. Liu and W. A. Krzymien, “A minimum mean-square error criterion based onlinear joint transmitter –receiver processing algorithm for the downlink of multi-user MIMO systems,” IEEE VTC2006-spring, May. 2006

[10]   C. Yuen and B. M. Hochwald, “How to gain 1.5dB in vector precoding,” IEEE Globecom2006, Nov. 2006