| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Mapping of UL control information to UL control channels |  |
| Date Submitted | 2008-09-09 |  |
| Source(s) | Allan Xu Huawei Technologies Co., Ltd. | Voice: +86-755-28970188 E-mail: zhier81@huawei.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-08/033: Call for Comments and Contributions on Project 802.16m System Description Document (SDD) : PHY group (SDD Cleanup) |  |
| Abstract | Mapping of UL control information to UL control channels in Section 11.9.4 is not consistent with the description in Section 11.9.2, we just modify Section 11.9.4 to make it identical to Section 11.9.2 |  |
| Purpose | Discuss and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Mapping of UL control information to UL control channels

Allan

Huawei Technologies Co., Ltd

# 1.  Problem Statement

In Section 11.9.4 of SDD document, the table contents are not consistent with the description in Section 11.9.2. According to Section 11.9.2, UL HARQ feedback channel is used to carry HARQ feedback information, The UL fast feedback channel carries channel quality feedback and MIMO feedback. But in the table of Section 11.9.4, any information can be carried in any channel, these two sections have different description.

# 2. Text Proposal

-------- Start of the text proposal ---------------------------------------------------------------------------------------

[Change Section 11.9.4 of IEEE 802.16m-08/003r4 as following:]

11.9.4 Mapping of UL control information to UL control channels

<Editors’ Notes: This table needs to be updated as the mapping of UL control information to UL control channels is developed.>

| Information | Channel |
| --- | --- |
| Channel quality feedback MIMO feedback | UL fast feedback channel |
| HARQ feedback | UL HARQ feedback channel |
| Sounding | UL Sounding channel |
| synchronization | UL ranging channel |
| Bandwidth request | Bandwidth request channel |
|  | UL Inband Control Signaling |
| E-MBS feedback | FFS |

------- End of proposed text ---------------------------------------------------------------------------------------------