| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | CQI calculation for MU-MIMO schemes |  |
| Date Submitted | 2008-09-04 |  |
| Source(s) | Bruno Clerckx, David Mazzarese Samsung Electronics Guangjie Li, Hongming Zheng, Senjie Zhang, Yang-Seok Choi Intel Corporation | E-mail: bruno.clerckx@samsung.com, d.mazzarese@samsung.com, E-mail: guangjie.li@Intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | SDD Session 56 Cleanup |  |
| Abstract | CQI calculation for MU-MIMO |  |
| Purpose | To discuss and adopt the proposal into the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# CQI calculation for MU-MIMO schemes

Bruno Clerckx, David Mazzarese

Samsung Electronics

Guangjie Li, Hongming zheng, Senjie Zhang, Yang-seok choi

Intel Corporation

# Introduction

The current SDD text specifies “If dedicated pilots are used, the form and derivation of   the assembled precoding matrix, , can be either standardized or vendor-specific. If the columns of the assembled precoding matrix are orthogonal to each other, it is defined as unitary MU-MIMO. Otherwise, it is defined as non-unitary MU-MIMO.”

In order to achieve significant gains with the MU-MIMO mode, it is very important to have an accurate estimate of the CQI at the BS. If the precoding matrix is vendor specific, the MS does not know what kind of filtering is applied at the BS and therefore cannot derive a CQI based on the BS filter. Hence it has to be able to compute a unique CQI that provides good performance for any vendor-specific MU-MIMO.

We show that the CQI calculation as used in unitary precoding can be re-used in non-unitary precoding (e.g. ZFBF) without affecting performance. Hence we propose to standardize the CQI calculation method at the MS in order to make sure that irrespective the exact implementation of the MU-MIMO scheme (unitary or non-unitary), good performance can be guaranteed. The proposed text is given at the end of the contribution.

# CQI calculation with unitary precoding

In a unitary MU-MIMO, users are scheduled using a unitary precoding matrix. This unitary matrix is chosen in a set of unitary matrices known at both the MS and the BS. Let us denote by Vgsuch unitary precoding matrix. At the time of measurement, the MS calculates (using measurement pilots, e.g. midamble) SINRs of every stream for all the unitary matrices in the set. The SINR of stream m for unitary precoding matrixVgat receiver k can be written as

SINR(Vg)k,m=bHHkVg(m)2bHHkVg(l)2+MtPb2                    =bHHkVg(m)2bHHk−bHHkVg(m)2+MtPb2

where bis the receive beamforming vector such as MMSE and Vg(m) is the mth column vector of unitary matrix Vg. Mt and P are the number of transmit antennas and total transmit power respectively. Since the unitary property, beamforming vectors Vg(l)l≠m are removed in the denominator. We assume equal power allocation among streams and the noise with unit power.

# Performance Analysis

This CQI calculation has been used for some time for unitary precoding. We investigate the use of such CQI calculation in non-unitary precoding and compare with CQI calculation methods well known for non-unitary precoding [1-4].

In particular, we compare this unitary precoding CQI with the following methods:

- dominant eigenvector combiner with Philips CQI [1]

- dominant eigenvector combiner with LTE rank 1 CQI [2]

- dominant eigenvector combiner with channel norm CQI

- Quantization-based combiner (QBC) with Philips CQI [3]

- Minimum Effective SINR Combiner (MESC) [4]

In Figure 1, performance of ZFBF using those different CQI methods is displayed. Rank adaptation at the BS is performed and the CQI is calculated assuming full rank transmission. MESC is well known to provide excellent performance for ZFBF. Figure 1 simply shows that assuming unitary precoding in CQI calculation leads to the same performance as MESC. Hence assuming unitary precoding when calculating the CQI can be used for both unitary and non-unitary precoding and it achieves the best performance for both approaches.

Figure 1. Performance of ZFBF in 4x4 uncorrelated channels with various CQI calculation methods.

Simulation conditions

| Transmission bandwidth | 10 MHz |
| --- | --- |
| Centre frequency | 2.5 GHz |
| Subframe duration | 0.6171 ms |
| Subcarrier spacing | 10. 938 kHz |
| FFT size | 1024 |
| Number of occupied subcarriers | 1008 |
| Number of OFDM symbols per subframe | 6 |
| Number of subcarriers per Resource Unit | 18 |
| Spatial channel environment | Modified PedB channel, 3 km/h, uncorrelated at MS - Uncorrelated case: 4 wavelengths spacing and 15 degree angular spread at the base station |
| CQI feedback | 6 subframes delay, error-free (~4ms delay) |
| Subchannelization and frequency granularity of feedback | LLRU (1 adjacent PRU) |
| Feedback load | Full feedback (for every resource unit), 10 users |
| pilot pattern | 16m |
| Channel estimation | Ideal |
| MIMO detection method | Linear MMSE |
| Modulation and coding | 10 MCS levels |
| HARQ | Chase Combining, non-adaptive, 8 subframes retransmission delay, maximum 4 retransmissions |

# References

[1] 	R1-062483, Philips, “Comparison between MU-MIMO codebook-based channel reporting techniques 	for LTE downlink”, 3GPP TSG RAN WG1 Meeting #46bis, Seoul, South Korea, 9th October – 13th 	October 2006.

[2]	R1-073225, “MIMO AH Summary”, 3GPP TSG RAN WG1 Meeting #49bis, Orlando, USA, June 25 – 	29, 2007

[3]	N. Jindal, Antenna Combining for the MIMO Downlink Channel, To Appear: IEEE Trans. Wireless 	Communications

[4]	M. Trivellato, H. Huang, and F. Boccardi, "Antenna Combining and Codebook Design for the MIMO 	Broadcast Channel with Limited Feedback," (invited paper), in Proc. Asilomar Conference on Signals, 	Systems, and Computers 2007, Pacific Grove, USA, Nov. 2007.

# Proposed text

# 11.8.2.2.3. Feedback for MU-MIMO

# 11.8.2.2.3.1. CQI feedback

For CQI feedback, the mobile station measures the downlink pilot channel, computes the channel quality information (CQI), and reports the CQI on the uplink feedback channel. Both wideband CQI and subband CQI may be transmitted by a mobile station. Wideband CQI is the average CQI of a wide frequency band. In contrast, sub-band CQI is the CQI of a localized sub-band.

[To add to current text: The CQI is calculated at the mobile station assuming that the interfering users are scheduled by the serving base station using precoders orthogonal to each other and orthogonal to the reported PMI.]