| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Differential codebook for closed loop SU and MU MIMO |  |
| Date Submitted | 2008-09-13 |  |
| Source(s) | Bruno Clerckx, David Mazzarese Samsung Electronics Mohammad (Amir) Khojastepour, Meilong Jiang, Yuefeng Zhou NEC Wookbong Lee, Jaewan Kim, Bin-Chul Ihm LGE Yinggang Du, Hongjie Si, Zhigang Rong, Jianmin Lu, Linfeng Xia, Yang Tang, Young Hoon Kwon Huawei Jun Yuan, Mo-Han Fong, Hosein Nikopourdeilami, Sophie Vrzic, Robert Novak, Dongsheng Yu Nortel Networks Minnie Ho, Yang-Seok Choi, Guangjie Li, Qinghua Li, Hongming Zheng, Shanshan Zheng Intel Corporation Ron Porat Nextwave | E-mail: bruno.clerckx@samsung.com, d.mazzarese@samsung.com, amir@nec-labs.com wbong@lge.com, lgjohnkim@lge.com, bcihm@lge.com duyinggang@huawei.com junyu@nortel.com, mhfong@nortel.com minnie.ho@intel.com rporat@nextwave.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | SDD Session 56 Cleanup |  |
| Abstract | Differential codebook for closed loop SU and MU MIMO |  |
| Purpose | To discuss and adopt the proposal into the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Differential codebook for closed loop SU and MU MIMO

Bruno Clerckx, David Mazzarese

Samsung Electronics

Mohammad (Amir) Khojastepour, Meilong Jiang, Yuefeng Zhou

NEC

Wookbong Lee, Jaewan Kim, Bin-Chul Ihm

LGE

Yinggang Du, Hongjie Si, Zhigang Rong, Jianmin Lu, Linfeng Xia, Yang Tang, Young Hoon Kwon

Huawei

Jun Yuan, Mo-Han Fong, Hosein Nikopourdeilami, Sophie Vrzic, Robert Novak, Dongsheng Yu

Nortel Networks

Minnie Ho, Yang-seok Choi, Guangjie Li, Qinghua Li, Hongming Zheng, Shanshan Zheng

Intel Corporation

Ron Porat

Nextwave

# Introduction

In order to boost 16m performance much beyond 16e and LTE performance, we propose the use of a differential codebook-based feedback in addition to the standard SU-MIMO and MU-MIMO codebook. The differential codebook exploits the fact that CL MIMO typically works at low speed, where time correlation between two feedback instants is large. This enables either to refine the codebook as time goes by or to decrease the PMI feedback overhead.

Several contributions suggested to use a differential codebook. In this contribution, we collect simulation results of various contributions (each using a different approach) and show the benefits of differential codebooks.

# Summary of Performance Gain

|  | CL SU MIMO | CL MU MIMO |
| --- | --- | --- |
| C80216m-08_1187.pdf (4 bits basic codebook + 4 bits differential codebook) | throughput gain of up to 5% over 16e 6bits codebook throughput gain of up to 10% over basic codebook 4bits | throughput gain of about 20% over 16e 6bits codebook |
| C80216m-MIMO-08_064 (Full feedback + differential feedback for both 16e and DFT codebooks) | Differential feedback significantly saves uplink overhead without much throughput loss. |  |
| C80216m-08_464 (Full feedback + differential feedback) | The differential feedback scheme achieves similar performance as full feedback scheme but overhead is reduced. | - |
| C80216m-MIMO-08_333.doc (different scenarios with 3, 4, 5, and 6 bits main codebook + 3 bits differential codebook) | Differential feedback significantly saves uplink overhead without much throughput loss. |  |
| C80216m-08_1182r1.doc |  |  |
| C80216m-08_914.doc |  |  |

# Proposed Text

# 1.1.x.2.1.3. Feedback for SU-MIMO

In FDD systems and TDD systems, a mobile station may feedback some of the following information in Closed loop SU-MIMO mode:

- Rank (Wideband or sub-band)

- Sub-band selection

- CQI (Wideband or sub-band, per layer)

- PMI (Wideband or sub-band for serving cell and/or neighboring cell)

- Doppler estimation

[To add to current text: For codebook based precoding, two different feedback modes for the PMI are supported:

- The standard mode: the feedback from a mobile station shall be based on the same codebook as used by base station for transmission, and it shall be sufficient for the base station to determine a new precoder.

- The differential mode: the feedback from a mobile station provides a differential knowledge which represents information that is used along with other feedback information known at the base station for determining a new precoder.

Mobile station shall support the standard mode and may support the differential mode.]

11.x.2.2.3.2. CSI feedback

Channel state information feedback may be employed for MU-MIMO. Codebook based feedback is supported

in both FDD and TDD. Sounding based feedback is supported in TDD.

[To add to current text: For codebook based precoding, two different feedback modes for the PMI are supported:

- The standard mode: the feedback from a mobile station shall be based on the same codebook as used by base station, and it shall be sufficient for the base station to determine a new precoder.

- The differential mode: the feedback from a mobile station provides a differential knowledge which represents information that is used along with other feedback information known at the base station for determining a new precoder.

Mobile station shall support the standard mode and may support the differential mode.]

# Appendix: Details of simulation results

##### C80216m-08_1187.pdf

##### CL SU MIMO

In Figure 1, 2 and 3, we compare the performance of CL SU MIMO using 16e 6bits codebook with those obtained using a differential feedback scheme using a basic 4 bits codebook (standard mode) at the first time instant () and a 4bit rotation codebook for successive time instants. The details are provided in C80216m-08_1187.pdf. The measurement and feedback is done every midamble, i.e. every 5 ms and the mobile speed is assumed to be 3km/h. Significant (over 5%) gain over 16e codebook can be achieved with lower feedback overhead (4bits instead of 6bits). For the same overhead, the differential codebook provides gains of about 10% over the standard codebook.

Figure 1. Performance (SNR=0dB) of 16e (6bits) and differential (4bits) codebook-based CL SU MIMO in 4x2 uncorrelated at 3km/h.

Figure 2. Performance (SNR=5dB) of 16e (6bits) and differential (4bits) codebook-based CL SU MIMO in 4x2 uncorrelated at 3km/h.

Figure 3. Performance (SNR=10dB) of 16e (6bits) and differential (4bits) codebook-based CL SU MIMO in 4x2 uncorrelated at 3km/h.

##### MU MIMO

Figure 4. Performance of ZFBF using 16e 6bits, Grassmanian Line Packing (GLP) 11bits and standard 4bits+differential codebook 4bits in 4x2 uncorrelated at 3km/h.

Differential codebook provides significant gains over 16e codebook and comes quite close to the performance achievable with very large codebook (11 bits) but with a much lower feedback overhead (4 bits).

Simulation conditions

| Transmission bandwidth | 10 MHz |
| --- | --- |
| Centre frequency | 2.5 GHz |
| Subframe duration | 0.6171 ms |
| Subcarrier spacing | 10. 938 kHz |
| FFT size | 1024 |
| Number of occupied subcarriers | 1008 |
| Number of OFDM symbols per subframe | 6 |
| Number of subcarriers per Resource Unit | 18 |
| Spatial channel environment | Modified PedB channel, 3 km/h, uncorrelated at MS - Uncorrelated case: 4 wavelengths spacing and 15 degree angular spread at the base station |
| CQI feedback | 6 subframes delay, error-free (~4ms delay) |
| Subchannelization and frequency granularity of feedback | LLRU (1 PRU) |
| Feedback load | Full feedback (for every resource unit), 10 users |
| pilot pattern | 16m |
| Channel estimation | - Ideal - Real channel estimation on midamble and dedicated pilots using 16m pilot pattern |
| MIMO detection method | Linear MMSE |
| Modulation and coding | 10 MCS levels |
| HARQ | Chase Combining, non-adaptive, 8 subframes retransmission delay, maximum 4 retransmissions |
| Transmission bandwidth | 20 MHz |
| Centre frequency | 2.5 GHz |
| Subframe duration | 0.6171 ms |
| Subcarrier spacing | 10. 938 kHz |
| FFT size | 1024 |
| Number of occupied subcarriers | 1008 |
| Number of OFDM symbols per subframe | 6 |
| Number of subcarriers per Resource Unit | 18 |
| Spatial channel environment | Modified PedB channel, 3 km/h, uncorrelated at MS - Uncorrelated case: 4 wavelengths spacing and 15 degree angular spread at the base station |
| CQI feedback | 6 subframes delay, error-free |
| Uplink sounding | 18 dB power difference between DL and UL |
| Feedback load | Full feedback (for every resource unit), 10 users |
| Downlink pilot overhead | 2 Tx: 11.11%, 4 Tx: 22.22% |
| Channel estimation | Ideal |
| MIMO detection method | Linear MMSE |
| Modulation and coding | 10 MCS levels |
| HARQ | Chase Combining, non-adaptive, 8 subframes retransmission delay, maximum 4 retransmissions |