| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Intra-BS Carrier Handover for IEEE 802.16m |  |
| Date Submitted | 2008-09-05 |  |
| Source(s) | Ruixia Zhang, Yuqin Chen, Wenhuan Wang, Kaiying Lv, Min Liu, Nan Li, Huiying Fang, Hongyun Qu, ZTE Corporation Xiaolu Dong, Ying Du, Daning Gong CATR Juhee Kim, Sookjin Lee, Eunkyung Kim, Kyungsoo Kim ETRI | Voice: [Telephone Number (optional)]] E-mail: chen.yuqin@zte.com.cn zhang.ruixia@zte.com.cn duying@mail.ritt.com.cn juhee@etri.re.kr, sjlee@etri.re.kr, ekkim@etri.re.kr, kksu@etri.re.kr <http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | MAC: Multi-Carrier Operation; in response to the TGm Call for Contributions and Comments 802.16m-08/033 for Session 57 |  |
| Abstract | This contribution proposes intra-BS Carrier Handover process for IEEE 802.16m multi-carrier system. |  |
| Purpose | To be discussed and adopted by TGm for use in the 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Intra-BS Carrier Handover for IEEE 802.16m

Ruixia Zhang, Yuqin Chen, Wenhuan Wang, Kaiying Lv, Min Liu, Nan Li, Huiying Fang, Hongyun Qu

ZTE Corporation

Xiaolu Dong, Ying Du, Daning Gong

CATR

Juhee Kim, Sookjin Lee, Eunkyung Kim, Kyungsoo Kim

ETRI

# Introduction

In multicarrier operation, due to system load balancing and carrier’s varying channel quality, MSs may switch to another carrier which is controlled by the same BS to get better performance, called intra-BS carrier handover. Accordingly, this contribution proposes a seamless intra-BS carrier handover process for IEEE802.16m to keep continuous data exchanging.

# Proposed intra-BS carrier handover process

In intra-BS carrier handover scheme, two scenarios exist.

Scenario 1:

One case is the primary carrier handover without scanning. In other words, the BS switches the primary carrier of the MS to one of the secondary carriers of the MS, which are fully configured. In this case, there is no need for handover re-entry procedure like ranging as well as scanning, since the BS and the MS maintain all information of the target carrier. The BS indicates this handover to the MS though USCCH, or MAC management message, or MAC subheader.

Figure 1 is an example of the primary carrier handover without scanning. In the example, the MS maintains connections through RF1 as the primary carrier, RF2 and RF3 as the secondary carriers. For the purpose of load balancing and resource efficiency problem due to varying channel status, the BS recognizes the necessity of changing the primary carrier of the MS. As the channel quality of RF2 is much better among all primary carrier and the secondary carriers of the MS, the BS decides to change the primary carrier to RF2 and transmits the Primary Carrier Change Indication message to the MS. After receiving the indication message, the MS changes to maintain full control of MS mobility, state and context through the new primary carrier.

Figure 1 Example of the primary carrier handover without scanning

Scenario 2:

In the second case, MS scans all the other carriers under the same BS. According to the scanning report, BS determines which carrier MS should switch to.

Figure 2 Example of the carrier handover with scanning

As intra-BS carrier handover occurs between different carriers of the same BS, many steps during the network reentry are not required, such as basic capacity negotiation, authentication and authorization, registration processes etc. The intra-BS carrier handover procedure could be a simple one consisting of the following steps:

Figure 3 flow chart of Intra-BS carrier handover with scanning

Figure 4 Intra-BS primary carrier handover procedure with scanning

Step 1: MS scans other carriers of BS to acquire the channel quality and sends the scanning results to BS.

MS can acquire the information and parameters regarding multi-carrier operation and other carriers through primary carrier, in which may include multi-carrier scanning information. According to these scanning information, MS may periodically or event-triggered scan other carriers of the BS. BS may also transmit unsolicited message to enable MS to start scanning. The message may include scanning carrier list, scanning start time and scanning interval.

Step 2: BS selects target carrier and transmits carrier handover indication message to MS. If MS needs to synchronize with the target carrier, go to step 3; else, go to step 4.

Criteria for BS to select target carrier may include factors such as carrier loading condition and channel quality. If the source carrier is served as MS’s primary carrier, the target carrier should be a fully configured carrier. If the source carrier is served as MS’s secondary carrier, the target carrier could be a fully configured or a partially configured carrier.

Carrier handover indication message may include target carrier, source carrier and synchronization flag. Synchronization flag indicates whether MS needs to synchronize with the target carrier or not, 0 = doesn’t need synchronization, 1 = need synchronization. If synchronization flag =1, carrier handover indication message also include dedicated ranging code and dedicated ranging resource allocated on the target carrier. BS reserves MS identifier and dedicated ranging code and resource. Upon receiving the dedicated ranging code on the dedicated ranging resource, BS can identify the MS based on the mapping and transmit ranging response message on the MS’s current primary carrier.

△f is a frequency threshold configured by the system. If the center frequency difference between target carrier and source carrier is greater than △f, MS needs to synchronize with the target carrier, else, MS doesn’t need to synchronize with the target carrier.

Step 3: MS synchronizes with the target carrier of the BS.

MS transmits the dedicated ranging code on the dedicated ranging resource. MS does not have to wait for RNG-RSP from the target carrier. Instead, MS receives RNG-RSP on its current primary carrier.

Step 4: MS executes handover to the target carrier.

# 3. Text proposal

Add the following text into Section 19.x of SDD 003r4:

-------------------------------  Text Start  ---------------------------------------------------

19.x  Intra-BS carrier handover

In multicarrier operation, due to system load balancing and carrier’s varying channel quality, MSs may switch to another carrier which is controlled by the same BS to get better performance, called intra-BS carrier handover.

The intra-BS primary carrier handover can be triggered by BS’s unsolicited handover indication for MS to switch to a secondary carrier which is fully configured; also it can start with MS’s scanning on other carriers.

Scenario 1: the primary carrier handover without scanning

The BS switches the primary carrier of the MS to one of the secondary carriers of the MS, which are fully configured. Since the BS and the MS maintain all information of the target carrier, there is no need for handover re-entry procedure as well as scanning. The BS indicates the primary carrier change to the MS though USCCH, or MAC management message. After the action time specified by the BS, the MS changes to maintain full control of MS mobility, state and context through the new primary carrier.

Scenario 2: the primary carrier handover procedure with scanning

In this case, intra-BS carrier handover process consists of the following steps:

Step 1: MS scans other carriers of the BS to acquire the channel quality and sends the scanning results to BS.

Step 2: BS selects target carrier and transmits carrier handover indication message to MS. If MS needs to synchronize with the target carrier, go to step 3; else, go to step 4.

Step 3: MS synchronizes with the target carrier of the BS.

MS transmits the dedicated ranging code on the dedicated ranging resource. MS does not have to wait for RNG-RSP from the target carrier. Instead, MS receives RNG-RSP on its primary carrier.

Step 4: MS executes handover to the target carrier.

Fig. xx. Procedure of Intra-BS HO with scanning

-------------------------------  Text End  ---------------------------------------------------

# 4. References

[1] IEEE 802.16m-08/003r4, Project 802.16m System Description Document (SDD)