| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | A transmission scheme of DL Control information for Multi-carrier support in IEEE 802.16m |  |
| Date Submitted | 2008-09-16 |  |
| Source(s) | Nan Li, Min Liu, Changyin Sun, Wenhuan Wang, Ruixia Zhang, Huiying Fang ZTE Corporation | Voice: [Telephone Number (optional)]] E-mail: li.nan25@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | MAC: Multi-Carrier Operation; in response to the TGm Call for Contributions and Comments 802.16m-08/033 for Session 57 |  |
| Abstract | This contribution proposes a transmission scheme of DL control information for Multi-carrier support in IEEE 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

A transmission scheme of DL Control information

for Multi-carrier support in IEEE 802.16m

Nan Li, Min Liu, Changyin Sun, Wenhuan Wang,Ruixia Zhang, Huiying Fang

ZTE Corporation

# 1 Introduction

According to IEEE 802.16m SDD, carriers of a multi-carrier system may be configured as fully configured carrier and partially configured carrier. Fully configured carrier is configured with all control channels, while partially configured carrier may be configured with only essential control channel configuration to support traffic exchanges during multi-carrier operation.

It is well known that PBCH and SBCH are used to transmit essential system parameters and system configuration information. Besides, there are extended system parameters and system configuration information, control and signaling for DL notifications, control and signaling for traffic in the DL control information category. Therefore, it will be necessary to clarify how PBCH, SBCH and other DL control information are configured on these two kinds of carriers for the benefit of system overhead reduction and improvement of transmission efficiency.

Fully configured carrier is used for MS to complete access. During initialization, MS scans possible channels of the DL frequency band of operation. If a partially configured carrier doesn’t have SCH or only part of SCH, then the MS only need to detect fully configured carrier by checking the corresponding synchronization sequences transmitted on fully configured carriers to finish synchronization. However, if the partially configured carrier has SCH, there comes the problem that if the MS could not acquire complete control and system information after finishing synchronization with the current carrier, it shall recognize it as a partially configured carrier and begin to scan other DL frequency again. That will surely lead to long access delay if MS has to decode the whole BCH information to find the configuration of the carrier. This contribution proposes a Superframe control header（SFCH）to solve this problem.

2 BCH Considerations in multi-carrier system

BCH can be divided into PBCH and SBCH. PBCH carries deployment-wide common information .Since all MSs in a cell complete network entry through fully configured carriers, each fully configured carrier needs to be configured with PBCH. Thus MS can get Deployment-wide common information during Initialization State. After network entry, if the serving BS assigns partially configured carriers to MS as secondary carriers or do carrier switch for the purpose of system load balancing or QoS requirement according to the MS’s multi-carrier capability, MS may use the PBCH information acquired during network access. It can be concluded that PBCH information could only be sent on fully configured carriers with fixed length.

SBCH carries downlink and uplink sector-specific information, which may change along with the sector’s configuration and the carrier’s operation.

Single-radio MS if uses a partially configured carrier as its secondary carrier should return to its primary carrier periodically or event-triggered for maintaining its control by the BS. It, therefore, could receive DL/UL resource allocation configuration and other channel configuration parameters, which are part of SBCH information on partially configured carrier for the convenience of work. Information about initial ranging may not be sent on partially configured carrier since it would not be used for the purpose of access.

As for multi-radio MS, it may simultaneously working on one or more fully configured carriers and one or more partially configured carriers, thus the SBCH information may be configured only on fully configured carriers.

In conclusion, two methods of sending SBCH information could be applied.

Option 1:

SBCH can be configured on both fully configured carrier and partially configured carrier.

Option 2:

SBCH could be configured only on fully configured carriers. SBCH information about partially configured carriers will be sent by one or more fully configured carriers MS is working with in the following three ways.

- Once a partially configured carrier is assigned to MS, or traffic switches from one partially configured carrier to another, SBCH information of the target partially configured carrier could be sent in unicast way to MS, which may be a complete one or just information different from the current carrier.

- Multicast could be a choice of sending SBCH information to a group of MSs which work on the same fully configured carrier and have been assigned the same target partially configured carrier.

- BS may also broadcast SBCH information of all partially configured carriers on all fully configured carriers. In this case, when finish accessing one of the fully configured carrier, MS could obtain Multi-carrier information about the system, including the frequency number, downlink/uplink bandwidth, downlink/uplink center frequency and the corresponding carrier configuration information.

3 Other DL control information in multi-carrier system

Extended system parameters and system configuration information is used by all MSs after access. It may include parameters for bandwidth request based on CDMA ranging and parameters for periodic ranging, etc. Partially configured carrier needs bandwidth request and periodic ranging for traffic exchange, so these parameters should be sent both on fully and partially configured carrier. Extended system parameters and system configuration information can be sent on these two kinds of carriers based on the function and the frequency in use.

Control and signaling for DL notifications is used by the BS to transmit network notifications to a single user or a group of users in the power saving mode (idle mode or sleep mode).MS may enter idle mode and only monitor its primary carrier (fully configured) for DL traffic notification and paging message. Sleep is a mode that permits MS’s absence from the air interface for some agreed intervals. The powers saving classes are defined based on connection so the traffic trigger may be received on fully configured carrier or partially configured carrier.

Control and signaling for traffic is mapped on USCCH. Fully /partially configured carrier may have its own USCCH, so this information could be sent on these two carriers.

The configuration of DL Control information on fully/partially carrier is shown in Table 1.

Table 1 DL Control information configuration on fully/partially carrier

| Information |  | Channel | Carrier configuration |
| --- | --- | --- | --- |
| Essential system parameters and system configuration information | Deployment-wide common information | PBCH | Fully |
|  | Downlink sector-specific information | SBCH | Fully Partially |
|  | Uplink sector-specific information |  |  |
| Extended system parameters and system configuration information |  | FFS | Fully Partially |
| Control and signaling for DL notifications |  | FFS | Fully Partially |
| Control and signaling for traffic |  | USCCH | Fully Partially |

4 Superframe control header (SFCH) design considerations

In a Multi-carrier system, an MS will finish the network entry procedure on fully configured carrier since fully configured carrier is configured with all synchronization, broadcast, multicast and unicast control signaling channels. According to the analysis above, fully configured carrier is configured with complete BCH information, while partially configured carrier may not sent BCH information or only part of SBCH information. In the case that both fully and partially configured carrier transmit the whole SCH, MS scans DL frequency band for possible channel, if MS could not acquire complete essential system parameters and system configuration information on BCH after finishing synchronization with the current carrier, it shall recognize it as a partially configured carrier and begin to scan another DL frequency again. Long access delay will be introduced if MS has to decode the whole BCH information to acquire the configuration of the carrier.

This contribution proposes to use a Superframe control header (SFCH) in the case that MS could not identify whether a carrier is fully configured or partially configured after synchronization, as described in last paragraph, to indicate the configuration of a carrier for the sake of accelerating the MS access procedure.

The PBCH and SBCH are transmitted using predetermined modulation and coding schemes .Since only fully configured carriers need to be configured with PBCH, a PBCH indicator (PI) Bit in SFCH is used to specify the existence of PBCH. PI=1 means PBCH is present. PBCH is transmitted using predetermined scheme with fixed length and right after the SFCH, if PI=1. The purpose of SBCH indicator（SI）Bit and SBCH length is to indicate the presence and the size of SBCH, which should be known by MS before decoding SBCH. The SFCH should be transmitted with predefined transmission format, length, MCS and location so that the MS can decode this without any other information. SFCH may be TDM with SCH and followed by BCH. Thus MS only need to decode SFCH after synchronization to determine whether the current carrier is fully configured and continue to decode the whole BCH.

The SFCH format is shown in Table 1.

Table 1 SFCH format

| Syntax | Size | Notes |
| --- | --- | --- |
| PBCH indicator(PI) | 1 | 0: PBCH is not present 1: PBCH is present |
| SBCH indicator(SI) | 1 | 0: SBCH is not present 1: SBCH is present |
| SBCH length |  |  |
| Reserved |  |  |

5 Proposed Text for SDD

[Insert the following text into Section 19.x of SDD 003r4:]

------------------------------- Text Start ----------------------------------------------------------------------------------

19.x DL broadcast information for Multi-carrier support

Each fully configured carrier needs to be configured with PBCH. After network entry, the serving BS may assign another carrier to MS as secondary carrier or do carrier switch for the purpose of system load balancing or QoS requirement according to the MS’s multi-carrier capability. PBCH information could only be sent on fully configured carriers with fixed length.

SBCH information may be transmitted through either of the following two options:

Option 1:

SBCH can be configured on both fully configured carrier and partially configured carrier.

Option 2:

SBCH can be configured only on fully configured carriers. SBCH information about partially configured carriers will be sent by one or more fully configured carriers MS is working with.

19.x.1 Superframe control header(SFCH)

A Superframe control header (SFCH) is used to indicate the configuration of a carrier in the case that partially configured carrier has SCH. A PBCH indicator (PI) Bit in SFCH specifies the existence of PBCH. PI=1 means PBCH is present. PBCH is transmitted using predetermined scheme with fixed length and right after the SFCH, if PI=1. SFCH indicator（SI）Bit and SBCH length are used to indicate the presence and the size of SBCH. SFCH should be transmitted with predefined transmission format, length, MCS and location. SFCH may be TDM with SCH and followed by BCH.

The SFCH format is shown in Table 1.

Table 1 SFCH format

| Syntax | Size | Notes |
| --- | --- | --- |
| PBCH indicator(PI) | 1 | 0: PBCH is not present 1: PBCH is present |
| SBCH indicator(SI) | 1 | 0: SBCH is not present 1: SBCH is present |
| SBCH length |  |  |
| Reserved |  |  |

------------------------------- Text End ------------------------------------------------------------------------------------

5 References

[1] IEEE Standard for Local and metropolitan area networks,

Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems, P802.16Rev2/D6 (July 2008)

[2] IEEE 802.16m-08/003r4, “The Draft IEEE 802.16m System Description Document”