| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Paging Mechanism in Multi-Carrier Operation |  |
| Date Submitted | 2008-09-05 |  |
| Source(s) | Lina Liu, Junxian Mo, Jia Lin Huawei Technologies Co.,Ltd. | Email: liulina@huawei.com |
| Re: | IEEE 802.16m-08/033: Call for Comments and Contributions on Project 802.16m System Description Document (SDD). Target topic: “Multi - Carrier Operation”, MAC aspect. |  |
| Abstract | This contribution defines the paging carrier for the MS to receive paging messages |  |
| Purpose | For discussion and approval by IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Paging Mechanism in Multi-carrier Operation

Lina Liu, Junxian Mo, Jia Lin

Huawei Technologies Co.,Ltd.

# Introduction

To achieve the peak data in SRD, multi-carrier operation has been included in the SDD. The wideband MS has one primary carrier but one or more secondary carriers. After the service is finished, the MS can enter idle mode to achieve power savings.

# Discussion

The MS shall enter idle mode from the primary carrier, and before entering idle mode, the MS shall close the transceivers on the secondary carriers. In this contribution, the concept of a paging carrier is defined as the carriers which the paging messages are transmitted. The paging carrier can be MS based, BS based or paging group based. The paging carrier can be the primary carrier of the MS in the connected state or other carriers which have the capability to send the paging messages.

We suggest that a MS only be assigned one paging carrier when it enters Idle Mode, under this principle, the BS should only page the MS on its paging carrier, and the MS just monitor its paging carrier for receiving the paging messages, which results in more efficient power management and reduce the system overhead. The paging carrier can be broadcast by the BS or negotiated between MS and BS during the deregistration procedure.

The MS should perform synchronize operation on its paging carrier after it enters Idle Mode, if the MS moved to another BS and can not find its paging carrier, then it shall perform enter/re-enter network procedure.

# SDD Text Proposal

The following text is proposed to be captured in the IEEE 802.16m system description document (SDD) .

================================= Start of Proposed Text =============

19.x  Idle Mode and Paging in Multi-carrier

The MS shall enter idle mode from the primary carrier, and before entering idle mode, the MS shall close the transceivers on the secondary carriers. The concept of a paging carrier is defined as the carriers which the paging messages are transmitted. The paging carrier can be MS based, BS based or paging group based. The paging carrier can be the primary carrier of the MS in the connected state or other carriers which have the capability to send the paging messages.	A MS only be assigned one paging carrier when it enters Idle Mode, the BS should only page the MS on its paging carrier.

The MS should perform synchronize operation on its paging carrier after it enters Idle Mode, if the MS moved to another BS and can not find its paging carrier, then it shall perform enter/re-enter network procedure.

==================================== End of the Text ================================