| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Bandwidth Request indicator for uplink FFR |  |
| Date Submitted | 2008-09-05 |  |
| Source(s) | Junichi Suga Fujitsu | E-mail: suga.junichi@jp.fujitsu.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | SDD Session 56 Cleanup: BW request channel IEEE 802.16m-08/033, Call for Contributions and Comments on Project 802.16m System Description Document (SDD)” |  |
| Abstract | To propose to add FFR group indication to BW request indicator |  |
| Purpose | To propose to add FFR group indication to BW request indicator |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Bandwidth Request Indicator for uplink FFR

Junichi Suga

Fujitsu

# Introduction

Two random access bandwidth request procedures have been adapted in SDD. In 5-step regular procedure, after MS receives a UL grant for BW-REQ message from BS, MS sends a BW-REQ message to BS through slots which BS indicates by the UL grant for BW-REQ message.

When FFR is used for uplink, a shared Bandwidth request channel might be allocated to a UL sub-frame for some FFR group region for the resource efficiency. Figure 1 shows the UL sub-frame with two FFR group regions and one shared Bandwidth request channel. In this case, it is efficient that BS allocates slots for MS sending a BW-REQ message on an FFR group region which is suitable for MS.

Figure 1 Shared Bandwidth request channel

# Proposed Bandwidth Request Method

In order that BS allocates slots on a suitable FFR group region for MS sending a BW-REQ message, BS informs MS of an FFR group selection rule by a broadcast message. This FFR group selection rule includes some thresholds such as MS Tx Power, CINR and interference. And MS sends a Bandwidth Request indicator with FFR Group indication.

When MS sends a bandwidth request indicator to BS, MS selects an FFR group region to send BW-REQ message according to the FFR group selection rule. Then, MS sends a bandwidth request indicator with the FFR group indication corresponding to the selected FFR group. BS, which receives a bandwidth request indicator with FFR group indication, allocates UL grant to send BW-REQ message on the selected FFR group region.

Figure 1 shows the proposed bandwidth request procedure for uplink FFR.

Figure 1. Bandwidth Request Procedure for uplink FFR

# Proposed Text to the SDD

[Change the first paragraph of 11.9.2.5 as indicated]

11.9.2.5 Bandwidth Request Channel

Contention based or non-contention based random access is used to transmit a bandwidth request indicator on this control channel. To support different levels of QoS, the bandwidth request channel provides a mechanism for prioritized bandwidth requests. Inclusion of additional information in bandwidth request indicator such as bandwidth request size, MS-ID, flow identifier, uplink transmit power and report CINR report is FFS. A bandwidth request indicator can include bandwidth request size, MS-ID, flow identifier, uplink transmit power report, CINR report and FFR group indication.

[Change the second paragraph of 11.9.2.5 as indicated]

The random access bandwidth request procedure is described in Figure 35. A 5-step regular procedure (step 1 to 5) or an optional quick access procedure (step 1, 4 and 5) may be supported concurrently. Step 2 and 3 are used only in 5-step regular procedure. In step 1, MS sends a bandwidth request indicator that may indicate information such as MS addressing and/or request size (FFS) and/or uplink transmit power report (FFS) and FFR group indication when FFR is used for uplink, and the BS may allocate uplink grand based on certain policy. The 5-step regular procedure is used independently or as fallback mode for quick access procedure. The MS may piggyback additional BW-REQ information along with user data during uplink transmission (step 5).