| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IEEE 802.16m ID Management for Location Privacy |  |
| Date Submitted | 2008-09-16 |  |
| Source(s) | GeneBeck Hahn, KiSeon Ryu and Ronny YongHo Kim LG Electronic Inc. LG R&D Complex, 533 Hogye-1dong, Dongan-gu, Anyang, 431-749, Korea Kanchei (Ken) Loa, Shiann-Tsong Sheu, Whai-En Chen, Chih-Cheng Yang, Yi-Ting Lin, Yung-Ting Lee, Youn-Tai Lee, Hua-Chiang Yin, Yi-Hsueh Tsai, Chun-Yen Hsu Yang-Han Lee, Yih Guang Jan | Voice: +82-31-450-7188 E-mail: gbhahn@lge.com, ksryu@lge.com and ronnykim@lge.com Institute for Information Industry Tamkang University |
| Re: | Re: MAC: Security; in response to the TGm Call for Contributions and Comments in 802.16m-08/033 for Session 57 |  |
| Abstract | This contribution describes a method of IEEE 802.16m ID management to ensure location privacy. |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

IEEE 802.16m ID Management for Location Privacy

Gene Beck Hahn, Ki Seon Ryu and Ronny Yong Ho Kim

LG Electronic

Kanchei (Ken) Loa, Shiann-Tsong Sheu, Whai-En Chen, Chih-Cheng Yang, Yi-Ting Lin, Yung-Ting Lee, Youn-Tai Lee, Hua-Chiang Yin, Yi-Hsueh Tsai, Chun-Yen Hsu

Institute for Information Industry

Yang-Han Lee, Yih Guang Jan

Tamkang University

# Problem Definition

The threat to location privacy is made from active or passive attacks to compromise the MS (Mobile Station) MAC address. There are few ongoing works to offer strong level of user identity and location confidentiality. Ideally, the user identity and location information shall be protected against various types of attacks. There have been many works illustrating the necessity of location privacy support in wireless networks [3][4][5][6]. When the location privacy is not supported, attackers can find where a user is, and can further use this information to track the user. This is because a user’s communications can be easily correlated in case all communications come from the same address and the address can be used to trace user’s movements. Currently, there are few safeguards on location privacy although location-based services are emerging as the future killer applications in wireless devices. The wireless privacy protection act of 2003, currently under consideration by US congress, proposes to revise the communications act of 1934, “To require customer consent to the provisions of wireless call location information.”

In legacy IEEE 802.16 systems, MAC address is a globally unique value for each IEEE 802.16-based device. In many situations, MAC address is treated as a means for authentication or as an identifier to grant a varying level of network privilege to a user. That is, MAC address of a user is matched to the authentication credentials and network permits the communication depending on a list of authorized MAC addresses [1]. Hence, the location of a user can be easily inferred from fixed MAC address.

According to IEEE 802.16m SRD, IEEE 802.16m shall include a security function which provides the necessary means to achieve the protection and confidentiality of user-generated and user-related data (e.g., location privacy, user identity). In IEEE 802.16 systems, MS MAC address (i.e., user identity) is sent from MSs to BS in an unprotected way (during initial ranging), result of which violates security aspects of IEEE 802.16m SRD. Hence, low cost solution to protect the user and identity location in the legacy IEEE 802.116 systems shall be proposed [2].

# IEEE 802.16m ID Management using Temporary Station Identifier

During initial ranging, BS allocates new temporary station identifier that can be used instead of MS MAC address. This can avoid the compromise of MS MAC address sent through air interface while minimizing network overhead incurred from the use of temporary station identifier. For doing this, network shall store the mapping of temporary station identifier and flow ID prior to the initiation of authorization phase. During authorization phase, the encrypted MS MAC address is used instead of temporary station ID. To identify MS, the further management messages are exchanged between MS and BS by using the permanent station identifier to identify MS after the authorization is completed.

Figure 1 depicts the allocation of temporary station identifier to MS during initial ranging. As we can see, BS uses temporary station identifier instead of MS MAC address to allocate resources to MS. The lightweight allocation procedure of temporary station ID meets the requirements of 802.16m SRD with regard to location privacy. By using the temporary station ID, MS MAC address is not revealed via air interface and thus can be protected.

Figure 1: Network Entry Procedure to Support MS Location Privacy in IEEE 802.16m

# References

[1] IEEE 802.16e Rev2/D2, Part 16: Air interface for Broadband Wireless Access Systems, December 2007

[2] IEEE 802.16m-07/002r2150, Draft IEEE 802.16m Requirements, 2007-06-08

[3] “A Framework for Location Privacy in Wireless Networks”, Yin-Chun Hu, Helen J. Wang, ACM SIGCOMM Asia Workshop 2005, April 12-14, 2005

[4] “A Study on the value of Location Privacy”, Dan Cvrcek, Marek Kumpost, WPES’06, October 30, 2006

[5] “How Much is Location Privacy Worth?”, George Danezis, Stephen Lewis and Ross Anderson, Workshop on the Economics of Information Security, June 1-3, 2005

[6] “Towards Mobile Internet : Location Privacy Threats and Granular Computation Challenges”, Ling-Lu, Granular Computing 2007, IEEE International Conference on, Nov 2-4, 2007

Text Proposal for IEEE 802.16m SDD

============================= Start of Proposed Text =============================

# 12 Security

# 12.1 Location Privacy Support

# 12.1.x ID Management

Figure 1: Network Entry Procedure to Support MS Location Privacy in IEEE 802.16m

For location privacy support, the temporary station ID is utilized instead of MS MAC address for initial network entry till the permanent station ID is allocated. The temporary station ID is assigned to MS by BS during initial ranging. Figure 1 shows allocation of temporary station ID to MS. This will avoid the compromise of MS MAC address sent via air interface while minimizing network overhead incurred from the use of temporary station ID. For doing this, BS manages the mapping of temporary station ID and flow ID prior to the initiation of authorization phase. Before the authorization, BS uses temporary station ID to allocate resources to MS. During and after the authorization phase, encrypted MS MAC address sent from MS to BS. BS assigns permanent station ID immediately after it receives encrypted MAC address from MS. To allocate resources to MS, BS then utilizes permanent station ID. Through the use of such temporary station ID, identity privacy is ensured during initial ranging. The lightweight assignment procedure of temporary station ID meets the requirements of IEEE 802.16m SRD regarding location privacy.

============================= End of Proposed Text =============================