| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IEEE 802.16m Protocol Structure |  |
| Date Submitted | 2008-09-05 |  |
| Source(s) | Ronny Yongho Kim, Jin Lee LG Electronics | Voice: +82-31-450-1879 Email: {ronnykim, jins978}@lge.com |
| Re: | MAC: SDD Session 56 Cleanup- 802.16m protocol structure |  |
| Abstract | This contribution is to propose to classify multi-radio coexistence functional block as RRCM functions. |  |
| Purpose | Discuss and adopt by TGm for SDD Text changes |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

The IEEE 802.16m Protocol Structure

Ronny Yongho Kim, Jin Lee

LG Electronics

1. Introduction

MAC common Part Sub layer is classified into Radio Resource Control and Management (RRCM) functions and Medium Access Control (MAC) functions in 802.16m protocol structure.

According to Figure 9, multi-radio coexistence function is classified as one of the MAC functions. However, multi-radio coexistence function needs to obtain information about other co-located radio activities in order to perform co-existence operation. The information can be carried out via M_SAP/C_SAP to communicate with other co-located radios. NCMS may define co-existence service to tunnel into non 802.16m radios so that information on other co-located radio activities can be gathered by the co-existence service.

This contribution proposes to modify the Figure 11 in order for multi-radio coexistence functional block to communicate with NCMS through M_SAP/C_SAP.

2. Proposed Text

1. Modify Figure 11 and insert “multi-radio coexistence” as in text proposal.

2. Move “Multi-Radio Coexistence” function block from MAC to RRCM in figure 9, 10

3. Move “Multi-Radio Coexistence” in line 34 page 19 to after line 22, page 18.

4. Move line 23~24 page 20 to after line 23 page 19.

-------------------------------------------------- Start of Text Proposal ---------------------------------------------------------

8.1.2 The IEEE 802.16m MS/BS Control Plane Processing Flow

This primitives to/from M_SAP/C_SAP define the network involved functionalities such as inter-BS interference management, inter/intra RAT mobility management, etc, and management related functionalities such as location management, system configuration, multi-radio coexistence etc.

Figure 11 The IEEE 802.16m MS/BS Control Plane Processing Flow Note: The MS may not utilized all the blocks shown in this figure

-------------------------------------------------- End of Text Proposal ----------------------------------------------------------