| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for DL OL SU-MIMO Transmit Diversity Scheme of Rate 2 |  |
| Date Submitted | 2008-09-05 |  |
| Source(s) | Yinggang Du, Hongjie Si, Yin Wei Zhao, Xin Chang, Yang Tang Huawei Peter Wang, Adrian Boariu, Chao Wei, Joon Chun, Shaohua Li, Xin Qi NSN Zexian Li, Andrei Malkov Nokia Hosein Nikopourdeilami, Mo-Han Fong, Jun Yuan, Sophie Vrzic, Robert Novak, Dongsheng Yu, Kathiravetpillai Sivanesan Nortel Isamu YOSHII, Takaaki KISHIGAMI Matsushita Electric (Panasonic) Xiaolu Dong, Ying Du CATR Wataru Matsumoto, Toshiyuki Kuze Mitsubishi Electric Corporation. Philip V. Orlik, Andreas F. Molisch, Ramesh Annavajjala Mitsubishi Electric Research Lab. | Email: duyinggang@huawei.com peter.wang@nsn.com hosein@nortel.com, mhfong@nortel.com yoshii.isamu@jp.panasonic.com kishigami.takaaki@jp.panasonic.com Dongxiaolu@mail.ritt.com.cn duying@mail.ritt.com.cn molisch@merl.com |
| Re: | IEEE 802.16m-08/033 “Call for Contributions and Comments on Project 802.16m System Description Document (SDD)” Target topic: “PHY : MIMO, Call for Detailed Physical Layer Comments” |  |
| Abstract | This contribution proposes DL OL SU-MIMO schemes |  |
| Purpose | To develop harmonized SDD text for 802.16m on DL OL SU-MIMO schemes |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for DL OL SU-MIMO Transmit Diversity Scheme of Rate 2

Yinggang Du, Hongjie Si, Yinwei Zhao, Xin Chang, Yang Tang

Huawei

Peter Wang, Adrian Boariu, Chao Wei, Joon Chun, Shaohua Li, Xin Qi

# NSN

# Zexian Li, Andrei Malkov
Nokia

Hosein Nikopourdeilami, Mo-Han Fong, Jun Yuan, Sophie Vrzic,

Robert Novak, Dongsheng Yu, Kathiravetpillai Sivanesan

Nortel

Isamu YOSHII,  Takaaki KISHIGAMI

Matsushita Electric (Panasonic)

Dong Xiaolu, Du Ying

CATR

# 1. Introduction

This contribution is to propose DL OL SU-MIMO scheme in response to C802.16m MIMO-08/005r1.  We propose a 4Tx/8Tx antennas rate 2 scheme to be included in the MIMO SDD RG document. The D-STTD simulation results can be referred in C802.16m MIMO-08/056(Huawei) and C802.16m MIMO-08/058(Nortel).

2. Multiplexing scheme with 4 TX antennas and rate 2

# Text Proposal modification to SDD

(L19, P68 of 003r4)

# Open-loop SU-MIMO

| NT | Rate | M | NF |
| --- | --- | --- | --- |
| 2 | 1 | 1 | 1 |
| 2 | 1 | 2 | 2 |
| 4 | 1 | 1 | 1 |
| 4 | 1 | 2 | 2 |
| 8 | 1 | 1 | 1 |
| 8 | 1 | 2 | 2 |
| 2 | 2 | 2 | 1 |
| 4 | 2 | 4 | 2 |
| 8 | 2 | 4 | 2 |
| 4 | 3 | 3 | 1 |
| 8 | 3 | 3 | 1 |
| 4 | 4 | 4 | 1 |
| 8 | 4 | 4 | 1 |

Table 5 Matrix dimensions for open-loop SU-MIMO modes

(L8, P70 of 003r4)

# Spatial Multiplexing

The following spatial multiplexing modes are supported for open-loop single-user MIMO:

- Rate-2 spatial multiplexing modes:

  - 2Tx rate-2: rate 2 SM

  - 4Tx rate-2: rate 2 D-STTD

  - 8Tx rate-2: rate 2 D-STTD

[Delete the content from L31 to L41in P70 and Insert the following text in the section 11.8.2.1.1.2 of 80216m-08_003r4.]

For 4Tx antennas rate2 mode, the input to the MIMO encoder is represented as a 4  1 vector

x=s1s2s3s4

The output of the MIMO encoder is a 4  2 matrix

z=s1−s2s2s1s3−s4s4s3

the output of the precoder is a 4  2 matrix

y=D×W×z,

where W is a 4  4 unitary precoder and D is a 4  4 identity matrix (D=I).

When using Antenna Hopping, W is a set of 3 antenna circulation matrices, i.e.,

W=1000010000100001,1000001001000001,1000001000010100.

W can be changed every pair of tones or symbols.

For 8Tx antennas rate2 mode, the input to the MIMO encoder is represented as a 4  1 vector

x=s1s2s3s4

The output of the MIMO encoder is a 4  2 matrix

z=s1−s2s2s1s3−s4s4s3

the output of the precoder is a 4  2 matrix

y=D×W×z,

where D is a 8  8 identity matrix (D=I) and W is defined as follows:

W=W1×W2.

W1 is a 8x4 matrix which is implementation specific and W2 is a 4  4 unitary precoder.

When using Antenna Hopping, W2 is a set of 3 antenna circulation matrices, i.e.,

W=1000010000100001,1000001001000001,1000001000010100.

W can be changed every pair of tones or symbols.

------------------------------------End text proposal------------------------------------