| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IEEE 802.16m SDD changes for Security |  |
| Date Submitted | 2009-01-14 |  |
| Source(s) | Shashikant Maheshwari, Haihong Zheng, Yousuf Saifullah Nokia Siemens Networks Jan Suumaki Nokia | E-mail: shashi.maheshwari@nsn.com E-mail: jan.suumaki@nokia.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | 802.16m-08/052, Call for Comments on 802.16m SDD (802.16m-08/003r6) Section 10.6 Security |  |
| Abstract | This contribution proposes changes to security text (Chapter 10.6) for the 802.16m SDD |  |
| Purpose | For review and adoption into 802.16m System Description Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

IEEE 802.16m SDD Text Proposal for Security

Shashikant Maheshwari, Haihong Zheng, Yousuf Saifullah

Nokia Siemens Networks

Jan Suumäki

Nokia

# Introduction

This contribution proposes changes for the section “10.6 Security” in IEEE 802.16m System Description Document [1].

The purpose of these changes is to align and clean up SDD text throughout security section.

This contribution proposes the following changes to chapter 10.6.1:

Change 1: Updating and aligning figure 17 and text in clause 10.6.1 Security Architecture.

Other proposed changes:

- Word ‘Processing’ is not relevant for the architecture, thus it is proposed to remove.

- Reference to sub-clauses added when possible.

- Capital/small letter usage unified

- ‘User data’ replaced with ‘transport data’. This transport data term is more commonly used.

Change 2: Updating and aligning figures 18 and 19 and text in clause 10.6.3.2.

Also clarifying key exchange procedure in general and proposing to remove all FFSs.

Details of distribution and update mechanisms could be defined only in stage 3.

Change 3: Changing clause 10.6.5.1.2 to more generic because currently it is too detailed for SDD and overlaps with MPDU header design.

Also currently only PN is agreed to use in AES-CCM (no ROC or such supported)

Additionally chapter name is proposed to change to ‘Multiplexing and encryption of MPDUs’ because scope should be security in this ‘10.6 Security’ chapter.

# Proposed Changes to SDD

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

[Change 1]

10.6.1 Security Architecture

…

Figure 17 Functional Blocks of IEEE 802.16m Security Architecture

Within AMS and ABS the security architecture is divided into two logical entities:

• Security management entity

• Encryption and integrity entity

Security management entity functions include:

• Overall security management and control

• EAP encapsulation/decapsulation for authentication & authorization - see 10.6.2

• Privacy Key Management (PKM) control

(e.g. key generation/derivation/distribution, key state management) - see 10.6.3

• Authorization and Security Association (SA) control - authorization is described in 10.6.2 and SA control in 10.6.4

• Location privacy - see 10.6.2.1

[Change 2]

10.6.3.2 Key Exchange

The key exchange procedure is controlled by the security key state machine, which defines the allowed

operations in the specific states. The key exchange state machine is similar to the reference system

The main difference is that instead of the exchanging the traffic encryption keys as in the reference system, only Nonce is exchanged. This Nonce is then  used to derive traffic encryption keys (TEK / GTEK)

locally.

The ABS and the AMS derive the TEK / GTEK through the key derivation mechanism at each side respectively.

In addition of Nonce also some other security material like cipher suite may be exchanged.

The Nonce and other security material may be exchanged with the following messages:

• Key Request / Reply

• Key Agreement

• Ranging

Figure 18 Initial or Re-authentication - Key Derivation and Exchange

Figure 19 Key Update Procedure

[Change 3]

10.6.5.1.2 Multiplexing and encryption of MPDUs

When some connections identified by flow ids are mapped to the same SA, their payloads can be multiplexed

together into one MPDU. The multiplexed payloads are encrypted together. In Figure 20, Flow_x and Flow_y

have payload x and y respectively which are mapped to the same SA. The MAC header provides the details of

payloads which are multiplexed.

Note that the multiplexed MPDU format in Figure 20 can be changed according to mechanism for single

MDPU encryption.

Figure 20 multiplexed MAC PDU format

In case of the multiplexed MPDU, the multiplexed MPDU is encrypted by using Packet Number (PN) of the first

flow PDU only. Hence the other flow’s PNs are to be omitted, but the PNs are maintained per flow

implicitly..

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

# References

[1] The Draft IEEE 802.16m System Description Document, IEEE 802.16m-08/003r6