| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed SDD Text for DL OL SU-MIMO |  |
| Date Submitted | 2009-01-13 |  |
| Source(s) | Hosein Nikopourdeilami, Mo-Han Fong, Sophie Vrzic, Robert Novak, Dongsheng Yu, Kathiravetpillai Sivanesan Nortel Networks Sylvain Labonte Sequan | Email: hosein@nortel.com mhfong@nortel.com sylvain@sequans.com |
| Re: | TGm SDD section 11.8.2.1.1 In response to IEEE 802.16m-08/052 “Call for Contributions and Comments on Project 802.16m System Description Document (SDD)” for Session 59 |  |
| Abstract | This contribution proposes SDD text for DL OL SU-MIMO schemes |  |
| Purpose | For discussion and approval into TGm SDD text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed SDD Text for DL OL SU-MIMO

# Hosein Nikopourdeilami, Mo-Han Fong, Sophie Vrzic, Robert Novak, Dongsheng Yu, Kathiravetpillai Sivanesan

# Nortel Networks

Sylvain Labonte

Sequan

# 1. Introduction

This contribution is to propose DL OL SU-MIMO schemes.

# 2. Text Proposal modification to SDD

(L5, P100 of C802.16m-08/003r6)

# 11.8.2.1.1 Open-loop SU-MIMO

| NT | Rate | M | NS | NF |
| --- | --- | --- | --- | --- |
| 2 | 1 | 2 | 2 | 2 |
| 4 | 1 | 2 | 2 | 2 |
| 8 | 1 | 2 | 2 | 2 |
| 2 | 2 | 2 | 2 | 1 |
| 4 | 2 | 2 | 2 | 1 |
| 4 | 2 | 4 | 4 | 2 |
| 8 | 2 | 2 | 2 | 1 |
| 8 | 2 | 4 | 4 | 2 |
| 4 | 3 | 3 | 3 | 1 |
| 8 | 3 | 3 | 3 | 1 |
| 4 | 4 | 4 | 4 | 1 |
| 8 | 4 | 4 | 4 | 1 |

Table 1 Matrix dimensions for open-loop SU-MIMO modes

[modify section 11.8.2.1.1.1 and 11.8.2.1.1.2 of C802.16m-08/003r6 as follows]

# 11.8.2.1.1.1 Transmit Diversity

The following transmit diversity modes are supported for open-loop single-user MIMO:

- 2Tx rate-1: SFBC

- 4Tx rate-1: SFBC with precoder

- 8Tx rate-1: SFBC with precoder

For the transmit diversity mode, the input to the MIMO encoder is represented a 2 × 1 vector.

x=s1s2,                            (Equation 11.8.2.1.1.1-1)

The MIMO encoder generates the SFBC matrix.

,                          (Equation 11.8.2.1.1.1-2)

Then the output of the MIMO encoder is multiplied by NT × 2 precoder matrix W, where W is described in section .

For the 2Tx rate-1 mode, the output of the precoder is a 2  2 matrix

y=z,                             (Equation 11.8.2.1.1.1-3)

For the 4Tx rate-1, the output of the precoder is a 4  2 matrix

y=W×z,                          (Equation 11.8.2.1.1.1-4)

where W is a 4  2 unitary precoder. Note that W may be frequency and/or time dependent as described in section 11.8.2.1.1. W is a set of Np=6 antenna circulation matrices.

W=10010000,10000100,10000001,00100100,00100001,00001001,         (Equation 11.8.2.1.1.1-5)

W can be changed every pair of tones or every symbol.

For the 8Tx rate-1, the output of the precoder is a 8  2 matrix

y=W×z,                         (Equation 11.8.2.1.1.1-6)

where W is a 8  2 unitary precoder. Note that W may be frequency and/ or time dependent as described in section 11.8.2.1.1. W is defined as follows:

W=W1×W2,                       (Equation 11.8.2.1.1.1-7)

W1 is a 8x4 matrix which is implementation specific, W2 is a 4  2 unitary precoder which consists of a set of Np =6 antenna circulation matrices.

W2=10010000,10000100,10000001,00100100,00100001,00001001,      (Equation 11.8.2.1.1.1-8)

W2 can be changed every pair of tones or every symbol.

The permutation rule of the antenna hopping (AH) precoders for transmit diversity mode is defined as follows:

- Let f is the index of the subcarrier starting from 0 (0 <= f <= Nfft-1). Note that f denotes the index of a physical tone not a logical tone. Nfft is the FFT size according to the bandwidth.

- Let t denote the OFDM symbol index starting form 0 ( 0<= t <= 5)

- The AH precoder set contains Np (=6 for diversity mode) precoders

- n = floor( Np / 2 )

- p denotes the precoder index used for a specific tone ( 0<= p <= Np-1)

- p = { floor( t / n ) + t * floor( Np / n ) + floor( f / 2 ) } mod Np           (Equation 11.8.2.1.1.1-9)

# Spatial Multiplexing

The following spatial multiplexing modes are supported for open-loop single-user MIMO:

- Rate-2 spatial multiplexing modes:

  - 2Tx rate-2: rate 2 SM

  - 4Tx rate-2: rate 2 DSFBC with precoding and rate 2 SM with precoding

  - 8Tx rate-2: rate 2 DSFBC with precoding and rate 2 SM with precoding

- Rate-3 spatial multiplexing modes:

  - 4Tx rate-3: rate 3 SM with precoding

  - 8Tx rate-3: rate 3 SM with precoding

- Rate-4 spatial multiplexing modes:

  - 4Tx rate-4: rate 4 SM

  - 8Tx rate-4: rate 4 SM with precoding

For 2Tx antennas rate-2 mode, the input and output of the MIMO encoder and the output of the precoder are represented as a 2  1 vector.

y=z=x=s1                         (Equation 11.8.2.1.1.2-1)

For 4Tx antennas rate-2 mode, the input to the MIMO encoder is represented as a 4  1 vector (DSFBC case) or a 2 x 1 vector (SM case).

x=s1s2s3s4 for DSFBC, x=s1 for SM.       (Equation 11.8.2.1.1.2-2)

The output of the MIMO encoder is a 4  2 matrix (DSFBC case) or a 2  1 vector (SM case).

z=s1−s2s2s1s3−s4s4s3 for DSFBC, z =x=s1 for SM.      (Equation 11.8.2.1.1.2-3)

The output of the precoder is a 4  2 matrix (DSFBC case) or a 4  1 vector (SM case),

y=W×z,                        (Equation 11.8.2.1.1.2-4)

where W is a 4  4 unitary precoder (DSFBC case) or a 4  2 unitary precoder (SM case).

When using DSFBC with precoding, W is a set of Np=3 antenna circulation matrices.

W=1000010000100001,1000001001000001,1000001000010100,        (Equation 11.8.2.1.1.2-5)

When using SM with precoding, W is a set of Np =6 antenna circulation matrices.

W=10010000,00001001,10000100,00100001,10000001,00100100,     (Equation 11.8.2.1.1.2-6)

In DSFBC case,W can be changed every pair of tones or every symbol. In SM case, W can be changed every tone or symbol.

For 8Tx antennas rate-2 mode, the input to the MIMO encoder is represented as a 4  1 vector (DSFBC case) or a 2 x 1 vector (SM case).

x=s1s2s3s4 for DSFBC, x=s1 for SM,        (Equation 11.8.2.1.1.2-7)

The output of the MIMO encoder is a 4  2 matrix (DSFBC case) or a 4  1 vector (SM case)

z=s1−s2s2s1s3−s4s4s3 for DSFBC, z =x=s1 for SM,   (Equation 11.8.2.1.1.2-8)

the output of the precoder is a 4  2 matrix

y=W×z,                      (Equation 11.8.2.1.1.2-9)

where W is defined as follows:

W=W1×W2,                   (Equation 11.8.2.1.1.2-10)

W1 is a 8x4 matrix which is implementation specific, W2 is a 4  4 unitary precoder (DSFBC case) or 4 x 2 unitary precoder (SM case).

When using DSFBC with precoding, W2 is a set of Np =3 antenna circulation matrices.

W2=1000010000100001,1000001001000001,1000001000010100,  (Equation 11.8.2.1.1.2-11)

When SM with precoding, W2 is a set of Np =6 antenna circulation matrices.

W2=10010000,00001001,10000100,00100001,10000001,00100100, (Equation 11.8.2.1.1.2-12)

In DSFBC case,W2 can be changed every pair of tones or every symbol. In SM case,W2 can be changed every tone or symbol.

For the rate-3 spatial multiplexing modes, the input to the MIMO encoder is represented as a 3  1 vector.

x=s1s2s3,                     (Equation 11.8.2.1.1.2-13)

The output of the MIMO encoder is a 3  1 vector.

z=x,                      (Equation 11.8.2.1.1.2-14)

For the 4Tx rate-3 mode, the output of the precoder is a 4  1 vector.

y=W×z,                    (Equation 11.8.2.1.1.2-15)

where W is a 4  3 unitary precoder. Note that W may be frequency and/or time dependent as described in section 11.8.2.1.1. W is a set of Np =4 antenna circulation matrices.

W=100010001000,100010000001,100000010001,000100010001,  (Equation 11.8.2.1.1.2-16)

W can be changed every tone or symbol.

For the 8Tx rate-3 mode, the output of the precoder is a 8  1 vector.

y=W×z,                    (Equation 11.8.2.1.1.2-17)

where W is defined as follows:

W=W1×W2,                  (Equation 11.8.2.1.1.2-18)

W1 is a 8x4 matrix which is implementation specific, W2 is a 4x3 unitary precoder which consists of a set of Np =4 antenna circulation matrices.

W2=100010001000,100010000001,100000010001,000100010001,  (Equation 11.8.2.1.1.2-19)

W2 can be changed every tone or symbol.

For the rate-4 spatial multiplexing modes, the input to the MIMO encoder is represented as a 41 vector

x=s1s2s3s4,                    (Equation 11.8.2.1.1.2-20)

The output of the MIMO encoder is a 4  1 vector.

z=x,                     (Equation 11.8.2.1.1.2-21)

For the 4Tx rate-4 mode, the output of the precoder is a 4  1 vector.

y=z,                     (Equation 11.8.2.1.1.2-22)

For the 8Tx rate-4 mode, the output of the precoder is a 8  1 vector.

y=W×z,                  (Equation 11.8.2.1.1.2-23)

where W is a 8  4 precoder which is implementation specific.

The permutation rule of the AH precoders for spatial multiplexing mode (including SM and DSFBC) is defined as follows:

- Let f is the index of the subcarrier starting from 0 (0 <= f <= Nfft-1). Note that f denotes the index of a physical tone not a logical tone. Nfft is the FFT size according to the bandwidth.

- Let t denote the OFDM symbol index starting form 0 ( 0<= t <= 5)

- The AH precoder set contains Np (=3,4, or 6) precoders

- n = floor( Np / 2 )

- p denotes the precoder index used for a specific tone ( 0<= p <= Np-1)

- p = { floor( t / n ) + t * floor( Np / n ) + f } mod Np           for SM   (Equation 11.8.2.1.1.2-24)

- p = { floor( t / n ) + t * floor( Np / n ) + floor( f / 2 ) } mod Np  for DSFBC  (Equation 11.8.2.1.1.2-25)

------------------------------------End text proposal------------------------------------