| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MIMO Midamble |  |
| Date Submitted | 2009-01-05 |  |
| Source(s) | Woosuk Kwon Jinyoung Chun Wookbong Lee Sukwoo Lee Bin-Chul Ihm LG Electronics | kytloze@lge.com jychun03@lge.com wbong@lge.com sugoo@lge.com bcihm@lge.com |
| Re: | Call for Comments on Project 802.16m System Description Document (SDD), IEEE 802.16m-08/052 (DL PHY section) |  |
| Abstract | The contribution proposes the text of downlink reference signal section to be included in the 802.16m SDD. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MIMO Midamble

Woosuk Kwon, Jinyoung Chun, Wookbong Lee, Sukwoo Lee and Bin-Chul Ihm

LG Electronics

- Introduction

The contribution proposes the MIMO midamble structure for channel estimation of TX antennas. And it also can be used for estimating channel states of neighbor cells for interference mitigation techniques. (See the session 20.2 in SDD.)

- Considerations for MIMO midamble structure

In SDD, only demodulated pilots are specified. The demodulate pilots are used for data detection. To estimate CQI or PMI properly, the downlink reference signal shall not be precoded and the AMS shall know exactly which pilot pattern was used. Since USCCH is dedicated to a certain AMS, AMS does not know which pilot pattern was used or even the pilots are precoded or not. So, we need to have a downlink reference signal to estimate CQI or PMI. We can consider reuse the demodulated pilot pattern for downlink reference signal by using a whole subframe’s pilots as common pilot. However, for sounding based precoding or non-unitary codebook based precoding, it is difficult to indicating which precoder was used for that subframe. Moreover, it increases pilot overhead.

Downlink reference signal is for the measurement of channel states. Therefore we have to consider below bullet items to design downlink reference signal as below:

- To avoid existing pilot subcarriers.

- To estimate channel states of all physical transmit antennas.

- To be distinguished between neighbor cells.

- To be maintained pair-tone based DRU.

- The structure of MIMO midamble

Figure 1 (a) is a pilot pattern for 2 streams and (b) is a pilot pattern for 4 streams. In figure 1 (c), ‘P’ shows pilot location to gather all pilot including (a), (b) and interlaced pattern.

(a) 2 stream pilot pattern	(b) 4 stream pilot pattern	(c) all pilot location

Figure 1. Pilot patterns of a RU in SDD text

Figure 2 (a) shows a proposed midamble pattern for 2Tx antennas. Each antenna has 3 pilots in a RU. Figure 2 (b) shows a proposed midamble pattern for 4Tx antennas. Midamble pattern for 8 Tx antennas is FFS.

(a) Midamble pattern for 2Tx		(b) Midamble pattern for 4Tx

Figure 2. Proposed midamble patterns in a RU

- The performance of proposed MIMO midamble pattern

In this section, we provide MIMO midamble performance. AMS estimates the channels of antennas and calculates and reports the values of MCS, Rank, PMI and so on.

Figure 3. MIMO midamble performance

- Appendix

There are simulation parameters.

| Parameter | Values |
| --- | --- |
| Channel Bandwidth | 10MHz |
| Over-sampling Factor | 28/25 |
| FFT Size | 1024 |
| Cyclic Prefix (CP) ratio | 1/8 |
| Resource Unit | CRU (4x18x6) |
| Channel condition | PED_B 3km/h |
| The number of antennas | 2 Tx, 2 Rx |
| Channel estimation for measurement | 1D-MMSE |
| Channel estimation for demodulation | 2D-MMSE |
| Receiver | MMSE receiver |

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Oct. 2008.

[2] IEEE 802.16m-07/002r7, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”

- Text proposal for inclusion in the 802.16m SDD

-------------------------------  Text Start  ---------------------------------------------------

Insert 11.5.3.2 MIMO midamble section

11.5.3.2 MIMO Midamble

MIMO midamble is used for measurement of channel quality indicators, precoding matrix index and so on.

There are midamble patterns in Figure xxx.

(a) Midamble pattern for 2Tx		(b) Midamble pattern for 4Tx

Figure xxx. Midamble patterns

-------------------------------  Text End  ---------------------------------------------------