| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Channel Coding Block Diagram |  |
| Date Submitted | 2009-01-05 |  |
| Source(s) | Chien-Yu Kao, Cheng-Ming Chen, Yu-Tao Hsieh, Pang-An Ting, Zheng Yan-Xiu, Richard Li ITRI | Voice: E-mail: ythsieh@itri.org.tw |
| Re: | IEEE 802.16m-08/052 “Call for Comments and Contributions on Project 802.16m System Description Document (SDD)” |  |
| Abstract | This document proposes a new block diagram for channel coding and HARQ. |  |
| Purpose | For 802.16m discussion and adoption for standardization |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Block Diagram Modification for Channel Coding and HARQ

Chien-Yu Kao, Cheng-Ming Chen, Yu-Tao Hsieh, Pang-An Ting, Zheng Yan-Xiu, Richard Li

ITRI

# Introduction

In section 11.13.2.2 of 802.16m’s SDD [1], the constellation re-arrangement (CoRe) has been modified as bit re-arrangement (BitRe). Namely, different BitRe versions can be generated through the bit-level interleaver and bit inverter, etc. After that the rearranged bit-sequence is feed into constellation mapping (QAM modulation). Therefore, the original block diagram for channel coding and HARQ should be modified accordingly. Besides, it was agreed that an FEC block CRC is appended to each FEC block before the FEC encoding. Therefore, a block called FEC block CRC attachment can be added to the diagram for completeness.

# Proposal

We propose a modified block diagram for channel coding and HARQ as shown in the figure below. Two blocks are added: one is called “FEC block CRC attachment “, which is placed between the “Bust partition into FEC blocks” and “FEC encoder” blocks, and the other is called “Bit rearrangement”, which is extracted from the “Modulation” block. The relevant output bit lengths denoted along with each block are also modified.

Figure XX Channel coding block diagram

# Text Proposal to SDD

The original block diagram should be modified as below for section 11.13.1.1:

------------------------------------Start text proposal------------------------------------

------------------------------------End text proposal------------------------------------

# Reference

[1] IEEE 802.16m-08/003r6, “IEEE 802.16m System Description Document [Draft]”.