| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Consideration on Uplink HARQ |  |
| Date Submitted | 2009-01-05 |  |
| Source(s) | Wang Junhu, Sun Bo ZTE Corporation | Voice: [Telephone Number (optional)]] E-mail: wang.junhu@zte.com.cn Sun.bo1@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-08/052 –Call for Comments on Project 802.16m System Description Document (SDD) on Section “10.2.2 HARQ in the Uplink” |  |
| Abstract | This document proposes an uplink HARQ scheme for IEEE 802.16m |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Consideration on Uplink HARQ

Wang Junhu, Sun Bo

ZTE Corporation

# 1 Introduction

Synchronous HARQ　has been adopted for uplink HARQ in IEEE 802.16m SDD[1]. The following HARQ parameters are defined while their associated values need further investigation:

•  Maximum retransmission delay: FFS

•  Maximum number of retransmissions: FFS

•  Maximum number of HARQ processes FFS

•  ACK/NACK delay: FFS

For synchronous HARQ, ACK/NACK feedback and retransmission occur at predetermined time. Maximum Retransmission Delay (MRD) actually is the required delay for synchronous HARQ.

The uplink HARQ MRD is determined by DL/UL ratio， proceeding delays of ABS and AMS, and the distribution of DL ACK channels on downlink subframes. Since DL ACK/NACK feedback is transmitted in USCCH channel, we have to concern about the pattern of USCCH distribution as well.

In [2], a synchronous HARQ timing method was proposed, which sets allocation period to be an integer number of radio frames. This method simplifies the timing pattern. The contribution gives the parameters listed above in different UL/DL ratios.

# 2 Proposed Solution

In different UL/DL ratio, DL ACK channels are assigned to each UL HARQ process. The following tables are presented to associate the DL ACK channels with HARQ process with MRD included. And the figures illustrate the HARQ processes in a super-frame.

- UL/DL=2:6

| HARQ Process Index | DL ACK channel Index | MRD |
| --- | --- | --- |
| 1 | 2 | 5ms |
| 2 | 2 | 5ms |

2) UL/DL=3:5

| HARQ Process Index | DL ACK channel Index | MRD |
| --- | --- | --- |
| 1 | 2,5,8,11 | 5ms |
| 2 | 2，5,8,11 | 5ms |
| 3 | 3，9 | 10ms |
| 4 | 6，12 | 10ms |

3) UL/DL=4:4

| HARQ Process Index | DL ACK channel Index | MRD |
| --- | --- | --- |
| 1 | 2,6 | 10ms |
| 2 | 2，6 | 10ms |
| 3 | 3，7 | 10ms |
| 4 | 3，7 | 10ms |
| 5 | 4，8 | 10ms |
| 6 | 4，8 | 10ms |
| 7 | 5，（1） | 10ms |
| 8 | 5，（1） | 10ms |

4) UL/DL=5:3

| HARQ Process Index | DL ACK channel Index | MRD |
| --- | --- | --- |
| 1 | 1，3，5，7 | 5ms |
| 2 | 1，3，5，7 | 5ms |
| 3 | 1，3，5，7 | 5ms |
| 4 | 2，4，6，8 | 5ms |
| 5 | 2，4，6，8 | 5ms |

5) UL/DL=6:2

| HARQ Process Index | DL ACK channel Index | MRD |
| --- | --- | --- |
| 1 | 1，5 | 10ms |
| 2 | 1，5 | 10ms |
| 3 | 1，5 | 5ms |
| 4 | 2，6 | 5ms |
| 5 | 2，6 | 5ms |
| 6 | 3，7 | 10ms |
| 7 | 3，7 | 10ms |
| 8 | 3，7 | 10ms |
| 9 | 5，（1） | 10ms |

4  Text Proposal for the 802.16m SDD

=====================Start of Proposed Text =================================

10.2.2 HARQ in the Uplink

10.2.2.1 HARQ Timing and Protocol

IEEE 802.16m uses synchronous HARQ scheme in the uplink.

The following HARQ parameters and their associated values are defined in the following tables:

1) UL/DL=2:6

| HARQ Process Index | DL ACK channel Index | MRD |
| --- | --- | --- |
| 1 | 2 | 5ms |
| 2 | 2 | 5ms |

2) UL/DL=3:5

| HARQ Process Index | DL ACK channel Index | MRD |
| --- | --- | --- |
| 1 | 2,5,8,11 | 5ms |
| 2 | 2，5,8,11 | 5ms |
| 3 | 3，9 | 10ms |
| 4 | 6，12 | 10ms |

3) UL/DL=4:4

| HARQ Process Index | DL ACK channel Index | MRD |
| --- | --- | --- |
| 1 | 2,6 | 10ms |
| 2 | 2，6 | 10ms |
| 3 | 3，7 | 10ms |
| 4 | 3，7 | 10ms |
| 5 | 4，8 | 10ms |
| 6 | 4，8 | 10ms |
| 7 | 5，（1） | 10ms |
| 8 | 5，（1） | 10ms |

4) UL/DL=5:3

| HARQ Process Index | DL ACK channel Index | MRD |
| --- | --- | --- |
| 1 | 1，3，5，7 | 5ms |
| 2 | 1，3，5，7 | 5ms |
| 3 | 1，3，5，7 | 5ms |
| 4 | 2，4，6，8 | 5ms |
| 5 | 2，4，6，8 | 5ms |

5) UL/DL=6:2

| HARQ Process Index | DL ACK channel Index | MRD |
| --- | --- | --- |
| 1 | 1，5 | 10ms |
| 2 | 1，5 | 10ms |
| 3 | 1，5 | 5ms |
| 4 | 2，6 | 5ms |
| 5 | 2，6 | 5ms |
| 6 | 3，7 | 10ms |
| 7 | 3，7 | 10ms |
| 8 | 3，7 | 10ms |
| 9 | 5，（1） | 10ms |

=====================end of Proposed Text =================================

6 References

[1] IEEE 802.16m-08/003r6,IEEE 802.16m System Description Document [Draft]

[2]C802.16m-08/340, 802.16m HARQ, Huawei Technologies