| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Maximum Number of Retransmission for HARQ |  |
| Date Submitted | 2009-01-05 |  |
| Source(s) | Wang Junhu, Sun Bo ZTE Corporation | Voice: [Telephone Number (optional)]] E-mail: wang.junhu@zte.com.cn Sun.bo1@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-08/052 –Call for Comments on Project 802.16m System Description Document (SDD) on Section “10.2 HARQ Functions” |  |
| Abstract | This document proposes the value of the maximum number of retransmission for HARQ scheme. |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Maximum Number of Retransmission for HARQ

Wang Junhu, Sun Bo

ZTE Corporation

# Introductions

HARQ has been adopted for IEEE 802.16m SDD. The following HARQ parameters are defined but how to define the values needs further investigation:

• Maximum retransmission delay: FFS

• Maximum number of retransmissions: FFS

• Maximum number of HARQ processes FFS

• ACK/NACK delay: FFS

This document proposes the maximum number of retransmission for HARQ with simulation support.

# Proposed Maximum Number of Retransmission

We recommend the maximum number of retransmission for HARQ to be 4, basing on simulations. The simulation result is attached below. From the result we found:

- The 4th retransmission can still obtain 0.5dB gain in SNR when BLER（BER）is 10-3.

- The 5th retransmission can still obtain 0.4dB gain in SNR when BLER（BER）is 10-3.

Considering the time delay and occupied resource caused by retransmission, we propose the maximum number of retransmission to be 4 as a tradeoff solution.

# Simulation Results

Simulation parameters:

Fig.1 is based on the environment: Bandwidth 10MHz, FFT size 1024;Channel type: rayleigh; Coding Type: CC; Antenna: 2X2, multiplex;Receiver: MMSE

Fig.2 and Fig.3 are based on environment: Channel type: AWGN; Coding type: CTC

Fig.1

Fig.2

Fig.3

# Text Proposal to Amendment work document

------------------------------------Start text proposal------------------------------------

10.2.1 HARQ in the Downlink

10.2.1.1 HARQ Timing and Protocol

• Maximum number of retransmissions: FFS 4

……

10.2.2 HARQ in the Uplink

10.2.2.1 HARQ Timing and Protocol

• Maximum number of retransmissions: FFS 4