| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comment on USCCH Configuration Scheme for Multicarrier Operation |  |
| Date Submitted | 2009-01-05 |  |
| Source(s) | Yuqin Chen, Kaiying Lv ZTE Corporation Yih-Shen Chen, Kelvin Chou, I-Kang Fu and Paul Cheng MediaTek Inc. | chen.yuqin@zte.com.cn yihshen.chen@mediatek.com, Kelvin.chou@mediatek.com |
| Re: | Call for Contributions and Comments on Project 802.16m System Description Document (SDD) issued on 2008-12-12 (IEEE 802.16m-08/052), on the SDD Section 19.3.6.3. |  |
| Abstract | This contribution proposes the USCCH configuration scheme for multicarrier operation. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comment on USCCH Configuration Scheme for Multicarrier Operation

Yuqin Chen, Kaiying Lv

ZTE Corporation

Yih-Shen Chen, Kelvin Chou, I-Kang Fu and Paul Cheng

MediaTek Inc.

# Introduction

For multicarrier operation, USCCH could be present at both fully configured carrier and partially configured carrier. For AMS working under carrier aggregation mode, USCCH for partially configured carrier could be transmitted on fully configured carrier. However, for AMS working under carrier switching mode, to reduce the carrier switching process overhead, it is better to send the allocation information for each carrier on itself.

# Proposed SDD Text

[Modify the following text in Section 19.3.6.3]

--------------------------------------------------------  Text Start  -----------------------------------------------------------

19.3.6.3 USCCH

USCCH is present in a fully configured carrier. The location and transmission format of USCCH on the fully configured carrier is the same as that defined in 11.7.2.3.

The presence and use of USCCH on the partially configured carrier is FFS.

For AMS under carrier aggregation mode, USCCH for partially configured carrier may be transmitted on fully configured carrier. For AMS under carrier switching mode, each partially configured carrier’s USCCH is sent through on itself.

--------------------------------------------------------  Text End  ------------------------------------------------------------