| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Differential feedback for IEEE 802.16m MIMO Schemes |  |
| Date Submitted | 2009-01-05 |  |
| Source(s) | Qinghua Li, Shanshan Zheng Jiacheng Wang Xiaofeng Liu Feng Zhou Senjie Zhang Hongming Zheng Guangjie Li Yang-seok Choi Intel Corporation | Qinghua.li@intel.com Guangjie.li@intel.com |
| Re: | The TGm Call for Contributions and Comments 802.16m-08/052 |  |
| Abstract | A different feedback scheme is depicted for closed-loop MIMO beamforming. |  |
| Purpose | Discuss and adopt in 802.16m |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Differential feedback for IEEE 802.16m MIMO Schemes

- Qinghua Li, Shanshan Zheng, Jiacheng Wang, Xiaofeng Liu, Feng Zhou, Guangjie Li, Hongming Zheng, Eddie Lin

- Intel Corporation

- Introduction

Differential feedback is considered in the current SDD for closed-loop MIMO beamforming. We propose a differential feedback scheme to exploit both time and space correlations. Simulation results demonstrate that the differential feedback outperform the 802.16e for all scenarios in terms of throughput and feedback overhead.

2.	System Model

The assumed system model is

y=H V s+n,	(1)

where n is the complex AWGN with variance N0; s is the Ns by 1 transmitted vector with unit power; Ns is the number of spatial streams; y is the received vector; H is the channel matrix of size Nr by Nt; V is the beamforming matrix (or vector) of size Nt by Ns.

3.	Differential Feedback

There exist strong correlations between beamforming matrixes in adjacent frequencies and frames, and the correlation can be exploited to reduce feedback overhead. We take the example of time domain correlation to depict the differential feedback scheme. For quantization, the SS first rotates the center of the differential codebook to Vt−1, which is the beamforming matrix used for the t−1-th frame, and then computes and quantizes the difference (called differential matrix) between the ideal beamforming matrix and Vt−1. Equivalently, the SS first subtracts Vt−1 from the ideal beamforming matrix and then quantizes the remaining using the differential codebook, also known as the polar cap codebook. The BS receives the quantization index and adds the differential matrix to Vt−1 for reconstructing the beamforming matrix.

Let Vt−1Vt−1 be a square Nt×Nt unitary matrix. Vt−1 is the Nt×Ns the beamforming matrix for the (t-1)-th frame; Nt and Ns are the numbers of transmit antennas and spatial streams; Vt−1 is Nt×Nt−Ns with the complementary columns orthogonal to Vt−1.

Differential at SS:

D=Vt−1Vt−1HVt	(11)

Quantization at SS:

D=argmaxDi∈CdDHDiF	(12)

Beamforming matrix reconstruction at BS:

Vt=Vt−1Vt−1D	(13)

Beamforming at BS:

y=H Vt s+n.	(14)

In (11), the Nt×Ns Vt and Vt are the ideal and quantized beamforming matrixes for frame t, respectively; Vt−1 has the complementary columns orthogonal to Vt−1 and Vt−1 can be computed from Vt−1 by a Householder operation as in (9). In (12), Di is the codeword matrix of the differential codebook denoted by Cd; the quantization criterion maximizes the received signal power and other criteria such as channel capacity or MSE can be also used. The differential codebook Cd is referred to as the polar cap, which only covers a small portion of the quantization space of the 802.16e codebook as shown in Figure 1 for the vector case.

Figure 1. Full quantization space and polar cap.

4.	Feedback Overhead

The PMI feedback of the transformed codebook has two parts: the index of the transformed 80.216e codebook per 5 ms and the index of transformation matrix per 100 ms. The sum of them is the total feedback overhead. Here, the transformation matrix is computed by Cholesky decomposition. We assume the PMIs for all subbands are fed back for subband selection and there are 12 subbands across frequency. Note that all the subbands share the same transformation matrix. We compare the feedback overheads the table below.

| Feedback overhead | 802.16e |  | Differential |
| --- | --- | --- | --- |
| 2 Tx ant. (bits/5ms/subband) | 3 |  | 3+6/20/12 = 3.03 |
| 4 Tx ant. (bits/5ms/subband) | 6 |  | 3+28/20/12 = 3.12 |

- SLS Performance

- Conclusion

- Reference

- IEEE 802.16e-2005, “IEEE Standard for Local and Metropolitan Area Networks - Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems,” Feb. 28, 2006.

- Q. Li, G. Li, X. Lin, and S. Zheng, “A Low Feedback Scheme for WMAN MIMO Beamforming,” IEEE Radio and Wireless Symposium, pp.349-352, Long Beach, CA, Jan. 2007.

- Proposed Text

11.8.2.1.3 Feedback for SU-MIMO

[add one sentence at the end of line 31 in page 102]

The differential mode: the feedback from a mobile station provides a differential knowledge of the short-term channel information. This feedback represents information that is used along with other feedback information known at the base station for determining a new precoder. Rotation based scheme shall be supported.

11.8.2.2.3.2 CSI feedback

[add one sentence at the end of line 3 in page 104]

The differential mode: the feedback from a mobile station provides a differential knowledge of the short-term channel information. This feedback represents information that is used along with other feedback information known at the base station for determining a new precoder. Rotation based scheme shall be supported.