| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Missing Scenario of Handover Process Supporting Legacy System |  |
| Date Submitted | 2009-01-05 |  |
| Source(s) | Kyounghwan Lee Yerang Hur POSDATA | Voice: +1-408-986-1143 E-mail: khlee@posdata-usa.com yehur@posdata-usa.com |
| Re: | TGm SDD: IEEE 802.16m-08/052: Call for Comments and Contributions on Project 802.16m System Description Document (SDD) |  |
| Abstract | Provision of missing scenario for legacy support handover |  |
| Purpose | To discuss and adopt the proposed text in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Missing Scenario of Handover Process Supporting Legacy System

Kyounghwan Lee and Yerang Hur

POSDATA

- Introduction

The recent effort for handover process supporting legacy system is outlined in Section 10.3.5 in SDD [1]. Table 1 summarizes all possible scenarios of handover supporting legacy system. The corresponding requirement for each scenario from the perspective of advertising neighbor base stations is also provided. Each scenario is marked in SDD column to indicate if the scenario is captured in the current SDD [1].

| Handover Scenario | MS triggered for handover | Requirements | SDD |
| --- | --- | --- | --- |
| YBS to ABS | YMS | YBS advertises the Lzones of its neighbor ABSs. | O (Section 10.3.5.2) |
|  | AMS | YBS advertises the Lzones of its neighbor ABSs. ABS handover to the Lzone of a target ABS. It might be switched to Mzone. Or it handover to Mzone directly. | O (Section 10.3.5.2) |
| YBS to 16m only ABS | AMS | YBS advertises neighbor 16m only ABSs | O (Section 10.3.5.2) |
| ABS to YBS | YMS | ABS advertises its neighbor YBSs (in its Lzone). | O (Section 10.3.5.3) |
|  | AMS | ABS advertises its neighbor YBSs (in its both Lzone and Mzone). The serving ABS performs context mapping and protocol inter-working from 16m to 16e system. | O (Section 10.3.5.3) |
| 16m only ABS to YBS | AMS | 16m only ABS advertises its neighbor. This is not specifically mentioned. However, from a perspective of AMS, handover procedure from ABS or 16m only ABS toYBS is same. | O (Implicitly in Section 10.3.5.3) |
| ABS to ABS | YMS | ABS broadcasts the Lzones of its neighbor ABSs in its Lzone. | x |
|  | AMS | This would follow general 16m handover procedure. | These are not legacy support HO scenarios. |
| 16m only ABS to 16m only ABS | AMS | This would follow general 16m handover procedure. |  |
| ABS to 16m only ABS | AMS | This would follow general 16m handover procedure. |  |
| 16m only ABS to ABS | AMS | This would follow general 16m handover procedure. |  |

< Table 1. All possible scenarios of handover supporting legacy system>

We observe that one feasible legacy supporting handover scenario is not reflected in the current SDD. The missing scenario is for YMS to handover from the Lzone of serving ABS to the Lzone of a target ABS. We note that this is clearly a feasible legacy supporting handover scenario. To support this handover scenario, ABS should advertise Lzones of neighbor ABSs in its Lzone. For seamless operation of legacy supporting handover, we propose reflecting this missing scenario and the corresponding requirement of neighbor advertisement in SDD. Also, section 10.3.4 is duplicated with 10.3.5. We suggest moving all texts in 10.3.5 into 10.3.4.

Finally, we also propose replacing 16m zone and 16e zone with Mzone and Lzone, respectively which are adopted in Section 3.1 as new terms.

- Proposed Text for the 802.16m SDD

---------------------------  Start of the proposed text  --------------------------------------------------------------

10.3.4 Handover Process supporting Legacy system

[Editor’s Note: This section is only related to intra-RAT 16e/IEEE 802.16m HO.]

10.3.4.1 Network topology acquisition

10.3.4.2 Handover from 16e to IEEE 802.16m

10.3.4.3 Handover from IEEE 802.16m to 16e

10.3.5 4 Handover Process supporting Legacy system

[Editor’s Note: This section is only related intra-RAT 16e/IEEE 802.16m HO.]

10.3.5 4.1 Network topology acquisition

The 16e/16m co-existing system consists of 16e and 16m cells/sectors. A YBS advertises the system information for its neighbor YBSs and the LZones of its neighbor ABSs. A ABS advertises the system  information for its neighbor YBSs in its both LZone and 16m zone Mzone. It advertises the Lzones of its neighbor ABSs in its Lzone. It also advertises the system information for its neighbor ABSs in 16m zone Mzone only.

The ABS may indicate its 16m capability and information in its 16e zone Lzone. The signaling for transmitting 16m zone Mzone information is FSS. In addition, the MOB_NBR-ADV message in the LZone of ABS may contain information of a neighboring 16m-only ABS, or 16m zoneMzone of a neighbor ABS, or Lzone of a neighbor ABS.

[Editors note: the 16-m only BS definition is unclear. Since clearly 16m support is indicated the BS is interpreted to be an ABS ]

10.3.5 4.2 Handover from 16e to 16m

When a 16e-to-16m handover is triggered for a YMS, the YMS handover is from the serving YBS to the LZone of the target ABS using 16e handover signaling and procedures.

When a 16e-to-16m handover is triggered for a AMS, the AMS may handover from the serving YBS to the

LZone of the target ABS using 16e handover signaling and procedures. The target YBS/ABS may direct the

AMS to switch to 16m zone Mzone. The detail procedure for zone switching is FFS.

A AMS may also handover from a YBS to a 16m only ABS or 16m zone Mzone of ABS directly if AMS is able to scan 16m-only ABS or 16m zone Mzone prior to handover. The detail procedure is FFS.

10.3. 5 4.3 Handover from 16m to 16e

10.3.4.4 Handover from 16m to 16m for YMS

When a 16m to 16m handover is triggered for a YMS, the YMS handover is from Lzone of its serving ABS to the Lzone of a target ABS.

---------------------------  End of the proposed text   ---------------------------------------------------------------

References

- IEEE 802.16m-08/003r6. The Draft IEEE 802.16m System Description Document, December 2008.