| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC Header Formats |  |
| Date Submitted | 2009-01-05 |  |
| Source(s) | Xiao Xu, Hua Xu | hua.xu@motorola.com |
| Re: | IEEE 802.16m-08/052: Call for Contributions and Comments on Project 802.16m System Description Document (SDD), MAC, MAC header design |  |
| Abstract | This contribution proposes MAC header formats for 802.16m |  |
| Purpose | For review and adoption of proposed text in SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAC Header Formats

Xiao Xu, Hua Xu

Motorola, Inc.

# Introduction

The generic MAC header (GMH) in IEEE 802.16e-2005 reference system is 6 bytes, which needs to be present in every MAC PDU. Lots of other overheads also need to be added, including subheaders, extended subheaders and CRCs. For some applications with small payload sizes such as VoIP, this significantly reduces efficiency and degrades system capacity.

The 16e Generic MAC Header (GMH) is illustrated by Figure 19 below. Careful analysis will show that many of the fields can be reduced or eliminated without functional impairment to the system.

# Detailed discussions of MAC headers

## Generic MAC Header Fields

The following changes will happen to generic MAC header fields:

- Header type (HT) – denotes whether the header is generic or compact..

- FlowID – if the flow ID (transport CID) is used in the resource allocation, we do not need to repeat this every packet unless the allocation is for multiple flows for a single MS. In that case, a shorter 4-bit flow identifier can be used to identify flows within one MS.

- EC – can be removed from the header and exchanged during connection establishment process only.  No need to be transmitted in every MAC PDU.

- CI – No need to retain 4-byte MAC PDU CRC. CI is also unnecessary.

- Type – after changing all the subheaders and extended subheaders to the extended header format, we no longer need these Type bits for the five 16e subheaders. For example we no longer need a bit to indicate fragmentation subheader as it is replaced by the FFI field and the PDU SN field. Also since ARQ is not used for every connection the ‘Extended’ and ‘ARQ feedback’ bits can be moved to an extended header.

- EH – New 1-bit field which Indicates if extended headers are present.

- FFI – New 1-bit field to indicate whether it’s the first fragment. This field together with the PDU SN field eliminates the need for FSH (fragmentation subheader).

- PDU SN – PDU sequence number, to enable HARQ reordering.

- EKS – If encryption is enabled, this 2 bits is needed for every PDU.  Can be put into the 2 LSB of PN (4 bytes), leaving 30 bits for PN.

- HCS – extra protection for header, if HARQ is enabled this is redundant since HARQ CRC is enough.

## Extended Headers

In 16e there are 5 subheaders (packing, fragmentation, and grant management, etc) and some extended headers. The extended subheaders are organized into a group format while the subheaders are not. There is really no point in differentiating the subheaders from the extended subheaders. They can all be called extended headers’ and organized into a group format that is very similar to the 16e extended subheader group format. This is a more logical and uniform way of organizing these extended headers. The order of the extended headers is FFS.

# Proposed SDD text Changes

================= Start of Proposed Text ========================

[Delete all text in 10.12.1.1 in SDD and insert the following text:]

10.12.1.1 Generic MAC Header

In 802.16m, MAC headers consist of the generic MAC header (GMH) or the compact MAC header (CMH) and extended headers. The first bit of the header is Header Type (HT). When HT = 0, it indicates a generic MAC header which follows the format in Figure 22. Description of the fields of the GMH is in Table 1.

Figure 22. Generic MAC Header (GMH) Format (HT = 0)

Table 1. Generic MAC Header Fields

[Delete all text in 10.12.1.2 in SDD and insert the following text:]

10.12.1.2 Compact MAC Header

The compact MAC header is used for persistent allocations where the flowID and length are constant during the allocation period.

When HT = 1, it indicates a compact MAC header (CMH) which follows the format in Figure 23.

Figure 23. Compact MAC Header (CMH) Format (HT = 1)

10.12.2 Extended header

The inclusion of extended header is indicated by EH indicator bit in MAC Header. The EH format is shown in Figure 23 Figure 24 and will be used unless specified otherwise.

Length of extended header group

Figure 24. Extended Header Group Format

[Delete all text in 10.12.2.1 in SDD and insert the following text:]

10.12.2.1 Packing extended header

The format of the packing extended header is illustrated in figure 25. The packing extended header is only included when there are more than one SDU fragments in the MAC PDU.  There is one packing extended header for each MAC PDU, covering all SDU fragments.

- ‘Num SDUs’ field denotes the number of SDU fragments in this MAC PDU.

- ‘Length field’ is variable length, which contains (n-1) x 11 bits, where n is the number of SDU fragments in this MAC PDU, corresponding to the ‘Num SDUs’ field.  Since the GMH includes a Length field, which covers the total length of the MAC PDU, the length of the nth SDU fragment can be derived.

Figure 25. Packing Extended Header Format

============================== End of Proposed Text ===============

# References

[1] IEEE Std. 802.16e-2005, IEEE Standard for Local and metropolitan area networks, Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems, Amendment 2: Physical and Medium Access Control Layers for Combined Fixed and Mobile Operation in Licensed Bands, and P802.16Rev2/D5 (June 2008).

[2] IEEE 802.16m-08/003r2, “The Draft IEEE 802.16m System Description Document”