| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Handover Enhancements |  |
| Date Submitted | 2009-01-05 |  |
| Source(s) | Stavros Tzavidas Motorola | stavros.tzavidas@motorola.com |
| Re: | IEEE 802.16m-08/052 Call for Comments and Contributions on Project 802.16m SDD TGm SDD: Other (for contributions in support of a comment) – MAC Regarding SDD section 10.3.2 |  |
| Abstract | This contribution proposes enhancements to the Handover algorithm as is currently described in the 802.16m SDD document |  |
| Purpose | For discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Handover Enhancements

Stavros Tzavidas

Motorola

# Introduction

In the current SDD document the term “Network controlled HO” is being used. Some members have also proposed introducing the term “MS controlled HO” in contributions to previous meetings. We believe that both of these terms are ill-defined and are often interpreted in different ways by different readers. In order to avoid the resulting confusion and improve the clarity of the SDD text we propose to remove these terms and to instead define the handover procedure that will be used in 16m (without explicitly calling it “MS controlled” or “network controlled”).

In this contribution we propose modifications to the SDD text which aim to describe the handover procedure without reference to the above mentioned terms.

Furthermore, other enhancements are proposed to better define the handover procedure. More specifically we address the issue of when the MS disconnects from the serving BS during a handover and also the issue of how the MS determines that some or all of the proposed target BSs are unreachable and the procedure that the MS follows in this case.

# Text Proposal

----------------------------------------------Start of the text proposal ------------------------------------------------------

Modify section 10.3.2 as follows:

10.3.2 Handover Process

10.3.2.1 HO Framework

The handover algorithm is a network-controlled, AMS-assisted handover. Although The handover procedure may be initiated by either AMS or ABS, the final HO decision and target ABS(s) selection are performed by the serving ABS or the network. The AMS initiates a HO by sending an HO initiation message to the serving ABS (S-ABS). The S-ABS responds to the HO initiation message by sending an HO command message to the AMS. The S-ABS initiates a HO by sending an HO Command control message to the AMS. In both cases (HO initiated by AMS or S-ABS) the HO command message includes zero, one or more target ABSs (T-ABSs).

If the HO command message includes only one target ABS, the AMS executes the HO as directed by the ABS at the disconnect time without further control signaling, or cancels the HO procedure through HO cancellation message.

If the HO command message includes more than one target ABSs, the AMS selects one of these targets and informs the S-ABS of its selection by sending an HO indication message to the S-ABS before the expiration of disconnect time.

If the HO Command control message includes zero T-ABSs, the MS sends an HO initiation message to the S-ABS with a recommended T-BS of its own choosing.

The network re-entry procedure with the target ABS may be optimized by target ABS possession of AMS information obtained from serving ABS over the backbone network. AMS may also maintain communication with serving ABS while performing network re-entry at target ABS as directed by serving ABS. Figure 16 shows a general call flow for handover.

The S-ABS defines error conditions based on which the AMS decides when a T-ABS among those that are included in HO command control signaling is unreachable. If all the target ABSs that are included in the HO command signaling are unreachable, the AMS selects a new T-ABS and performs network entry at the new T-ABS after the expiration of disconnect time. The AMS indicates the identity of its old S-ABS to the new T-ABS during network entry at the new T-ABS.

HO indication

Figure 16 A general call flow for HO

The handover procedures are divided into three phases, namely, HO initiation, HO preparation and HO execution. When HO execution is complete, the AMS is ready to perform Network re-entry procedures at target ABS. In addition, HO cancellation procedure is defined to allow AMS cancel a HO procedure.

10.3.2.2 HO Procedure

10.3.2.2.1 HO initiation

Handover procedure may be initiated by either AMS or ABS and controlled by the ABS. When handover is initiated by the AMS, the serving ABS defines the triggers and conditions based on which the AMS initiates a handover. When multiple triggers and conditions are defined, the serving ABS may use combination of multiple conditions to trigger HO. When HO is initiated by AMS, a HO Initiation control signaling is sent by the AMS to start the HO procedure. In case of ABS initiated HO, HO initiation and HO preparation phases are carried out together.

10.3.2.2.2 HO preparation

During HO preparation phase, the serving ABS communicates with target ABS(s) selected for HO. The target ABS may obtain AMS information from the serving ABS via backbone network for HO optimization. If

ranging with target ABS not performed prior to or during HO preparation, dedicated ranging resource (e.g. code, channel, etc.) at target ABS may be reserved for the AMS to facilitate non-contention-based HO ranging. Information regarding AMS identity (e.g. TEK, STID, FIDs, etc.), may be pre-updated during HO preparation. Any mismatched system information between AMS and the target ABS, if detected, may be provided to the AMS by the Serving ABS during HO preparation.

When only one target ABS is included in the HO Command control signaling, the HO preparation phase completes when serving ABS informs the AMS of its handover decision via a HO Command control signaling. When multiple target ABSs are included in the HO Command control signaling, the HO preparation phase completes when the AMS informs the ABS of its target ABS selection via HO indication control signaling The HO Command control signaling may include dedicated ranging resource allocation and resource pre-allocations for AMS at each target ABS for optimized network re-entry. The HO Command control signaling includes an action time for the AMS to start network re-entry at the each target ABS and an indication whether AMS should

maintain communication with serving ABS during network re-entry. The HO Command control signaling further includes a disconnect time, which indicates when the serving ABS will stop sending downlink data and stop providing any regularly scheduled unsolicited uplink allocations for the AMS. In the case that AMS maintains communication with serving ABS during network reentry, the parameters associated with the scheme of multiplexing transmission with serving and target ABS are determined by serving ABS based on the AMS capability and negotiated between the serving and target BSs.

The control signaling indicates if the static and/or dynamic context and its components of the AMS is available at the target ABS.

10.3.2.2.3 HO execution

At the action time specified in the HO command control signaling, the AMS performs network re-entry at the target ABS. If communication is not maintained between AMS and serving ABS during network re-entry at the target ABS, serving ABS stops allocating resources to AMS for transmission at action time. If directed by serving ABS via HO Command control signaling, the AMS performs network re-entry with the target ABS at action time while continuously communicating with the serving ABS. However, the AMS stops communication with serving ABS after network re-entry at target ABS is completed. In addition, AMS cannot exchange data with target ABS prior to completion of network re-entry. Multiplexing of network re-entry signaling with target ABS and communications with serving ABS is done by using negotiated intervals with serving ABS for network re-entry signaling with target ABS, and the remaining opportunities with serving ABS for data communication. If the negotiated interval is set to 0, the AMS communicates with the serving ABS continuously while concurrently performing network re-entry with the target ABS.

10.3.2.2.4 HO cancellation

After HO is initiated, the handover could be canceled AMSat any phase during HO procedure. After the HO cancellation is processed, the AMS and serving ABS resume their normal operation.

-------------------------------------------------- End of the text --------------------------------------------------