| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modified SDD Text for E-MBS Frame and Control Channel Structure |  |
| Date Submitted | 2009-01-15 |  |
| Source(s) | Chun-Yen Wang, Chun-Yuan Chiu, Fang-Ching Ren, Chie-Ming Chou, Richard Li, ITRI Wern-Ho Sheen, NCTU/ITRI | E-mail: ChunYen@itri.org.tw, ccy@itri.org.tw, frank_ren@itri.org.tw, richard929@itri.org.tw, whsheen@itri.org.tw |
| Re: | Call for Comments on Project 802.16m System Description Document (SDD) |  |
| Abstract | This contribution suggests a frame and control channel structure for E-MBS operation. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modified SDD Text for E-MBS Frame and Control Channel Structure

Chun-Yen Wang, Chun-Yuan Chiu, Fang-Ching Ren, Chie-Ming Chou, Richard Li, ITRI

Wern-Ho Sheen, NCTU/ITRI

# Proposed SDD Text

[Modify the following text in Section 14.4.1.3]

--------------------------------------------------------  Text Start  -----------------------------------------------------------

14.4.1.3 Frame and Control Channel Structure

In unicast/multicast mixed carrier, EMBS uses the same frame structure used for unicast carrier. The EMBS data is multiplexed with Unicast traffic. The SBCH indicates E-MBS region which may span over multiple subframes. If a super-frame contains MBS subframes, MBS sub-frames are allocated with fixed pattern within super-frame. The pattern may vary between super-frames. The Figure 56 below illustrates the frame structure when MBS subframes are present in super-frames.

Figure 56 Illustration for EMBS Channel support in Mixed Broadcast/Unicast Carrier

For unicast/multicast mixed carrier, the control channel design to support E-MBS is as follows

- It is FFS to use BCH or reserved SCH to indicate if a carrier is broadcast only or cannot be used for AMS entry to the network.

- SBCH

  - Provides pointers to help AMS find the location of the cell-specific MSCCH and non-cell specific MSCCHs.

- Cell-specific MSCCH (Multicast Service Control Channel)

  - Provides all essential parameters for retrieving single-BS E-MBS.

  - Contains some control parameters which are cell-specific for multi-BS E-MBS.

- Non-cell specific MSCCH

  - Indicates physical layer parameters of MBS data channels for each service using joint coding.

  - MSCCH iIs transmitted at the beginning of MBS resource during one MBS transmission cycle.

  - MSCCH cCan point to burst locations in up to N super-frames later within the transmission cycle.

--------------------------------------------------------  Text End  ------------------------------------------------------------