| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | LBS Support for Multi-carrier in 16m |  |
| Date Submitted | 2009-01-11 |  |
| Source(s) | Jia Lin, Sean McBeath, Junxian Mo Huawei Technologies I-Kang Fu, Yih-Shen Chen, Kelvin Chou MediaTek Inc. Ying Du, Daning Gong CATR | E-mail: linjia@huawei.com smcbeath@huawei.com IK.Fu@mediatek.com duying@mail.ritt.com.cn |
| Re: | IEEE 802.16m-08/052: Call for Comments on Project 802.16m System Description Document (SDD), Section 13.1 Location Based Services Overview and Section 19.4 MAC Aspect of OFDMA Multi-carrier Operation |  |
| Abstract | This contribution proposes LBS Support for Multi-carrier in 16m. |  |
| Purpose | For discussion and adoption by IEEE 802.16m group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# LBS Support for Multi-carrier in 16m

Jia Lin, Sean McBeath, Junxian Mo

Huawei Technologies

I-Kang Fu, Yih-Shen Chen, Kelvin Chou

MediaTek Inc.

Ying Du, Daning Gong

CATR

# Introduction

Location based services (LBS) are information services accessible with mobile devices through the mobile network and utilizing the ability to make use of the location of the mobile device.

As described in the SDD, for non-GPS measurement method in the DL, the AMS receives signals which are existing signals (e.g. preamble sequence) or new signals designed specifically for the LBS measurements, if it is needed to meet the requirement from the serving/attached ABS and multiple neighboring ABSs/ARSs. The AMS accurately calculates the required measurements, even in the presence of multi-path channel and heavy interference environment, and then estimates its location accordingly.

In UL, various approaches can be utilized at the serving/attached ABS/ARS to locate the AMS such as TOA and AOA. These measurements are supported via existing UL transmissions (e.g. ranging sequence) or new signals designed specifically for the LBS measurements.

Since Multi-carrier is introduced into IEEE802.16m, in this contribution, we propose LBS techniques which take advantage of the multi-carrier operation.

# 2. LBS support for Multi-carrier

For non-GPS measurement method, in DL/UL, AMS receives/transmits signals from/to multiple ABSs for LBS measurement. As is known for single carrier operation, most LBS measurement methods (such as TOA and U-TDOA) need to be performed between AMS and multiple ABSs at different times. It means that AMS only receives/transmits measurement signals from/to one ABS per transmission/reception opportunity. For example, when U-TDOA measurement is being performed, serving ABS should negotiate with neighbor ABSs to get dedicated ranging resource for location measurement, and send these measurement parameters to AMS. Then, AMS can use the dedicated ranging information and measurement parameters to send dedicated CDMA codes to these ABSs through single carrier at different rendezvous times.

For multi-carrier scenario, we propose that LBS measurement can be performed through multiple carriers at the same time, which means that it can reduce signaling overhead and time delay. The ABS can inform the AMS to perform LBS measurement procedures (such as TOA, TDOA) through one or multiple AMS’s carriers (primary carrier or secondary carriers) for LBS measurement. The message may include carrier indices, LBS measurement methods, ABS IDs, Rendezvous time, CDMA codes, Tx opportunity offsets, etc.

Meanwhile, as Figure 3 show, when data is being transmitted between AMS and ABS through one or multiple carriers, LBS measurement can be performed in the AMS’s other carriers at the same time, which helps maintain QoS of data traffic.

The following are some examples of LBS measurement procedures for Multi-carrier:

1) U-TDOA measurement for Multi-carrier

Figure 1: Example of U-TDOA measurement

2) RTD based measurement for Multi-carrier

Figure 2: Example of RTD based measurement

3) Perform LBS measurement and data transmission through different carriers

Figure 3: Example of perform LBS measurement and data transmission through different carriers

# 3. Summary

LBS support in Multi-carrier provides the following benefits.

1) Reduced time delay for performing LBS measurement

2) Increased LBS measurement accuracy

3) Reduced signaling overhead for performing LBS measurement

4) Increased QoS support for data traffic

# 4 Proposed Text

The following text is proposed to be captured in the IEEE 802.16m system description document (SDD).

================================= Start of Proposed Text ==============================

13.1 Location Based Services Overview

In order to enhance location based service, AMS should send report location-related information which includes the location information or the measurement for determining location in response to the request of ABS or according to the location information report condition to measure its location. In addition, LBS is supported for AMS in connected state as well as idle state. For the connected state, AMS can report location information when it is needed. For the idle state, AMS should perform location information reporting when it is paged, and report location information when it is needed.

The AMS positioning is performed by using measurements methods, such as TDOA, TOA, AOA, and etc., whose relevant location-related parameters may includes cell-ID, RSSI, CINR, RD, RTD, angle, and Spatial Chanel Information. These parameters are exchanged between the AMS and its serving/attached and neighboring ABSs/ARSs. These measurements of these parameters are extracted by processing DL and/or UL signals at the AMS and ABSs, respectively. Positioning algorithms that depend on such measurements have certain performance tradeoffs in terms of positioning accuracy, latency, and signaling overhead. Two or more measurements can be utilized to provide higher accuracy estimate of the AMS position.

The multi-carrier AMS which is capable of processing multiple radio carriers at the same time may perform LBS measurement on one or multiple carriers.

19.4.11 LBS Support

The multi-carrier AMS which is capable of processing multiple radio carriers at the same time may perform LBS measurement on one or multiple carriers.

==================================== End of the Text ================================