| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for 6bit codebooks for 4 transmit antennas |  |
| Date Submitted | 2009-01-05 |  |
| Source(s) | Yang Tang, Yajun Kou, Ping Yu, Young Hoon Kwon, Zhigang Rong, Jianmin Lu, Ping Yu, Yinggang Du Huawei Ying Du CATR Liu Jianhua. Alcatel Shanghai Bell Xin Su, Xiaofeng Zhong Tsinghua University | E-mail: ytang@huawei.com Email: duying@mail.ritt.com.cn Email : jianhua.liu@alcatel-sbell.com.cn. Email : suxin@mail.tsinghua.edu.cn zhongxf@tsinghua.edu.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-08/052: “Call for Comments on Project 802.16m System Description Document (SDD)” SDD Section: 11.8.2.1.2 Closed-loop SU-MIMO |  |
| Abstract | This contribution proposes 6bit codebook for 4 transmit antennas |  |
| Purpose | For discussion and approval by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

The 6-bit codebook set for 4 transmit antennas

Yang Tang, Yajun Kou, Ping Yu, Young Hoon Kwon, Zhigang Rong, Jianmin Lu, Ping Yu, Yinggang Du

Huawei

Ying Du

CATR

Liu Jianhua.

Alcatel Shanghai Bell

Xin Su, Xiaofeng Zhong

Tsinghua University

# Introduction

The 6-bit codebook set, which includes rank-1 and 2 codebooks, for 4 transmit antennas, is proposed for IEEE802.16m to achieve the follow features:

- One codebook for all channel correlation conditions: The good performance in terms of throughput can be achieved with one codebook for a wide range of channel conditions: uncorrelated, correlated and dual-polarized.

- Low codeword searching complexity: Compared to the conventional 6-bit codebook, the proposed rank-1 codebook design can at least save the codeword selection complexity by 61%.

- Nest property: The column vectors of the rank-2 codebook are the rank-1 codewords. In this case, the codeword selection computation complexity can be further saved by 50%.

- Low power imbalance: The codebook is designed with the consideration of the power imbalance issue. Compared to 802.16e 6-bit codebook, the proposed codebook can dramatically decrease the power imbalance level.

- Low feedback overhead: The codebook is designed in a hierarchical structure to support differential feedback, which can lower the feedback overhead.

# 2. Text proposals

------------------------------------Begin text proposal------------------------------------

(Line 33, Page 103)

###### 11.8.2.1.2    Precoding technique

In FDD and TDD systems, unitary codebook based precoding is supported. The notation V(Nt,S,L)denotes the vector codebook, which consists of 2Lcomplex, unit vectors of a dimension Ntand Sdenotes the number of streams. The integerLis the number of bits required for the index that can indicate any codeword in the codebook. The codebooks for dimension of 2,4 and 8 are listed in Table xxx.

Table1: List of codebooks for spatial dimension of 2,4 and 8

| Spatial dimension | Rank-1 | Rank-2 | Rank-3 | Rank-4 | Rank-M (M>4) |
| --- | --- | --- | --- | --- | --- |
| 2 | V(2,1,3) | V(2,2,3) | N/A | N/A | N/A |
| 4 | V(4,1,6),V(4,1,4) | V(4,2,6),V(4,2,4) | FFS | FFS | N/A |
| 8 | FFS | FFS | FFS | FFS | FFS |

Table x: V(4,1,6)=[W1,⋯,W64]

| # Binary Index | # i | Wi |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 000000 | 1 | 0.5000 | -0.5000 | 0.5000 | -0.5000 |
| 010000 | 2 | 0.5000 | -0.4619 - 0.1913i | 0.3536 + 0.3536i | -0.1913 - 0.4619i |
| 100000 | 3 | 0.5636 | -0.3332 - 0.2672i | 0.1174 + 0.5512i | -0.3308 - 0.2702i |
| 110000 | 4 | 0.5000 | -0.4619 + 0.1913i | 0.3536 - 0.3536i | -0.1913 + 0.4619i |
| 000001 | 5 | 0.5000 | 0.5000 | -0.5000 | -0.5000 |
| 010001 | 6 | 0.3117 | 0.6025 + 0.1995i | -0.4030 - 0.4903i | -0.1122 - 0.2908i |
| 100001 | 7 | 0.5587 | 0.3361 + 0.2735i | -0.3361 - 0.2735i | -0.1135 - 0.5471i |
| 110001 | 8 | 0.3117 | 0.4030 + 0.4903i | -0.6025 - 0.1995i | -0.1122 - 0.2908i |
| 000010 | 9 | 0.5000 | -0.5000 | -0.5000 | 0.5000 |
| 010010 | 10 | 0.3117 | -0.6025 - 0.1995i | -0.1122 - 0.2908i | 0.4030 + 0.4903i |
| 100010 | 11 | 0.5587 | -0.3361 - 0.2735i | -0.1135 - 0.5471i | 0.3361 + 0.2735i |
| 110010 | 12 | 0.3117 | -0.4029 - 0.4904i | -0.1184 - 0.2883i | 0.6067 + 0.1865i |
| 000011 | 13 | 0.5000 | 0.0000 - 0.5000i | 0.5000 | 0.0000 - 0.5000i |
| 010011 | 14 | 0.3058 | 0.1901 - 0.6052i | 0.1195 + 0.2866i | 0.4884 - 0.4111i |
| 100011 | 15 | 0.5587 | 0.2735 - 0.3361i | 0.1135 + 0.5471i | 0.2735 - 0.3361i |
| 110011 | 16 | 0.3082 | 0.4887 - 0.4077i | 0.1205 + 0.2837i | 0.1842 - 0.6092i |
| 000100 | 17 | 0.5000 | 0.0000 + 0.5000i | -0.5000 | 0.0000 - 0.5000i |
| 010100 | 18 | 0.5000 | -0.1913 + 0.4619i | -0.3536 - 0.3536i | 0.4619 - 0.1913i |
| 100100 | 19 | 0.3082 | -0.4887 + 0.4077i | -0.6092 - 0.1842i | 0.2837 - 0.1205i |
| 110100 | 20 | 0.5000 | 0.1913 + 0.4619i | -0.3536 + 0.3536i | -0.4619 - 0.1913i |
| 000101 | 21 | 0.5000 | 0.0000 - 0.5000i | -0.5000 | 0.0000 + 0.5000i |
| 010101 | 22 | 0.5000 | 0.1913 - 0.4619i | -0.3536 - 0.3536i | -0.4619 + 0.1913i |
| 100101 | 23 | 0.5636 | 0.2673 - 0.3331i | -0.1222 - 0.5501i | -0.2673 + 0.3331i |
| 110101 | 24 | 0.5000 | -0.1913 - 0.4619i | -0.3536 + 0.3536i | 0.4619 + 0.1913i |
| 000110 | 25 | 0.5000 | 0.5000 | 0.5000 | 0.5000 |
| 010110 | 26 | 0.5000 | 0.4619 + 0.1913i | 0.3536 + 0.3536i | 0.1913 + 0.4619i |
| 100110 | 27 | 0.5636 | 0.3691 + 0.5142i | 0.3331 + 0.2673i | 0.0862 + 0.3032i |
| 110110 | 28 | 0.5000 | 0.4619 - 0.1913i | 0.3536 - 0.3536i | 0.1913 - 0.4619i |
| 000111 | 29 | 0.5000 | 0.0000 + 0.5000i | 0.5000 | 0.0000 + 0.5000i |
| 010111 | 30 | 0.3082 | 0.0104 + 0.3151i | 0.4077 + 0.4887i | -0.4783 + 0.4145i |
| 100111 | 31 | 0.5587 | -0.2990 + 0.0880i | 0.3361 + 0.2735i | -0.5216 + 0.3616i |
| 110111 | 32 | 0.3117 | -0.2452 + 0.3573i | 0.6025 + 0.1995i | -0.5360 + 0.1578i |
| 001000 | 33 | 0.5000 | 0.5000 | 0.5000 | -0.5000 |
| 011000 | 34 | 0.3117 | 0.3573 - 0.2452i | 0.6025 - 0.1995i | -0.1578 + 0.5360i |
| 101000 | 35 | 0.5587 | 0.0880 - 0.2990i | 0.3361 - 0.2735i | -0.3616 + 0.5216i |
| 111000 | 36 | 0.3117 | 0.3117 - 0.0000i | 0.4030 - 0.4903i | -0.4030 + 0.4903i |
| 001001 | 37 | 0.5000 | 0.0000 + 0.5000i | -0.5000 | 0.0000 + 0.5000i |
| 011001 | 38 | 0.3117 | 0.2452 + 0.3573i | -0.6025 + 0.1995i | 0.5360 + 0.1578i |
| 101001 | 39 | 0.5587 | 0.2990 + 0.0881i | -0.3362 + 0.2735i | 0.5216 + 0.3616i |
| 111001 | 40 | 0.3117 | -0.0000 + 0.3117i | -0.4030 + 0.4903i | 0.4903 + 0.4030i |
| 001010 | 41 | 0.5000 | -0.5000 | 0.5000 | 0.5000 |
| 011010 | 42 | 0.3082 | -0.3666 + 0.2426i | 0.6092 - 0.1842i | 0.1615 - 0.5298i |
| 101010 | 43 | 0.5587 | -0.0880 + 0.2990i | 0.3361 - 0.2735i | 0.3616 - 0.5216i |
| 111010 | 44 | 0.3082 | -0.3152 - 0.0036i | 0.4076 - 0.4888i | 0.4040 - 0.4872i |
| 001011 | 45 | 0.5000 | 0.0000 - 0.5000i | -0.5000 | 0.0000 - 0.5000i |
| 011011 | 46 | 0.3117 | -0.2452 - 0.3573i | -0.6025 + 0.1995i | -0.5360 - 0.1578i |
| 101011 | 47 | 0.5587 | -0.2990 - 0.0880i | -0.3361 + 0.2735i | -0.5216 - 0.3616i |
| 111011 | 48 | 0.3082 | 0.0036 - 0.3152i | -0.4076 + 0.4888i | -0.4872 - 0.4040i |
| 001100 | 49 | 0.5000 | 0.3536 + 0.3536i | 0.0000 + 0.5000i | -0.3536 + 0.3536i |
| 011100 | 50 | 0.3117 | 0.4260 + 0.0793i | 0.1995 + 0.6025i | 0.2674 + 0.4906i |
| 101100 | 51 | 0.5636 | 0.2741 - 0.1559i | 0.2672 + 0.3332i | 0.1081 + 0.6236i |
| 111100 | 52 | 0.3117 | 0.2204 + 0.2204i | 0.4903 + 0.4030i | 0.0618 + 0.6317i |
| 001101 | 53 | 0.5000 | -0.3536 + 0.3536i | 0.0000 - 0.5000i | 0.3536 + 0.3536i |
| 011101 | 54 | 0.3117 | -0.0793 + 0.4260i | -0.1995 - 0.6025i | 0.4906 - 0.2674i |
| 101101 | 55 | 0.5636 | 0.1559 + 0.2741i | -0.2672 - 0.3332i | 0.6236 - 0.1081i |
| 111101 | 56 | 0.3117 | -0.2204 + 0.2204i | -0.4903 - 0.4030i | 0.6317 - 0.0618i |
| 001110 | 57 | 0.5000 | -0.3536 - 0.3536i | 0.0000 + 0.5000i | 0.3536 - 0.3536i |
| 011110 | 58 | 0.3117 | -0.4260 - 0.0793i | 0.1995 + 0.6025i | -0.2674 - 0.4906i |
| 101110 | 59 | 0.5587 | -0.2737 + 0.1492i | 0.2735 + 0.3361i | -0.1132 - 0.6245i |
| 111110 | 60 | 0.3082 | -0.2154 - 0.2302i | 0.4887 + 0.4077i | -0.0451 - 0.6313i |
| 001111 | 61 | 0.5000 | 0.3536 - 0.3536i | 0.0000 - 0.5000i | -0.3536 - 0.3536i |
| 011111 | 62 | 0.3117 | 0.0793 - 0.4260i | -0.1995 - 0.6025i | -0.4906 + 0.2674i |
| 101111 | 63 | 0.5587 | -0.1492 - 0.2737i | -0.2735 - 0.3361i | -0.6245 + 0.1132i |
| 111111 | 64 | 0.3082 | 0.2254 - 0.2204i | -0.4888 - 0.4076i | -0.6302 + 0.0588i |

Table 2: V(4,2,6)=[W21,⋯,W264]

| # W2m=Wi,Wj |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| index | m | i,j | index | m | i,j | index | m | i,j |  | index | m | i,j |
| 000000 | 1 | 1,5 | 010000 | 17 | 9,13 | 100000 | 33 | 33,37 | 110000 |  | 49 | 49,53 |
| 000001 | 2 | 1,7 | 010001 | 18 | 9,25 | 100001 | 34 | 33,45 | 110001 |  | 50 | 49,57 |
| 000010 | 3 | 1,9 | 010010 | 19 | 9,29 | 100010 | 35 | 33,46 | 110010 |  | 51 | 49,61 |
| 000011 | 4 | 1,17 | 010011 | 20 | 9,31 | 100011 | 36 | 33,41 | 110011 |  | 52 | 49,62 |
| 000100 | 5 | 1,21 | 010100 | 21 | 10,14 | 100100 | 37 | 34,37 | 110100 |  | 53 | 50,53 |
| 000101 | 6 | 1,25 | 010101 | 22 | 10,25 | 100101 | 38 | 35,45 | 110101 |  | 54 | 51,59 |
| 000110 | 7 | 3,11 | 010110 | 23 | 11,15 | 100110 | 39 | 36,41 | 110110 |  | 55 | 52,57 |
| 000111 | 8 | 5,11 | 010111 | 24 | 13,17 | 100111 | 40 | 36,44 | 110111 |  | 56 | 53,57 |
| 001000 | 9 | 5,13 | 011000 | 25 | 13,21 | 101000 | 41 | 37,41 | 111000 |  | 57 | 53,61 |
| 001001 | 10 | 5,25 | 011001 | 26 | 15,29 | 101001 | 42 | 37,43 | 111001 |  | 58 | 54,57 |
| 001010 | 11 | 5,29 | 011010 | 27 | 17,25 | 101010 | 43 | 38,41 | 111010 |  | 59 | 55,63 |
| 001011 | 12 | 5,32 | 011011 | 28 | 17,29 | 101011 | 44 | 39,47 | 111011 |  | 60 | 56,61 |
| 001100 | 13 | 5,9 | 011100 | 29 | 20,24 | 101100 | 45 | 40,45 | 111100 |  | 61 | 56,64 |
| 001101 | 14 | 6,9 | 011101 | 30 | 21,25 | 101101 | 46 | 40,48 | 111101 |  | 62 | 57,61 |
| 001110 | 15 | 6,13 | 011110 | 31 | 21,29 | 101110 | 47 | 41,45 | 111110 |  | 63 | 57,63 |
| 001111 | 16 | 8,25 | 011111 | 32 | 21,30 | 101111 | 48 | 41,47 | 111111 |  | 64 | 58,61 |

In TDD systems, sounding based precoding is supported.

------------------------------------End text proposal------------------------------------

Appendix:

- Rank-1 codebook structure

Fig 1: rank-1 codebook hierarchical structure

Layer 1

000000

010000

100000

110000

000001

010001

100001

110001

001111

011111

101111

111111

Layer 2

Branch 1

Branch 2

Branch 16

…

…

The codewords of each branch in the layer 2 are defined as the daughter codewords of the one in the layer 1 of the same branch, which is named as the mother codeword. As shown in fig 1, each mother codeword has three daughter codewords. It is noted the daughter codeword has higher correlation with its mother codeword than with other codeword in the layer 1. The binary indices of the codewords in the same branch share the same last four digits.

Layer 1: Layer 1 codewords are DFT based and tailored for the correlated  channel.

Layer 2: The design criterion of Layer 2 codewords is to maximize the chordal distance to guarantee the performance in the uncorrelated channel. Meanwhile, the layer 2 codewords are formed in a hierarchical structure to facilitate the codeword searching.

In general, with different application focuses of layer 1 and 2 codewords, the codebook, in overall, can provide superior performance in both correlated and uncorrelated channel.

- Rank-1 codeword searching

With the hierarchical structure, the conventional codeword searching mechanism, in which all the codewords have to be tried, can be significantly simplified. The simplified rank-1 codeword searching method based on the proposed codebook is given as

Step 1: Find the top N layer-1 codewords in terms of maximizing the

effective channel gainHWi, where His the channel matrix.

Step 2: Calculate the effective channel gains of the N layer-1 codewords

found in Step 1 and their corresponding 3N daughter codewords.

Totally, 3N+16 out of 64 codewords are searched.

Step 3: The codeword, which maximizes the effective channel gain, is

selected.

As an example, when N=3, only 25 out of 64 codewords are searched. In other words, 61% of codeword searching complexity can be saved. Most importantly, based on the simulations, the corresponding performance degradation is negligible.

- Rank-2 codeword searching

For Rank-2 codeword searching, the effective channel HW2i=HW2i(:,1),HW2i(:,2) is required. In conventional codeword searching,  there are 128 matrix multiplications, which consist of HW2i(:,j),i∈[1,⋯,64]∧j∈[1,2].

With the nested property achieved in the proposed codebooks, the vectors, W2i(:,1),W2i(:,2) of the rank-2 codeword are the codewords in the rank-1 codebook. As a result, the number of matrix multiplication can be reduced from 128 to 64 or by 50%.

- Rank adaptation

In rank adaptation, the effective channels calculation for both rank-1 and 2 codewords, HWi and HW2i, are required. In conventional rank adaptation method, there are 192 matrix multiplications, which consist of 64 for rank-1 codebook and 128 for rank-2 codebook. However, with the proposed codebook, the number of matrix multiplications is reduced from 192 to 64 or by 67%.