| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | LBS measurement report in idle mode |  |
| Date Submitted | 2009-01-05 |  |
| Source(s) | Jia Lin, Sean McBeath, Junxian Mo Huawei Technologies Yih-Shen Chen, Dora Chen and Paul Cheng MediaTek Inc. | E-mail: linjia@huawei.com yihshen.chen@mediatek.com |
| Re: | IEEE 802.16m-08/052: Call for Comments on Project 802.16m System Description Document (SDD), Section 13.4.2 Idle State |  |
| Abstract | This contribution proposes LBS measurement report in Idle Mode in 16m. |  |
| Purpose | For discussion and adoption by IEEE 802.16m group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

LBS measurement report in idle mode

Jia Lin, Sean McBeath, Junxian Mo

Huawei Technologies

Yih-Shen Chen, Dora Chen and Paul Cheng

MediaTek Inc.

# Introduction

Location based service (LBS) are information services accessible with mobile devices through the mobile network and utilizing the ability to make use of the location of the mobile device.

As SDD describes for idle state of LBS measurement, The AMS in idle mode can receive a paging message which may include identifier of the AMS and indicator for LBS measurement method; other associated parameters for LBS measurement are FFS. AMS should perform LBS measurement with attached ABS and neighbor ABSs in terms of LBS measurement method that indicated by paging message for the AMS. When AMS gets LBS measurement parameters, AMS may report them as location information to attached ABS.

In this contribution, we propose LBS measurement parameters (location information) can be transmitted through request message in random initial ranging process.

# 2. LBS measurement report in idle mode

For idle state of LBS measurement, When AMS gets LBS measurement parameters, AMS may report them as location information to attached ABS.

Once AMS receives LBS measurement indicator from ABS, or AMS satisfy conditions for reporting LBS measurement parameters in idle state, AMS can perform random initial ranging process for network re-entry. LBS measurement parameters (location information) may be got before or after triggered LBS measurement report. In this case, LBS measurement parameters (location information) can be reported to the attached ABS through ranging message in random initial ranging process, while other network entry process (such as capability negotiation, registration signaling and service related process) can be omitted.

- Various LBS measurement method may require different resource size to report LBS measurement parameters (location information).

- Since AMS performs random initial ranging process for reporting LBS measurement parameter, ABS can not distinguish with which AMS want to report LBS when ABS receives random CDMA code. If the resource allocated for report LBS measurement parameter is not enough. AMS can require more resources for report residual LBS measurement parameters through ranging message, then ABS will allocate resource to AMS for satisfy LBS measurement report.

- Indicator of report LBS measurements can be included into the RNG-REQ message, which indicates its contents are used for LBS measurement report. Meanwhile, the AMS may inform ABS to know which LBS report mode is used and how many resource it needs.

- The ABS can inform AMS to report LBS information in idle mode or connection mode.

Following are some examples of report LBS measurement parameters:

1) AMS receives LBS measurement indicator from ABS

Figure 1

2) Satisfy condition for Report LBS measurement results

Figure 2

# 3. Conclusion

In order to solve the scenario for report LBS report in idle mode, we propose that when AMS gets LBS measurement parameters, AMS may report them as location information to attached ABS through ranging message in initial ranging process.

# 4 Proposed Text

The following text is proposed to be captured in the IEEE 802.16m system description document (SDD).

================================= Start of Proposed Text ==============================

13.4.2 Idle State

The system should be able to locate the mobile when in idle state. In some use cases the AMS may not need to enter the connected state to be located. The ABS may use paging or other network initiated multicast signaling to initiate a location process on the AMS.

The AMS in idle mode can receive a paging message which may include identifier of the AMS and indicator for LBS measurement method; other associated parameters for LBS measurement are FFS. AMS should perform LBS measurement with attached ABS and neighbor ABSs in terms of LBS measurement method that indicated by paging message for the AMS. When AMS gets LBS measurement parameters, AMS may report them as location information to attached ABS through message in initial ranging procedure. The message may include indicator of LBS measurement report, LBS measurement parameters and indicator of more resource for LBS measurement report.

==================================== End of the Text ================================