| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> C80216m-09_0111r1 |  |
| --- | --- | --- |
| Title | Clarification on the Processing of Secondary Carriers in Multicarrier Mode |  |
| Date Submitted | January 14, 2009 |  |
| Source(s) | Kamran Etemad Intel Corporation Youngsoo Yuk, Inuk Jung, Ronny Yong-ho Kim and Kiseon Ryu LG Electronics, Sean McBeath Huawei Jaehee Cho Samsung Electronics Zexian.li, Jan Suumaki, Roberto Albanese, Nokia I-Kang Fu MediaTek Inc. Mo-Han Fong Nortel Yuqin Chen, Kaiying Lv: ZTE | kamran.etemad@intel.com sixs@lge.com, cooper@lge.com, ronnykim@lge.com, ksryu@lge.com smcbeath@huawei.com jaehee1.cho@samsung.com; jan.suumaki@nokia.com; zexian.li@nokia.com, roberto.albanese@nokia.com IK.Fu@mediatek.com mhfong@nortel.com chen.yuqin@zte.com.cn, lv.kaiying@zte.com.cn |
| Re: | Call for Contributions on Project 802.16m System Description Document (SDD) |  |
| Abstract | Stage 2 proposes text to complement existing SDD text draft |  |
| Purpose | Discussion and Approval |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on the Processing of Secondary Carriers in Multicarrier Mode

Kamran Etemad

Intel Corporation

Youngsoo Yuk, Inuk Jung, Ronny Yong-ho Kim and Kiseon Ryu

LG Electronics

Sean McBeath

Huawei

Jaehee Cho

Samsung Electronics

Zexian.li, Jan Suumaki

Nokia

I-Kang Fu

MediaTek Inc.

Mo-Han Fong

Nortel

Yuqin Chen,  Kaiying Lv:

ZTE

# Introduction and Background

The current SDD draft defines multicarrier operation between fully configured carriers and partially configured carriers. However some clarification is needed to show when MS should start and stop MC mode and when start or stop processing secondary carrier.

The following proposed text suggests some such clarifications without adding new concepts. To explain the process additional terms such as available, candidate and active carriers are introduced to help with the clarifications.

# Text Proposal

---------------------------------------------------------Start of the Text--------------------------------------

[Make the following clarifications in relevant sections of SDD.]

….

19.4.7 Carrier Management

The following steps summarize the high level sequence of procedures involved in the MC operation:

- ABS periodically broadcasts its MC mode and MC configuration

- The carriers listed in the MC configuration message are called Available Carriers.  Not all available carriers can be assigned to an AMS but all available carriers are introduced to AMS’s along with their respective Physical Carrier Index.

- The ABS may also send the detailed MC configuration to the AMS broadcast messaging.

- AMS Performs initialization and network entry. The process is the same as SC mode.

- AMS and ABS perform MC Capability negotiation.

- Example Capabilities may include:

- Carrier Switching Only

- Capability to concurrently receive and aggregate MC’s and Max No. of Carriers

- Capability to concurrently aggregate and transmit on MC’s, Max No. of Carriers. Note the AMS’s MC capability may be different for TX and RX.

- Capability to support Aggregation across Non-contiguous Spectrum, Max RF distance between carriers. This is in addition to AMS’s support for multiple band classes.

Based on AMS RF capabilities, loading of available carriers or other factors, the ABS may provide more detailed configuration information on subset of available carrier designated as Assigned Secondary Carriers to AMS.

The AMS does not perform any PHY/MAC processing on Assigned Secondary Carriers until directed by the ABS.

- The ABS allocates a subset of assigned secondary carriers to be ready for the potential use for MC data transmission based on QoS requirement, loading and other factors. This subset is called the Active Secondary Carriers.

- AMS performs PHY/MAC processing on those active carriers. The ABS may update the active secondary carriers based on QoS requirement, loading and other factors.

- The ABS may assign a logical carrier index to each active secondary carrier for the AMS. Primary carrier is always assigned with logical carrier index 0. .

- The ABS makes MC traffic allocation which may be:

- Aggregation across all fully configured active carriers.

- Aggregation involving at least one partially configured active carrier

- Switching from one fully configured active carrier to another fully configured carrier which will result in primary carrier change

- Switching to a partially configured active secondary carrier.

Figure xxx. Relation between Available, Assigned and Active Carriers

Table 1. Definitions of Available, Assigned and Active Carriers

|  | Definition and Properties |
| --- | --- |
| Available Carriers | Multiple carriers which are available in an ABS - Not all Available carriers may be supported by the AMS - No Processing on these Carriers - Referred to with Physical Carrier Indexes Unique within an ABS. |
| Assigned Carriers | Subset of Available Carriers which may be potentially used by the AMS - Determined according to the capability of the AMS, SLA’s , loading of available carriers of the ABS or other factors. - No processing on these carriers until directed by the ABS. - Referred to with Physical Carrier Indexes |
| Active Carriers | Subset of Available Carriers which are ready to be used for MC assignments. - Determined based on QoS requirement and other factors - PHY/MAC processing are required for the active carriers. - Referred to with Logical Physical Carrier Indexes Unique for each AMS. [- Resource allocation information (in USCCH / MSCCH) May be monitored.] - Broadcast messages (in BCH/Data Burst) should be monitored for Tx/Rx of data.] |

The ABS may instruct the AMS, through control signaling on the current primary carrier, to switch its primary carrier to one of the available fully configured carriers within the same ABS for load balancing purpose, carriers’ varying channel quality or other reasons. AMS switches to the target fully configured carrier at action time specified by the ABS. The carrier switch may also be requested by the AMS through control signaling on the current primary carrier. Given that a common MAC manages both serving and target primary carriers, network re-entry procedures at the target primary carrier is not required. ABS may direct an AMS to perform the primary carrier switching without scanning.

ABS may also assign different secondary carriers to the AMS, through control signaling on the current primary carrier. The ABS may instruct AMS to perform scanning on other carriers which are not serving the AMS. In this case, if the target carrier is not currently serving the AMS, the AMS may perform synchronization with the target carrier if required.