| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> C80216m-09_0112r1 |  |
| --- | --- | --- |
| Title | Clarification on Dedicated E-MBS Carriers and Reuse of MBS in 16e/16m |  |
| Date Submitted | January 13, 2009 |  |
| Source(s) | Kamran Etemad Intel Corporation JaeJeong Shim, Hyunjeong Kang, Kaushik Josiam, BaoWei Ji, Samsung | kamran.etemad@intel.com |
| Re: | Call for Contributions on Project 802.16m System Description Document (SDD) |  |
| Abstract | Stage 2 proposes text to complement existing SDD text draft |  |
| Purpose | Discussion and Approval |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on Dedicated E-MBS Carriers and Reuse of MBS in 16e/16m

Kamran Etemad

Intel Corporation

JaeJeong Shim, Hyunjeong Kang, Kaushik Josiam, BaoWei Ji,

Samsung

# Introduction and Background

The current SDD draft defines E-MBS operation using unicast/multicast mixed carriers and dedicated E-MBS carriers. The E-MBS carrier is also considered partially configured carriers in multicarrier operation.

There is need to define mechanisms to notify MS’s processing E-MBS data on a dedicated E-MBA carrier to return to their primary carriers for any urgent messaging.

There is also need to clarify how the system should handle scenarios in which the same MBS content is delivered to both legacy 16e users and new 16m users in mixed migration deployments.

The proposed text changes captures the above two clarifications and enhancements to current SDD text.

# Text Proposal

---------------------------------------------------------Start of the Text--------------------------------------

[Make the following clarifications in relevant sections of SDD.]

14.5 E-MBS Transmission on Dedicated Broadcast Carriers

E-MBS could be transmitted in a dedicated carrier, or a unicast/EMBS mixed carrier.

14.5.1 Deployment mode for E-MBS transmission on dedicated broadcast carrier

IEEE 802.16m system may designate the carriers for E-MBS only. The multi-carrier AMS which is capable of processing multiple radio carriers at the same time may perform normal data communication at one carrier while receiving the E-MBS content over another carrier.

14.5.2 E-MBS Dedicated Carrier

EMBS data can be transmitted in broadcast only carrier. In this case a fully configured unicast or unicast/EMBS mixed carrier could be used to provide signaling support needed for service initiation, and additions and terminations as well as other service and security related exchanges between the AMS and the ABS or the MBS servers in the network. The Broadcast Only carrier, may be transmitted at higher power and be optimized for improve performance.

The multi-carrier AMS which is capable of processing multiple radio carriers at the same time may perform normal data communication at one carrier while receiving E-MBS data over another carrier. It may also receive multiple E-MBS streams from multiple carriers simultaneously.

Transmission of indications to all AMSs or those in the same paging Group on the E-MBS Dedicated Carrier is FFS.

14.6 Reusing MBS transmission in 802.16e Zones or Carriers

MBS content which is transmitted to 802.16e MSs can be accessed by 16m AMS’s operating on the same or a different carrier.

The EMBS Control signaling in ABS indicates the availability of the service as well as contents. If the MBS content is also being transmitted to 16e MS’s in an MBS zone, the ABS may direct the AMS to 16e zone in the same carrier or other carriers.

The information provided by the ABS should be sufficient for the AMS to synchronize with the MBS data transmissions in a timely manner. E-MBS connection setup and updates for AMSs may be performed using E-MBS control signaling in 16m. AMSs in 16e zone use the connection setup mechanisms in the reference system.