| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | CTC Inner Interleaver Optimization for 16m |  |
| Date Submitted | 2009-01-05 |  |
| Source(s) | Ying Jin, Wenjian Hua, Shaofang Wang, Chongli Liu, Yuejun Wei Huawei Technologies Co., Ltd. | E-mail: jinying12@huawei.com |
| Re: | IEEE 80216-08_052--Call for Comments on Project 802.16m System Description Document (SDD) |  |
| Abstract | A set of CTC encoding block sizes is proposed in this contribution. |  |
| Purpose | To be discussed and adopted by TGm SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

CTC Inner Interleaver Optimization for 16m

Ying Jin, Wenjian Hua, Shaofang Wang, Chongli Liu, Yuejun Wei

Huawei Technologies Co., Ltd.

# Introduction

It has been decided that IEEE 802.16m uses CTC as in 16e standard, and the CTC scheme needs to be extended to support additional FEC block sizes [1].

In 16m standard, the target peak spectral efficiency is about 15.0bps/Hz in downlink and 6.75bps/Hz in uplink, which requires high throughput for CTC decoder. So the supported parallelization orders for CTC decoder must be high enough, especially for larger block sizes.

In this contribution, a set of FEC block sizes, ranging from 48 bits to 4800 bits is designed based on the FEC block sizes of CTC in 16e. The decoding parallelization orders and BLER performance for every FEC block size are also given.

# Proposed scheme

The second step of the inner interleaver for CTC in 16e [2] is as follows:

Forj=0,1,2,…,N−1,

if jmod4==0π(j)=(P0⋅j+1)modN

if jmod4==1 π(j)=(P0⋅j+1+N2+P1)modN

if jmod4==2π(j)=(P0⋅j+1+P2)modN

if jmod4==3π(j)=(P0⋅j+1+N2+P3)modN.

Where π(j) is the interleaving address corresponding to position j, N is the number of input information couples, P0 is coprime with N, andP1, P2 and P3 are the dither parameters. The period of the dither parameters for ARP interleaver is 4, which can be called cycle length.

ARP interleaver is naturally contention-free, and the supported parallelization orders by CTC in 16e are as below:

Table 1 Parallelization orders for CTC decoding in 16e

| FEC Block Size (bits) | M (parallelization orders) | FEC Block Size (bits) | M (parallelization orders) (up to 12) |
| --- | --- | --- | --- |
| 48 | 1 2 3 4 6 8 12 | 384 | 1 2 3 4 6 8 12 |
| 72 | 1 2 3 4 6 9 12 | 432 | 1 2 3 4 6 8 9 12 |
| 96 | 1 2 3 4 6 8 12 | 480 | 1 2 3 4 5 6 8 10 12 |
| 144 | 1 2 3 4 6 9 12 | 960 | 1 2 3 4 5 6 8 10 12 |
| 192 | 1 2 3 4 6 8 12 | 1920 | 1 2 3 4 5 6 8 10 12 |
| 216 | 1 3 9 | 2880 | 1 2 3 4 5 6 8 9 10 12 |
| 240 | 1 2 3 4 5 6 8 10 12 | 3840 | 1 2 3 4 5 6 8 10 12 |
| 288 | 1 2 3 4 6 8 9 12 | 4800 | 1 2 3 4 5 6 8 10 12 |
| 360 | 1 2 3 4 5 6 9 10 12 |  |  |

From Table 1, the parallelization orders have two common values, 1 and 3. In order to backward compatible with 16e in parallelization orders, the proposed FEC block sizes should be multiples of 3. Since the constitute code of CTC is duo-binary convolutional code, the proposed FEC block sizes should be multiples of 2. Furthermore, to obtain better performance for larger block size as well as low storage requirements, 4 should be a proper cycle length for ARP interleaver, which is the same as that of CTC in 16e, thus the interleaving length of the proposed block sizes should be multiples of 4. So the proposed FEC block sizes should be multiples of3×2×4.

The proposed FEC block size set is given below, which is composed of the FEC block sizes of CTC in 16e. K denotes the FEC block size and is not multiples of 7 due to tail-biting.

48≤K≤480K=23×3×ll=2,3,4,…,20

480<K≤960K=24×3×ll=11,12,13,…,20

960<K≤4800K=25×3×ll=11,12,13,…,50

The parallelization orders for the proposed FEC block sizes are given in Table 2 below. In this table, the parallelization orders of CTC in 16e are also given, which are marked by blue font.

Table 2 Parallelization orders for proposed FEC block size

| i | K | N | parallelization orders (up to 12) |
| --- | --- | --- | --- |
| 1 | 48 | 24 | 1 2 3 4 6 8 12 |
| 2 | 72 | 36 | 1 2 3 4 6 9 12 |
| 3 | 96 | 48 | 1 2 3 4 6 8 12 |
| 4 | 120 | 60 | 1 3 5 |
| 5 | 144 | 72 | 1 2 3 4 6 9 12 |
| 6 | 192 | 96 | 1 2 3 4 6 8 12 |
| 7 | 216 | 108 | 1 3 9 |
| 8 | 240 | 120 | 1 2 3 4 5 6 8 10 12 |
| 9 | 264 | 132 | 1 3 11 |
| 10 | 288 | 144 | 1 2 3 4 6 8 12 |
| 11 | 312 | 156 | 1 3 |
| 12 | 360 | 180 | 1 2 3 4 5 6 9 10 12 |
| 13 | 384 | 192 | 1 2 3 4 6 8 12 |
| 14 | 408 | 204 | 1 3 |
| 15 | 432 | 216 | 1 2 3 4 6 8 9 12 |
| 16 | 456 | 228 | 1 3 |
| 17 | 480 | 240 | 1 2 3 4 5 6 8 10 12 |
| 18 | 528 | 264 | 1 2 3 6 11 |
| 19 | 576 | 288 | 1 2 3 4 6 8 9 12 |
| 20 | 624 | 312 | 1 2 3 6 |
| 21 | 720 | 360 | 1 2 3 5 6 9 10 |
| 22 | 768 | 384 | 1 2 3 4 6 8 12 |
| 23 | 816 | 408 | 1 2 3 6 |
| 24 | 864 | 432 | 1 2 3 4 6 9 12 |
| 25 | 912 | 456 | 1 2 3 6 |
| 26 | 960 | 480 | 1 2 3 4 5 6 8 10 12 |
| 27 | 1056 | 528 | 1 2 3 4 6 1112 |
| 28 | 1152 | 576 | 1 2 3 4 6 8 12 |
| 29 | 1248 | 624 | 1 2 3 4 6 12 |
| 30 | 1440 | 720 | 1 2 3 4 5 6 9 10 12 |
| 31 | 1536 | 768 | 1 2 3 4 6 8 12 |
| 32 | 1632 | 816 | 1 2 3 4 6 12 |
| 33 | 1728 | 864 | 1 2 3 4 6 8 12 |
| 34 | 1824 | 912 | 1 2 3 4 6 12 |
| 35 | 1920 | 960 | 1 2 3 4 5 6 8 10 12 |
| 36 | 2112 | 1056 | 1 2 3 4 6 8 1112 |
| 37 | 2208 | 1104 | 1 2 3 4 6 12 |
| 38 | 2304 | 1152 | 1 2 3 4 6 8 9 12 |
| 39 | 2400 | 1200 | 1 2 3 4 5 6 10 12 |
| 40 | 2496 | 1248 | 1 2 3 4 6 8 12 |
| 41 | 2592 | 1296 | 1 2 3 4 6 9 12 |
| 42 | 2784 | 1392 | 1 2 3 4 6 12 |
| 43 | 2880 | 1440 | 1 2 3 4 5 6 8 9 10 12 |
| 44 | 2976 | 1488 | 1 2 3 4 6 12 |
| 45 | 3072 | 1536 | 1 2 3 4 6 8 12 |
| 46 | 3168 | 1584 | 1 2 3 4 6 9 1112 |
| 47 | 3264 | 1632 | 1 2 3 4 6 8 12 |
| 48 | 3456 | 1728 | 1 2 3 4 6 8 9 12 |
| 49 | 3552 | 1776 | 1 2 3 4 6 12 |
| 50 | 3648 | 1824 | 1 2 3 4 6 8 12 |
| 51 | 3744 | 1872 | 1 2 3 4 6 912 |
| 52 | 3840 | 1920 | 1 2 3 4 5 6 8 10 12 |
| 53 | 3936 | 1968 | 1 2 3 4 6 12 |
| 54 | 4128 | 2064 | 1 2 3 4 6 12 |
| 55 | 4224 | 2112 | 1 2 3 4 6 8 11 12 |
| 56 | 4320 | 2160 | 1 2 3 4 5 6 9 10 12 |
| 57 | 4416 | 2208 | 1 2 3 4 6 8 12 |
| 58 | 4512 | 2256 | 1 2 3 4 6 12 |
| 59 | 4608 | 2304 | 1 2 3 4 6 8 9 12 |
| 60 | 4800 | 2400 | 1 2 3 4 5 6 8 10 12 |

The comparison of average padding bit portion for the proposed FEC block sizes with the FEC block sizes of CTC in 16e is given in Figure 1.BLER performance for both the proposed FEC block sizes and the FEC block sizes in 16e system are shown in Figure 2. The simulation conditions are listed below in Table 3.

Table 3 Simulation condition for the proposed scheme

| Code Rate | 1/3 |
| --- | --- |
| Number of iterations | 8 |
| Decoding Algorithm | Improved Max-Log-MAP(with scaling factor 0.75 for the extrinsic information in iterative decoding) |
| Modulation | QPSK |
| Channel model | AWGN |

Figure 1 Comparison of average padding bit portion

Figure 2 BLER performance of CTC with FEC block sizes in Table 2

From Figure 2, it can be seen that the BLER performances of some FEC block sizes ,such as 360 bits, 432 bits, 480 bits, 1920 bits, 2880 bits, 3840bits and 4800 bits for CTC in 16e are worse than those of proposed adjacent ones, so the interleaver parameters of these block sizes should be optimized.

# Conclusions

A set of FEC block sizes is proposed in this contribution, which can reduce padding overhead, provide good parallelization order. Simulation results show that the proposed FEC block sizes have good BLER performances, but some ARP interleaver parameters of CTC in 16e should be optimized to achieve better BLER performances.

# Reference

[1] 80216m-08_003r6delta Draft IEEE 802.16m System Description Document

[2] P80216Rev2_D7 DRAFT Standard for Local and metropolitan area networks: Air Interface for Broadband Wireless Access Systems.

# Text Proposal to SDD

--------------------------------------------------- Start of the Text -----------------------------------------------------------

[Modify Chapter 11.13.1.3 in IEEE 802.16m-08/003r6 as follows;]

IEEE 802.16m uses the CTC (convolutional turbo code) of code rate 1/3 defined in the IEEE 802.16e standard where the CTC inner interleaver parameters for additional FEC block sizes are FFS while maintaining IEEE 802.16e CTC interleaver where the inner interleaver parameters for 802.16e CTC interleaver need to be  optimized.

--------------------------------------------------- End of the Text -----------------------------------------------------------