| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text changes to the IEEE 802.16m SDD (802.16m-08/003r6), Section 11.9.2.1 on the UL Fast Feedback Channel |  |
| Date Submitted | 2009-01-05 |  |
| Source(s) | Roshni Srinivasan, Hongmei Sun, Changlong Xu, Jong-Kae (JK) Fwu, Qinghua Li, Eddie Lin, Yuan Zhu, Hujun Yin, Rath Vannithamby, Sassan Ahmadi Intel Corporation | E-mail: {roshni.m.srinivasan, hongmei.sun, changlong.xu, jong-kae.fwu }@ intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | 802.16m-08/052, Call for Comments on 802.16m SDD (802.16m-08/003r6), Section 11.9.2.1 |  |
| Abstract | Proposal to adopt 2x6 tile structure for UL Fast Feedback. Design details for PHY structure of Primary and Secondary Fast Feedback Channels and corresponding text changes to 802.16m SDD (802.16m-08/003r6), Section 11.9.2.1 are also provided |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text changes to the IEEE 802.16m SDD (802.16m-08/003r6), Section 11.9.2.1 on the UL Fast Feedback Channel

Roshni Srinivasan, Hongmei Sun, Changlong Xu, Jong-Kae (JK) Fwu, Qinghua Li, Eddie Lin, Yuan Zhu, Hujun Yin, Rath Vannithamby, Sassan Ahmadi

Intel Corporation

# Introduction

This contribution provides details of the PHY structure for the IEEE 802.16m UL Primary and Secondary Fast Feedback channels.

Performance results to support the proposed design are provided in [1].

# References

[1] 	Hongmei Sun, Changlong Xu, Jong-Kae (JK) Fwu, Qinghua Li, Eddie Lin, Yuan Zhu, Hujun Yin, Roshni Srinivasan, Rath Vannithamby, Sassan Ahmadi, “Cross Evaluation of Proposed PHY Structures for the IEEE 802.16m UL Primary and Secondary Fast Feedback Channels”, IEEE C802.16m-09/0125, Jan 2009.

Comment: Delete Line 5, Page 107 since it is not necessary.

Update Lines 7 through 11 as shown by markup to clarify the difference between the primary and secondary fast feedback channels and the information that is carried

Move Lines 23 and 24 on Page 107 to the end of this subsection as shown by markup

Suggested Remedy: Adopt proposed SDD text change #1 in C802.16m-09/0128.doc or its latest revision (this document).

# Proposed SDD text Change #1

#### UL Fast Feedback Channel

The UL fast feedback channel carries channel quality feedback and MIMO feedback. Transmission of BW REQ indicators on the UL fast feedback channel is FFS.

The mapping of UL fast feedback information into physical channels is described in Section 1.1.1.1.1.

There are two types of UL fast feedback control channels: primary fast feedback channel (PFBCH) and secondary fast feedback channel (SFBCH). The UL primary fast feedback control channel provides carries 4 or 6 bits of information, providing wideband feedback information including channel quality feedback and MIMO feedback. It is used to support robust feedback reports. The UL secondary fast feedback channel carries 12 to 24 bits of narrowband CQI and MIMO feedback information. The SFBCH can be used to support CQI reporting at higher code rate and thus more CQI information bits. The SFBCH can be allocated in a non-periodic manner based on traffic, channel conditions etc. The number of bits carried in the SFBCH can be adaptive.

Comment: Update Lines 13 Page 107 through Line 4, Page 107 as shown by markup to clarify how multiplexing for the primary and secondary fast feedback channels with other control and data channels is done. Delete unnecessary text on lines 17 and 18  on page 107

Suggested Remedy: Adopt proposed SDD text change #2 in C802.16m-09/0128.doc or its latest revision

# Proposed SDD text Change #2

##### Multiplexing with other control channels and data channels

The PFBCH and SFBCH are UL fast feedback channel is FDM with other UL control and data channels within a UL subframe.

The PFBCH and SFBCH UL fast feedback channel starts at a pre-determined locations, with the size defined in a DL broadcast control message. PFBCH and SFBCH allocations to an AMS can be periodic and the allocations are configurable. For periodic allocations, the specific type of feedback information carried on each fast feedback opportunity can be different.

Comment: Update Line 20 and Line 21, Page 107 as shown by markup to indicate that FDM is used for multiplexing of multiple PFBCHs and SFBCHs.

Suggested Remedy: Adopt proposed SDD text change #3 in C802.16m-09/0128.doc or its latest revision (this document).

# Proposed SDD text Change #3

The UL fast feedback channel carries one or more types of fast feedback information. The use of TDM/FDM or CDM to multiplex fast feedback channels from one or more users is FFS. Multiple PFBCHs and SFBCHs for one or more users are multiplexed using FDM.

Comment: Adopt a 2x6 tile structure for the Fast Feedback Channel since performance evaluation has shown good performance under different mobility conditions. See contribution C802.16m-09/0125.ppt or its latest revision for details.

Delete the sentence on 6x2 tile structure on Lines 26 and 27, Page 107 as shown by markup.

Suggested Remedy: Adopt proposed SDD text change #4 in C802.16m-09/0128.doc or its latest revision(this document).

# Proposed SDD text Change #4

A UL feedback mini-tile (FMT) is defined as 2 contiguous subcarriers by 6 OFDM symbols. A UL FMT of 6 contiguous subcarriers by 2 OFDM symbols is FFS.

Comment: Adopt a 2x6 tile structure with 3 distributed tiles for the Primary Fast Feedback Channels since performance evaluation has shown good performance under different mobility conditions. See contribution C802.16m-09/0125.ppt or its latest revision for details.

Delete the sentences on Lines 27 through 29, Page 107 as shown by markup in the proposed text. Adopt the tile structures and corresponding text for PFBCH in the proposed text.

Suggested Remedy: Adopt proposed SDD text change #5 in C802.16m-09/0128.doc or its latest revision (this document).

Note that a similar comment has been submitted for the SFBCH and this text is intended to precede the text provided for the SFBCH.

# Proposed SDD text Change #5

The primary and secondary fast feedback channels are comprised of NFMT distributed FMTs, where NFMT is FFS. One LRU consists of 9 FMTs and can be shared by multiple fast feedback channels.

The UL FMT structure for the PFBCH is shown in Figure 1. The PFBCH structure comprises of 3 FMTs, which are chosen from different UL DRUs for frequency diversity as shown in Figure 2. The procedure for allocation of resources for transmission of UL control information and the formation of DRUs for such transmission is TBD.

Figure 1: UL FMT structure for the PFBCH

Figure 2: PFBCH comprising of three distributed 2x6 UL FMTs

Comment: Adopt a 2x6 tile structure with 3 distributed tiles for the Secondary Fast Feedback Channel since performance evaluation has shown good performance under different mobility conditions. See contribution C802.16m-09/0125.ppt or its latest revision for details.

Delete the sentences on Lines 27 through 29, Page 107 as shown by markup. Adopt the tile structures and corresponding text for the SFBCH shown below.

Suggested Remedy: Adopt proposed SDD text change #6 in C802.16m-09/0128.doc or its latest revision (this document).

Note that a similar comment has been submitted for the PFBCH and this text is intended to follow the text provided for the PFBCH.

# Proposed SDD text Change #6

The primary and secondary fast feedback channels are comprised of NFMT distributed FMTs, where NFMT is FFS. One LRU consists of 9 FMTs and can be shared by multiple fast feedback channels.

The UL FMT used for the SFBCH is a time-frequency block of 2 subcarriers by 6 OFDM symbols and has 2 pilots in fixed locations as shown in Figure 3. The SFBCH is constructed from 3 UL FMTs, which are chosen from different UL DRUs as shown in Figure 4. The procedure for allocation of resources for transmission of UL control information and the formation of DRUs for such transmission is TBD.

The pilot sequence p0p1=[1,ejθ] is used in each FMT for each user.  is a sector specific phase rotation that is configured to mitigate inter cell interference. The value of  for the three FMTs may be different and the detailed design of  is TBD.

Figure 3: UL FMT structure for the SFBCH

Figure 4: SFBCH comprising of three distributed 2x6 UL FMTs

Comment: Channel coding details and performance evaluation for PFBCH and SBCH are provided  contribution C802.16m-09/0125.ppt or its latest revision.

Adopt the following text for PFBCH and SFBCH PHY structure details shown below at the end of section 11.9.2.1.2. Add a new subsection with details on channel coding for SFBCH.

Suggested Remedy: Adopt proposed SDD text change #7 in C802.16m-09/0128.doc or its latest revision (this document).

# Proposed SDD text Change #7

Figure 5: Mapping of information in the PFBCH

Each PFBCH carries  bits, where . The number of information bits carried in the PFBCH is FFS. The symbol generation procedure is shown in Figure 5. First, the 4 to 6 information bits   shall be encoded into12 bits b0b1b2⋯b11using the semi-orthogonal sequences s0,n,s1,n,s2,n,...,s11,n (n=0,1,...,2l−1).

- For 4 information bits or l=4, sequences s0,n,s1,n,s2,n,...,s11,n (n=0,1,...,15) listed in Table 1 shall be used.

- For 5 information bits or l=5, sequences s0,n,s1,n,s2,n,...,s11,n (n=0,1,...,31) listed in Table 2 shall be used.

- For 6 information bit or l=6, sequences s0,n,s1,n,s2,n,...,s11,n (n=0,1,...,63) listed in Table 3 shall be used.

Table 1 : Semi-orthogonal sequences for 4bits information in PFBCH

| n | S0,n | S1,n | S2,n | S3,n | S4,n | S5,n |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | -0.19-0.24i | 0.28-0.05i | 0.26+0.12i | 0.28-0.08i | 0.26-0.18i | 0.21+0.14i |
| 1 | 0.06+0.28i | -0.03-0.31i | -0.08+0.24i | 0.16+0.23i | 0.13-0.26i | -0.15+0.27i |
| 2 | -0.09-0.29i | -0.25-0.18i | 0.1+0.27i | -0.26-0.06i | 0.27+0.03i | -0.18-0.23i |
| 3 | -0.23-0.19i | -0.3-0.05i | -0.23+0.15i | 0.07-0.28i | -0.11-0.25i | 0.2-0.2i |
| 4 | 0.08+0.29i | 0.26-0.12i | 0.31+0.03i | 0.29+0.09i | -0.04+0.32i | -0.17-0.22i |
| 5 | -0.29+0.05i | -0.04+0.26i | -0.05-0.26i | 0.21-0.19i | -0+0.3i | 0.06+0.3i |
| 6 | 0.23-0.19i | 0.26-0.16i | 0.13+0.18i | 0.2+0.24i | -0.22+0.22i | 0.24+0.16i |
| 7 | 0.25+0.15i | 0.11+0.29i | 0.12-0.27i | -0.02-0.28i | 0.25-0.17i | 0.15-0.23i |
| 8 | 0.27-0.03i | 0.08+0.3i | -0.24+0.17i | 0.24-0.15i | 0.08-0.23i | 0.16+0.23i |
| 9 | -0.07-0.31i | 0.17+0.23i | 0.09-0.3i | -0.21-0.19i | 0.04+0.26i | -0.11+0.28i |
| 10 | -0.22+0.06i | 0.24+0.18i | 0.28-0.04i | -0.28+0.1i | 0.29+0.03i | 0.29+0.01i |
| 11 | 0.18-0.23i | 0.15+0.27i | 0.31-0.02i | -0.07+0.27i | -0.04-0.3i | 0.29-0.02i |
| 12 | -0.15-0.28i | -0.04+0.3i | -0.15+0.2i | 0.22+0.11i | 0.26-0.2i | -0.05-0.26i |
| 13 | -0.01-0.32i | 0.28-0.06i | -0.11-0.29i | -0.04-0.25i | -0.16-0.25i | -0.26+0.15i |
| 14 | -0.27+0.14i | -0.15+0.22i | -0.08-0.29i | 0.22+0.21i | -0.23-0.22i | 0.29-0.01i |
| 15 | -0.07+0.28i | 0.27-0.07i | 0.14+0.27i | 0.05-0.29i | 0.08-0.26i | -0.1+0.25i |
| n | S6,n | S7,n | S8,n | S9,n | S10,n | S11,n |
| 0 | 0.01-0.27i | -0.23+0.18i | -0.04-0.27i | 0.22-0.21i | 0.27-0.09i | 0.28+0.11i |
| 1 | -0.24+0.13i | 0.07+0.27i | 0.3-0.02i | 0.3-0.03i | -0.16-0.25i | 0.21+0.16i |
| 2 | -0.05-0.26i | -0.1-0.3i | 0.25-0.15i | -0.15-0.23i | 0.25-0.14i | 0.05-0.27i |
| 3 | 0.18-0.26i | -0.2+0.14i | -0.28+0.08i | 0.29+0.12i | -0.1+0.26i | -0.1+0.27i |
| 4 | 0.28+0.03i | -0.3+0.03i | -0.21-0.1i | -0.14-0.26i | -0.21+0.17i | 0.28+0.01i |
| 5 | 0.07-0.28i | 0.15-0.25i | 0.17-0.19i | 0.27-0.14i | 0.04-0.29i | -0.08-0.31i |
| 6 | -0.17-0.24i | 0.04+0.23i | -0.29+0.05i | -0.25+0.12i | 0.25-0.15i | -0.31+0.05i |
| 7 | 0.25+0.13i | -0.02+0.3i | 0.07-0.28i | 0.26+0.1i | 0.26-0.07i | -0.29-0.1i |
| 8 | -0.28+0.07i | -0.02-0.3i | -0.16+0.24i | -0.19-0.2i | 0.29-0.08i | 0.3-0.07i |
| 9 | 0.22+0.2i | -0.2+0.22i | -0.08+0.26i | 0.04-0.27i | -0.12-0.24i | 0.26+0.14i |
| 10 | -0.23+0.1i | 0.3-0.1i | -0.22-0.21i | -0.03+0.31i | 0.09+0.28i | 0.23+0.17i |
| 11 | -0.18-0.18i | 0.04+0.27i | 0.15+0.25i | 0.29-0.03i | -0.2+0.23i | -0.03-0.27i |
| 12 | 0.27+0.13i | 0.29+0.05i | -0.13-0.26i | -0.28+0.08i | -0.09-0.25i | 0.3+0i |
| 13 | -0.3+0.03i | -0.27-0.05i | 0.1-0.25i | -0.14+0.23i | -0.08+0.25i | 0.28+0.12i |
| 14 | 0.21-0.19i | -0.28-0.05i | 0.24+0.11i | -0.3-0.1i | 0.25-0.01i | 0.12+0.24i |
| 15 | 0.27+0.13i | 0.25+0.17i | 0.21+0.21i | -0.28-0.06i | 0.23+0.08i | -0.24+0.18i |

Table 2 : Semi-orthogonal sequences for 5bits information in PFBCH

| n | S0,n | S1,n | S2,n | S3,n | S4,n | S5,n |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.22-0.14i | 0.23+0.2i | -0.07+0.14i | 0.27-0.18i | -0.07+0.08i | -0.2-0.25i |
| 1 | -0.3+0.05i | 0.03+0.26i | -0.07-0.32i | 0-0.12i | 0.26-0.07i | 0.05-0.29i |
| 2 | 0.03-0.29i | -0.25-0.03i | -0.1+0.28i | 0.28-0.13i | -0.09+0.08i | 0.28+0.1i |
| 3 | -0.07-0.26i | 0.24+0.21i | -0.1-0.16i | -0.25+0.17i | 0.07+0.31i | 0.19+0.11i |
| 4 | 0.17-0.32i | -0.1+0i | 0.24-0.15i | 0.02+0.09i | 0.19-0.27i | -0.12+0.31i |
| 5 | -0.11+0.17i | 0.16+0.25i | -0.15-0.2i | 0.29-0.18i | 0.3+0.05i | 0.23-0.2i |
| 6 | -0.11-0.34i | 0.12+0.19i | -0.29-0.18i | -0.03+0.3i | -0.23-0.25i | -0.25-0.24i |
| 7 | 0.26-0.19i | 0.26-0.04i | -0.27-0.13i | 0.3-0.03i | -0.01-0.06i | 0.11+0.3i |
| 8 | -0.25+0.01i | -0.08-0.26i | -0.21-0.23i | -0.09-0.3i | 0.2-0.23i | 0.24+0.2i |
| 9 | -0.29-0.07i | 0.02+0.27i | -0.09+0.33i | 0.23-0.24i | 0.16-0.24i | -0-0.23i |
| 10 | 0.29-0.14i | 0.12-0.3i | -0.02-0.33i | 0.2-0.16i | -0.07-0.32i | 0.05-0.13i |
| 11 | -0.26+0.22i | -0.15+0.12i | 0.21-0.17i | 0.03-0.29i | -0.23-0.24i | -0.19-0.27i |
| 12 | -0.28+0.13i | 0.19-0.2i | 0.29-0.09i | 0.35+0.02i | -0.25-0.14i | -0.08+0.05i |
| 13 | -0.28+0.16i | -0.27-0.04i | -0.1+0.28i | -0.15-0.04i | 0.17+0.16i | -0.29+0.12i |
| 14 | -0.29+0.12i | 0.27+0.14i | 0.04+0.31i | 0.26+0.13i | -0.14-0.29i | 0.29+0.1i |
| 15 | 0.1-0.22i | -0.07+0.14i | 0.02-0.13i | 0.27+0.19i | 0.01-0.35i | 0.06+0.28i |
| 16 | 0.28-0.04i | -0.3+0.16i | -0.18-0.22i | 0.33-0.1i | -0.16-0.29i | 0.1+0.16i |
| 17 | 0.06+0.17i | 0.12-0.31i | -0.05+0.3i | -0.32-0.05i | 0.27+0.18i | 0.02+0.32i |
| 18 | -0.3+0.07i | -0.14+0.27i | 0.16-0.23i | 0.01+0.03i | -0.08+0.31i | -0.27+0.07i |
| 19 | 0.1+0.22i | 0.18-0.24i | 0.32+0.11i | 0.29+0.04i | 0.08+0.22i | 0.27-0.12i |
| 20 | -0.27+0.15i | 0.21-0.27i | 0.3-0.06i | -0.29+0.08i | -0.01-0.32i | 0.05-0.01i |
| 21 | -0.21-0.06i | -0.28-0.1i | 0.08+0.15i | 0.3-0.1i | 0.03-0.29i | -0.24+0.24i |
| 22 | 0.19-0.22i | 0.26-0.05i | -0.1+0.25i | -0.22+0.23i | 0.15-0.31i | -0.16-0.28i |
| 23 | -0.01+0.35i | -0.18-0.23i | -0.32+0.01i | 0.28-0.12i | 0.22+0.02i | 0.28+0.07i |
| 24 | 0.17-0.21i | 0.11-0.28i | 0.1+0.33i | -0.21-0.13i | 0.25-0.19i | 0.01+0.26i |
| 25 | 0.27+0.02i | -0.16+0.12i | -0.25+0.13i | -0.11+0.31i | -0.32+0.08i | 0.21+0.27i |
| 26 | 0.22-0.13i | -0.23-0.05i | 0.31+0.02i | -0.22-0.22i | -0.29+0.17i | 0.13-0.32i |
| 27 | 0.27-0.05i | 0.3+0.11i | 0.26-0.2i | 0.07-0.31i | -0.11-0.03i | -0.1+0.27i |
| 28 | -0.27-0.19i | -0.24-0.16i | -0.18-0.13i | 0.01-0.24i | 0.21-0.27i | -0.28-0.12i |
| 29 | -0.15+0.17i | -0.28-0.11i | -0.29-0.1i | 0.29+0.13i | 0.24+0.25i | -0.25-0.2i |
| 30 | 0.25-0.02i | 0.3-0.07i | -0.26+0.04i | 0.27+0.13i | -0.28-0.04i | -0.17-0.11i |
| 31 | 0.23+0.07i | -0.23+0.09i | -0.08+0.19i | 0.31+0.01i | 0.1+0.23i | -0.33+0.07i |
| n | S6,n | S7,n | S8,n | S9,n | S10,n | S11,n |
| 0 | 0.04-0.21i | 0.2-0.2i | 0.05-0.34i | 0.35-0.07i | 0.22-0.25i | 0.18-0.28i |
| 1 | -0.15+0.22i | 0.04+0.3i | 0.17-0.2i | 0.09-0.31i | -0.05-0.3i | 0.06-0.34i |
| 2 | 0.05+0.31i | 0.03-0.32i | -0.17-0.25i | 0.14-0.33i | 0.32+0i | -0.12+0.15i |
| 3 | -0.3-0.07i | 0.1-0.28i | 0.01-0.32i | -0.27-0.09i | 0.26-0.18i | 0.28-0.04i |
| 4 | 0.08+0.32i | -0.17-0.28i | -0.22-0.17i | -0.3+0.16i | -0.04-0.16i | 0.21+0.28i |
| 5 | -0.14-0.3i | 0.26+0.01i | -0.31-0.13i | -0.03-0.24i | 0.26-0.03i | -0.06+0.3i |
| 6 | 0.01-0.31i | 0.07+0.16i | 0.11-0.17i | -0.06-0.19i | 0.21+0.25i | -0.24-0.09i |
| 7 | 0.29-0.04i | 0.2-0.21i | 0.22-0.18i | -0.18-0.29i | -0.32-0.01i | 0.24+0.11i |
| 8 | -0.11+0.21i | 0.15+0.09i | -0.14-0.29i | -0.09+0.33i | 0.21-0.19i | -0.28+0.14i |
| 9 | 0.05-0.13i | -0.19+0.27i | 0.3-0.04i | 0.14+0.13i | 0.25-0.16i | -0.27+0.24i |
| 10 | 0.25+0.25i | 0.05-0.28i | 0.21+0.21i | -0.01+0.17i | 0.27+0.07i | -0.21+0.23i |
| 11 | 0.07+0.22i | 0.3-0.17i | -0.19+0.17i | -0.04+0.26i | -0.01+0.28i | 0.3-0.04i |
| 12 | 0.02+0.29i | -0.24-0.05i | 0.15-0.28i | 0.22+0.25i | 0.33-0.08i | 0.16-0.16i |
| 13 | 0.17+0.24i | 0.06-0.24i | 0.22-0.19i | 0.13+0.32i | 0.23+0.28i | -0.15+0.21i |
| 14 | -0.07-0.2i | -0.31-0.09i | -0.27+0.08i | 0.02-0.16i | -0.09+0.28i | -0.23+0.18i |
| 15 | -0.2-0.27i | 0.26+0.18i | -0.04+0.3i | -0.08+0.3i | 0.18-0.26i | -0.15+0.24i |
| 16 | -0.02+0.07i | -0.24+0.22i | -0.09-0.26i | -0.31-0.14i | -0.15+0.17i | -0.32+0.04i |
| 17 | 0.08-0.17i | 0.16+0.27i | 0.27-0.12i | -0.29+0.05i | 0.01+0.14i | 0.3+0.19i |
| 18 | -0.02+0.34i | -0.16+0.27i | -0.13-0.25i | -0.27-0.02i | 0.22+0.15i | 0.22+0.24i |
| 19 | 0.17-0.29i | -0.08-0.34i | 0-0.17i | -0.23+0.03i | 0.26-0.21i | -0.26-0.14i |
| 20 | 0.29-0.11i | 0.22+0.04i | -0.18-0.15i | 0.34+0i | 0.12+0.25i | -0.3-0.13i |
| 21 | 0.3-0.18i | 0.21+0.2i | -0.06-0.35i | -0.1-0.27i | 0.18+0.04i | 0.22-0.2i |
| 22 | 0.17-0.32i | -0.17+0.02i | -0.21-0.27i | 0.03+0.29i | -0.07-0.16i | 0.15+0.17i |
| 23 | -0.1-0.31i | -0.23-0.05i | 0.23-0.15i | 0.31+0.07i | -0.19+0.12i | 0.13+0.26i |
| 24 | 0.05+0.27i | 0.02+0.22i | -0.1+0.32i | 0.26-0.12i | 0.32-0.14i | -0.09+0.18i |
| 25 | -0.24+0.24i | -0.12+0.31i | -0.26+0.03i | 0.08+0.23i | 0.17-0.21i | 0.09-0.21i |
| 26 | 0.3+0.1i | 0.26-0.18i | -0.08-0.3i | -0+0.2i | -0.14+0.06i | -0.29+0.03i |
| 27 | -0.05+0.06i | -0.24-0.16i | 0.24-0.25i | 0.29+0.17i | -0.3+0.11i | -0.29+0.1i |
| 28 | -0.17-0.15i | 0.32-0.1i | 0.03+0.26i | 0.33-0.06i | 0.34-0.05i | -0.09-0.16i |
| 29 | -0.08+0.21i | 0.28-0.2i | -0.26+0.01i | 0.24-0.24i | -0.3+0.05i | 0.09+0.09i |
| 30 | 0.28+0.15i | -0.3-0.05i | -0.32+0.02i | -0.2-0.15i | 0.34+0.03i | 0.29-0.08i |
| 31 | 0.18-0.17i | 0.18+0.27i | -0.05-0.33i | -0.01+0.3i | -0.24-0.17i | -0.09+0.31i |

Table 3 : Semi-orthogonal sequences for 6bits information in PFBCH

| n | S0,n | S1,n | S2,n | S3,n | S4,n | S5,n |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.1+0.09i | 0.09-0.23i | -0.18+0.32i | 0.05+0.24i | 0.07+0.1i | 0.12+0.08i |
| 1 | -0.17-0.2i | -0.18+0.18i | -0.22-0.1i | -0.31-0.01i | 0.13+0.33i | 0.25+0.19i |
| 2 | -0.01-0.38i | 0.06+0i | -0.05-0.29i | 0.05+0i | -0.22+0.29i | -0.02+0.13i |
| 3 | -0.27-0.27i | 0.24+0.32i | -0.03-0.37i | 0.37+0.12i | -0.22-0.17i | 0.15-0.36i |
| 4 | -0.27-0.24i | -0.15-0.19i | -0.01-0.02i | -0.17+0.22i | -0.19+0.24i | 0.17-0.33i |
| 5 | -0.15+0.05i | -0.19+0.27i | -0.01+0.37i | 0.39+0.04i | 0.14-0.04i | 0.15+0.33i |
| 6 | -0.04+0.16i | -0.41+0.1i | 0.15+0.19i | -0.01+0.32i | -0.06+0.22i | 0.19+0.26i |
| 7 | -0.1+0.22i | 0.36-0.12i | -0.4-0.01i | 0.29+0.23i | -0.11-0.4i | -0.09-0.06i |
| 8 | -0.22-0.13i | -0.08-0.02i | -0.26-0.15i | 0.41+0.05i | 0.07-0.03i | 0.13-0.29i |
| 9 | -0.23-0.17i | 0.21+0.21i | -0.08-0.2i | -0.02-0.33i | 0.05+0.22i | 0.35-0.15i |
| 10 | -0.3-0.1i | -0+0.05i | -0.21+0.12i | 0.36+0.19i | -0.29+0.05i | 0.25+0.19i |
| 11 | -0.27+0.27i | -0.12-0.07i | 0.23+0.25i | 0.2+0.03i | -0.34-0.18i | 0.32+0.13i |
| 12 | 0.2+0.04i | -0.16-0.07i | -0.08-0.29i | -0.13+0.15i | -0.12+0i | 0.26+0.22i |
| 13 | -0.33+0.06i | 0.01-0.3i | -0.09+0.33i | -0.01-0.25i | 0.16+0.28i | 0.17+0.17i |
| 14 | -0.15-0.05i | 0.16+0.18i | -0.15+0.15i | 0.25-0.31i | -0.24-0.01i | -0.16+0.39i |
| 15 | -0.05+0.41i | 0.11+0.31i | -0.05+0.07i | 0.35-0.26i | -0.38-0.05i | -0.04+0.34i |
| 16 | -0.23-0.09i | -0.36+0.12i | 0.05-0.37i | -0-0.17i | -0.3-0.16i | 0.26+0.07i |
| 17 | -0.14+0.3i | 0.22-0.12i | -0.34+0.02i | -0.16-0.02i | 0.01-0.34i | 0.12+0.29i |
| 18 | 0.05-0.33i | -0.33-0.06i | 0+0.24i | 0.14+0i | 0.07+0.17i | 0.19+0.38i |
| 19 | -0.13-0.32i | 0.1-0.27i | -0.25-0.17i | 0.23-0.04i | -0.36-0.12i | 0.13+0.12i |
| 20 | -0.07-0.2i | 0.2-0.31i | 0.03+0.41i | -0.19-0.04i | 0.02-0.14i | -0-0.12i |
| 21 | -0.06+0.1i | -0.32+0.17i | -0.31-0.08i | -0.25+0.17i | 0.21-0.2i | 0.28-0.14i |
| 22 | -0.3+0.11i | -0.3-0.19i | -0.04+0.09i | 0.02-0.18i | -0.23+0.16i | 0.04+0.27i |
| 23 | -0.23-0.07i | 0.03+0.05i | -0.12-0.34i | -0.04+0.32i | -0.24+0i | -0.36+0.1i |
| 24 | -0.35+0.03i | -0.1-0.41i | 0.1-0.3i | 0.05+0.25i | 0.19-0.23i | 0.06+0.02i |
| 25 | 0.21-0.2i | -0.39+0.05i | -0.11-0.04i | -0.12-0.17i | -0.28+0.02i | 0.37-0.06i |
| 26 | -0.27+0.26i | 0.21+0.07i | 0.28-0.07i | 0.16+0.22i | 0.09+0.15i | -0.04-0.04i |
| 27 | -0.27-0.22i | 0.22-0.2i | 0.14+0.02i | 0.28-0.05i | 0.06-0.15i | -0.09+0.32i |
| 28 | -0.09-0.08i | 0.29+0.06i | -0.13+0.33i | 0.18-0.29i | 0.22-0.26i | 0.01+0.06i |
| 29 | -0.04-0.28i | -0.02+0.23i | -0.32+0.22i | -0.03+0.03i | -0.16-0.3i | 0.26+0.17i |
| 30 | -0.26-0.05i | 0.43-0.06i | -0.24+0.35i | -0.03+0.41i | -0.18+0.02i | 0.09-0.13i |
| 31 | -0.23-0.22i | -0.19+0.05i | 0.13-0.03i | 0.05+0.35i | -0.09+0.2i | 0.05+0.29i |
| 32 | -0.13+0.01i | 0.32-0.07i | -0.3+0.04i | -0.22-0.02i | -0.15+0.32i | -0.09+0.13i |
| 33 | -0.3+0.25i | -0.3-0.03i | -0.16-0.08i | 0.18+0.07i | 0.11+0.14i | 0.36-0.05i |
| 34 | 0.01+0.27i | -0.18-0.13i | -0.25+0.05i | 0.06+0.27i | 0.01+0i | 0.07+0.34i |
| 35 | -0.23+0.19i | -0.14+0.11i | -0.01-0.36i | 0.17-0.18i | 0.05+0.16i | -0.19+0.21i |
| 36 | 0.06-0.35i | -0.19+0.12i | 0.06+0.13i | 0.26-0.15i | -0.06+0.13i | 0.2+0.09i |
| 37 | 0.11-0.1i | 0.33-0.04i | 0.36+0.24i | 0.15+0.39i | -0.08-0.3i | 0.01+0.33i |
| 38 | -0.03-0.15i | 0.07-0.31i | -0.02-0.23i | 0.13+0.24i | -0.02+0.32i | 0.19-0.07i |
| 39 | 0.09-0.08i | -0.26-0.14i | -0.37+0.14i | 0.32-0.01i | -0.19-0.08i | 0.17-0.39i |
| 40 | 0.11+0.16i | 0.08+0.05i | -0.04+0.38i | -0.33+0.22i | -0.3+0.02i | 0.14+0.31i |
| 41 | -0.28+0.09i | 0.14-0.03i | 0.09+0.17i | -0.01+0.3i | 0.05+0.39i | -0.09+0.2i |
| 42 | -0.07-0.23i | 0.07-0.2i | -0.25+0.13i | 0.22+0.22i | -0.33+0.01i | 0.18+0.04i |
| 43 | 0.08+0.01i | 0.26-0.15i | 0.26+0.2i | 0.2-0.11i | -0.17-0.31i | 0.16+0i |
| 44 | -0.02-0.34i | 0.02+0.03i | 0.2+0.05i | 0.12+0.31i | 0.16+0.23i | 0.16+0.17i |
| 45 | -0.33-0.02i | -0.38+0.02i | -0.02+0.16i | 0.07+0.14i | 0.05-0.34i | -0.05-0.23i |
| 46 | 0.18+0.23i | -0.18+0.08i | 0.09+0.3i | 0.13+0.26i | -0.23+0.03i | -0.2+0.22i |
| 47 | -0.1+0.31i | 0.17+0.27i | 0.13-0.17i | -0.2+0.01i | -0.16-0.07i | 0.28+0.14i |
| 48 | 0.18-0.28i | -0.06+0.14i | -0.1-0.14i | 0.3-0.24i | 0.08-0.14i | 0.09+0.3i |
| 49 | 0.26-0.06i | -0.11+0.25i | 0.03+0.29i | 0.31-0.02i | -0.4-0.09i | 0.25-0.21i |
| 50 | 0.17-0.36i | 0.09+0.23i | -0.29+0.17i | 0.03+0.04i | 0.04+0.39i | 0.32+0.09i |
| 51 | 0.19-0.27i | -0.08+0.2i | -0.11+0.26i | 0.12-0.35i | -0.31-0.23i | -0.21+0.2i |
| 52 | -0.31-0.17i | -0.11+0.16i | 0.14-0.19i | 0.04-0.26i | 0.12-0.38i | 0.19+0.32i |
| 53 | 0.25+0.08i | 0.01+0.35i | -0.12-0.19i | 0.24+0.21i | -0.18+0.38i | -0.36+0i |
| 54 | 0.17-0.09i | -0.32+0.05i | -0.17-0.19i | 0.17-0.23i | 0.32+0.06i | 0.2-0.12i |
| 55 | 0.16-0.26i | -0.18-0.08i | 0.13+0.35i | -0.13+0.31i | -0.08+0.08i | -0.1-0.16i |
| 56 | -0.35-0.01i | 0.35+0.22i | -0.22-0.01i | 0.17+0.12i | 0.04+0.04i | 0.23+0.27i |
| 57 | 0.15-0.09i | 0.24-0.23i | -0.25-0.32i | 0.32+0.06i | -0.06+0.24i | 0-0.26i |
| 58 | 0.11+0.13i | 0.36-0.13i | -0+0.08i | 0.03+0.26i | -0.12-0.17i | 0.28-0.21i |
| 59 | -0.16+0.26i | 0.37+0.23i | 0-0.25i | 0.04+0.26i | -0.1+0.34i | -0.05+0.2i |
| 60 | -0.22+0.17i | -0.28-0.01i | -0.24-0.04i | 0.19+0.1i | -0.24+0.28i | -0.32-0.11i |
| 61 | 0.05-0.01i | 0.13-0.28i | -0.23-0.06i | 0.31-0.3i | -0.06-0.27i | -0.1+0.14i |
| 62 | -0.15-0.34i | 0.1+0.14i | 0.25+0.15i | 0.33-0.03i | 0.11-0.3i | -0.05-0.17i |
| 63 | 0.32-0.06i | 0.01+0.09i | -0.33-0.02i | 0.01-0.08i | -0.29-0.05i | 0.06-0.31i |
| n | S6,n | S7,n | S8,n | S9,n | S10,n | S11,n |
| 0 | 0.21+0.34i | -0.31-0.26i | 0.09-0.25i | -0.27-0.2i | 0.23+0.21i | 0.31-0.05i |
| 1 | 0.16+0.1i | 0.3+0.3i | 0.11+0.12i | 0.23+0.15i | -0.35+0.11i | 0.14-0.1i |
| 2 | 0.39-0.07i | -0.23+0.15i | 0.24-0.06i | -0.37-0.09i | -0.02+0.29i | 0.26-0.12i |
| 3 | 0.12-0.1i | 0.05+0.1i | 0.03+0i | 0.14-0.17i | -0.19+0.19i | -0.14-0.03i |
| 4 | 0.27+0.19i | 0.13-0.04i | -0.07+0.03i | -0.15-0.39i | 0.09+0.36i | -0.22+0.07i |
| 5 | 0.24-0.21i | 0.08-0.11i | -0.03+0.21i | 0.24+0.32i | -0.21-0.03i | -0.14-0.2i |
| 6 | 0.2+0.03i | 0.07-0.26i | 0.24+0.33i | 0.18-0.21i | 0.04+0.1i | 0.11+0.3i |
| 7 | -0.14+0.19i | -0.13+0.1i | -0.17-0.16i | 0.34+0.05i | 0.13-0.09i | 0.16+0.11i |
| 8 | 0+0.33i | -0.27+0.2i | -0.26-0.08i | -0.32+0.07i | 0.11-0.26i | -0.22+0.2i |
| 9 | -0.19+0.38i | -0.31-0.04i | 0.07-0.19i | 0.09+0.16i | -0.26+0.06i | 0.08+0.2i |
| 10 | 0.28+0.17i | -0.4+0.07i | 0.12-0.02i | 0.17+0.14i | 0.05+0.3i | -0.07+0.26i |
| 11 | -0.22+0.11i | -0.09-0.22i | -0.17-0.24i | -0.02+0.29i | 0.2+0.13i | -0.09-0.24i |
| 12 | -0.16+0.27i | -0.35+0.19i | 0.26-0.18i | 0.35-0.03i | 0.19-0.02i | 0.33+0.22i |
| 13 | 0.21+0.26i | -0.07-0.09i | -0.04+0.08i | -0.14+0.16i | -0.14+0.38i | -0.25-0.23i |
| 14 | 0.19-0.23i | -0.27-0.03i | -0.15-0.33i | -0.21+0.09i | -0.03-0.1i | -0.33-0.07i |
| 15 | -0.02+0.3i | 0.02-0.18i | 0.19+0.21i | -0.08+0.08i | -0.03+0.06i | 0.16+0.09i |
| 16 | -0.1-0.14i | -0.13-0.03i | -0.25+0.25i | 0.14-0.16i | 0.22+0.29i | -0.12+0.26i |
| 17 | 0.06-0.16i | -0.06+0.32i | 0.2+0.2i | 0.27-0.03i | -0.12+0.34i | -0.21+0.07i |
| 18 | -0.25+0.21i | 0.22+0.26i | -0.32-0.22i | 0.04-0.03i | 0.19+0.04i | -0.22-0.17i |
| 19 | -0.36-0.13i | 0.16+0.2i | 0.08-0.09i | -0.12+0.24i | 0.31-0.03i | 0.05+0.28i |
| 20 | -0.19+0.1i | -0.24-0.07i | 0.01-0.36i | 0.37+0.1i | 0.23+0.25i | -0.21+0.18i |
| 21 | 0.33-0.02i | -0.16+0.36i | 0.2+0.11i | -0.37-0.08i | -0.09+0i | -0.06+0.07i |
| 22 | 0.11-0.31i | -0.23-0.25i | 0.24+0.05i | -0.16+0.22i | -0.35-0.01i | 0.3+0.08i |
| 23 | 0.24-0.14i | 0.11+0.33i | -0.03+0.2i | 0.28+0.16i | 0.09+0.14i | 0.25+0.27i |
| 24 | 0.17-0.15i | -0.31+0i | 0.23-0.02i | 0.16-0.03i | 0.19+0.29i | 0.18-0.23i |
| 25 | 0.1-0.01i | 0.02+0.19i | 0.17-0.32i | 0.05+0.33i | 0.03+0.2i | -0.34+0.19i |
| 26 | -0.08+0.27i | -0.16+0.06i | -0.14+0.37i | 0.17-0.22i | -0.04+0.39i | -0.27+0.21i |
| 27 | -0.19+0.14i | -0.02+0.42i | -0.06-0.13i | 0.06-0.27i | -0.33+0.01i | -0.07+0.33i |
| 28 | 0.24+0.19i | -0.35-0.23i | -0.26+0.13i | 0.03-0.32i | -0.11+0.1i | -0.06+0.22i |
| 29 | 0.08-0.06i | 0.14+0.19i | -0.04+0.25i | 0.29-0.15i | -0.13-0.32i | 0.1+0.37i |
| 30 | 0.17-0.27i | -0.21-0.05i | -0.04+0.29i | -0.2+0.12i | 0.15-0.01i | 0.02-0.18i |
| 31 | -0.06-0.3i | -0.31-0.14i | -0.17-0.14i | 0.18+0.27i | 0.38-0.21i | 0.18+0.04i |
| 32 | 0.24+0.11i | -0.29-0.24i | -0.04+0i | 0.31+0.04i | 0.16-0.35i | -0.09+0.35i |
| 33 | -0.32+0.15i | 0.23+0.02i | 0.38+0.09i | -0.16-0.07i | 0.01+0.12i | -0.37+0.13i |
| 34 | -0.11-0.15i | -0.28+0.16i | -0.3+0.29i | -0.24+0.1i | 0.21+0.34i | 0.25+0.13i |
| 35 | 0.38-0.07i | 0.14-0.18i | 0.34-0.16i | 0.13+0.17i | -0.04+0.37i | -0.19+0.14i |
| 36 | 0.18+0.15i | 0.15-0.36i | 0.09-0.34i | 0.18+0.08i | -0.07+0.37i | 0.3-0.19i |
| 37 | 0.06+0.05i | -0.03+0.12i | 0.24+0.02i | 0.03-0.04i | 0.19-0.13i | 0.39+0.1i |
| 38 | -0.2-0.31i | -0.08-0.01i | 0.08-0.22i | 0.4+0.13i | -0.36+0.19i | 0+0.27i |
| 39 | -0.28-0.01i | -0.31-0.04i | 0.35+0.03i | -0.2-0.05i | -0.15-0.05i | 0.21-0.08i |
| 40 | 0.23+0.08i | 0.03+0.25i | -0.17-0.3i | -0.14+0.13i | -0.3+0.13i | 0.26-0.01i |
| 41 | -0.2+0.19i | 0.26+0.07i | 0.14-0.02i | -0.11+0.41i | 0.19+0.34i | 0.07+0.24i |
| 42 | 0.1-0.32i | 0.16-0.23i | -0.24-0.2i | 0.29-0.19i | 0.16-0.1i | -0.36-0.02i |
| 43 | 0.18+0.32i | 0.22+0.13i | 0.06+0.19i | 0.15+0.4i | 0.08+0.18i | -0.23+0.28i |
| 44 | 0.21-0.12i | -0.1+0.28i | -0.06+0.42i | -0.29+0.08i | 0.02-0.18i | -0.1+0.37i |
| 45 | 0.28+0.04i | -0.25-0.02i | -0.04-0.36i | 0.23+0.04i | -0.35-0.02i | 0.23+0.11i |
| 46 | 0.24+0.26i | -0.01+0.05i | 0.21-0.34i | 0.31+0.09i | 0.13-0.28i | -0.23+0.08i |
| 47 | 0.16+0.22i | -0.07+0.1i | -0.01-0.31i | -0.36-0.13i | 0.16+0.35i | -0.33+0.06i |
| 48 | 0.29+0.05i | 0.25+0.1i | 0.15+0.26i | 0.13-0.37i | 0.11+0.23i | -0.21-0.22i |
| 49 | 0.21-0.1i | -0.1-0.22i | -0.1+0.24i | -0.33-0.01i | 0.03+0.28i | -0.16+0.05i |
| 50 | -0.18-0.12i | -0.31-0.04i | 0.02+0.31i | 0.21-0.25i | -0.01+0.24i | -0.02-0.05i |
| 51 | -0.29-0.09i | -0.09-0.05i | 0.03-0.14i | 0.1-0.26i | 0.05+0.31i | 0.14+0.3i |
| 52 | 0.27+0.15i | -0.19+0.04i | 0.02-0.14i | -0.19+0.29i | 0.27-0.04i | 0.2+0.15i |
| 53 | -0.12+0.12i | -0.28+0.06i | -0.08-0.2i | -0.02-0.02i | 0.19+0.24i | 0.03+0.33i |
| 54 | 0.2+0.14i | -0.13-0.16i | 0+0.36i | 0.2+0.31i | -0.01+0.27i | 0.18+0.29i |
| 55 | 0.19-0.08i | -0.14+0.32i | 0.42+0.09i | 0.3+0.04i | -0.17+0.32i | -0.05+0.01i |
| 56 | 0.18+0.22i | 0.32+0.13i | 0.19-0.26i | -0.08+0.12i | -0.05-0.32i | -0.19+0.13i |
| 57 | 0.03+0.27i | -0.17+0.24i | 0.32+0.08i | 0.19+0.3i | 0.1+0.1i | -0.09-0.2i |
| 58 | 0.28+0.2i | -0.29+0.09i | 0.33-0.05i | 0.03-0.33i | -0.07-0.23i | -0.32-0.01i |
| 59 | 0.17+0.07i | -0.2-0.05i | -0.01-0.35i | 0.23-0.05i | -0.2+0.03i | 0.09-0.33i |
| 60 | 0.21-0.02i | -0.39-0.02i | 0.03+0.39i | 0.17-0.15i | -0.07-0.09i | -0.14-0.23i |
| 61 | 0.18-0.23i | -0-0.25i | -0.15+0.23i | 0.07+0.31i | 0.29+0.24i | 0.11-0.27i |
| 62 | -0.31+0.03i | -0.32+0.08i | -0.2+0.13i | 0.13+0.25i | 0.05+0.24i | 0.32-0.08i |
| 63 | -0.2+0.21i | 0.16+0.24i | -0.35+0.12i | 0.03+0.19i | -0.1+0.4i | 0.27+0.05i |

The process of mapping feedback information to the PHY structure in the SFBCH is shown in Figure 6 . First, the SFBCH payload information bits a0a1a2⋯al−1 shall be encoded to 60 bits b0b1b2⋯b59using the linear block codes described in Section . When 6<l≤12, information bits a0a1a2⋯al−1 are encoded using the linear block code (60,l). When 12<l≤24, information bits a0a1a2⋯al−1are split into 2 parts: Part A consisting of a0a1a2⋯al2−1, and Part B consisting of al2al2+1al2+2⋯al−1. Part A shall be encoded to 30 bits b0b1b2⋯b29using linear block code (30,l2) and Part B shall be encoded to 30 bits b30b31b32⋯b59 using a linear block code (30,l−l2). The coded sequence b0b1b2⋯b59shall then be modulated to 30 symbols c0c1c2⋯c29using QPSK as described in Section TBD. Here, ci(i=0,1,⋯29)is formed by mapping coded bits b0b1b2⋯b29onto the in-phase component and code bits b30b31b32⋯b59 onto the quadrature component. The modulated symbols c0c1c2⋯c29 and pilot sequence p0p1are then mapped to the data subcarriers of the SFBCH FMTs as shown in Figure 6.

Figure 6: Mapping of information in the SFBCH

For each transmit antenna, power on the pilot subcarriers is boosted by 3dB over that of data subcarriers.

# < Note to editor: New subsection to be added below>

# Channel Coding in the SFBCH

The information bits in the SFBCH shall be  encoded using linear block codes. The codeword lengthshall not be larger than 60. The number of information bits Kcan vary from 6 to 12 bits, denoted by  where . The codeword shall be obtained as a linear combination of the K basis sequences denoted as where i=0,⋯,K−1 in Table 4. These linear block codes shall also used for bandwidth request channel (BRCH) described in Section 11.9.2.5.2.

Table 4: Basis sequences for (N,K) code

| N | S0,n | S1,n | S2,n | S3,n | S4,n | S5,n | S6,n | S7,n | S8,n | S9,n | S10,n | S11,n |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 1 |
| 2 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 |
| 3 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 |
| 4 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 |
| 5 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 0 |
| 6 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 1 |
| 7 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |
| 8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 |
| 9 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 10 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 11 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 0 |
| 12 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 |
| 13 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 |
| 14 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 |
| 15 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 1 |
| 16 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 0 |
| 17 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 |
| 18 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 19 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 |
| 20 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 1 |
| 21 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 1 |
| 22 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 1 |
| 23 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 |
| 24 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 |
| 25 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |
| 26 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 0 |
| 27 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |
| 28 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 0 |
| 29 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 |
| 30 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 |
| 31 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 |
| 32 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 1 |
| 33 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 34 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 |
| 35 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |
| 36 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 0 |
| 37 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |
| 38 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 0 |
| 39 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 |
| 40 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| 41 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |
| 42 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 0 |
| 43 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 |
| 44 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 1 |
| 45 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 0 |
| 46 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 1 | 0 |
| 47 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| 48 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 49 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 50 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 51 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 52 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 53 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 54 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 55 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 56 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| 57 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 58 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| N−1 | parity check bit |  |  |  |  |  |  |  |  |  |  |  |

If  denotes a codeword with length ofN, any component of the codeword shall be generated as: bn=i=0K−1ai⋅Si,nmod2, where n=0,1,2,⋯N−2. After the N−1components are generated, a parity check bit is appended to the codeword. The parity check bit is set to 1 when the number‘1’ in the codeword appears an odd number of times, otherwise it is set to ‘0’.