| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Handover and Load Balancing Rules for 16m |  |
| Date Submitted | 2009-01-05 |  |
| Source(s) | Hongseok Kim Xiangying Yang Muthaiah Venkatachalam Intel Corporation | Xiangying.yang@intel.com |
| Re: | IEEE 802.16m-08/052 Call for Comments and Contributions on Project 802.16m System Description Document (SDD) for Session 59. |  |
| Abstract | Provides SDD text to describe handover rules to be defined in neighbor advertisement message. |  |
| Purpose | Discussion and approval of the proposal into the IEEE 802.16m System Description Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Handover Rules and Load Balancing for 16m

Hongseok Kim, Xiangying Yang, Muthaiah Venkatachalam

- Intel Corp.

# Problem statement

In current HO frame, there may be cases that MS has to pick its own target BS. It is important for the network to define prioritization rules (in an efficient yet effective way) so that the MS could make the correct decision to avoid HO failure.

# Text proposal

Insert the following text to the end of Section 18.3.1.1 of SDD as follows:

-------------------------------  Text Start  ---------------------------------------------------

## 10.3.1.1 Network topology acquisition

The Neighbor Advertisement message may also contain parameters that defining prioritization rules for MS cell selection during handover. Examples of such parameters include cell load, cell types and weighting factors for these network parameters and measurement.

-------------------------------  Text End  ---------------------------------------------------