| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Consideration of Synchronization Channel over Partially Configured Carrier for Multi-Carrier Operation in IEEE 802.16m SDD |  |
| Date Submitted | 2009-01-05 |  |
| Source(s) | Pei-Kai Liao, Yu-Hao Chang, Chih-Yuan Lin, Ciou-Ping Wu and Paul Cheng MediaTek Inc. No.1, Dusing Road 1, Science-Based Industrial Park, Hsinchu, Taiwan 300, R.O.C. | pk.liao@mediatek.com yuhao.chang@mediatek.com paul.cheng@mediatek.com |
| Re: | SDD: Section 19.3 PHY Aspects of OFDMA Multi-carrier Operation In response to IEEE 802.16m-08/052 “Call for Comments on Project 802.16m SDD”. |  |
| Abstract | This contribution proposes some system design considerations for SCH over partially configured carrier in multi-carrier 802.16m systems. |  |
| Purpose | Propose to be discussed and adopted by TGm for the use in Project 802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Consideration of Synchronization Channel over Partially Configured Carrier for Multi-Carrier Operation in IEEE 802.16m SDD

Pei-Kai Liao, Yu-Hao Chang, Chih-Yuan Lin, Ciou-Ping Wu and Paul Cheng

MediaTek Inc.

- Introduction

SCH, which includes both Primary Synchronization Channel (P-SCH) and Secondary Synchronization Channel (S-SCH), provides an effective means for AMS to achieve DL synchronization in both time and frequency domains [1].  For multi-carrier operation in IEEE 802.16m system, the configuration of SCH in a fully configured carrier is the same as its single carrier counterpart, which is detailed in 11.7.2.1 [1].  In this case, AMS performs the same network entry procedure as that of the single carrier system on the fully configured carrier without any modification.  For a partially configured carrier, where network entry is not possible, the existence of SCH can provide improved network entry delay time and support link quality measurement. Longer periodicity of SCH appearance within one superframe could be increased to save the SCH overhead and reduce unnecessary computation power waste during network entry.  This contribution is to suggest the allocation of SCH over a partially configured carrier and propose a modified SDD text for SCH configuration in multi-carrier operation.

- Consideration of SCH in a partially configured carrier

Even though AMS cannot enter the 802.16m network via partially configured carriers as we mentioned earlier, it does not mean that we can completely remove all SCH sequences in partially configured carriers.  There are main reasons.

- Network entry delay time

Suppose that the periodicity of SCH appearance is TSCH and there are Np partially configured carriers. Without allocation of SCH in partially configured carrier, the maximal network entry delay time is Np×(TSCH+Tp), where Tp is the processing time, since it requires at least TSCH to confirm there is no such SCH in one carrier and switch to another for fully configured carrier. With allocation of SCH in partially configured carrier, it can be reduced to Np×(TSCH/2+Tp). Because Tp is much less than TSCH, the allocation of SCH can reduce at most one half delay time during the network entry.

- Link quality measurement

Without SCH in partially configured carrier, MS will lose a useful reference for link quality measurement such as AGC adjustment and other purposes. This is because SCH is a series of known data and its time domain power is relatively more stable than that of regular data OFDM symbols. With this good reference for link quality measurement, the link performance can be largely improved over partially configured carrier.

Because of above two reasons, it is suggested to have SCH in partially configured carrier.

However, should SCH configuration in a partially configured carrier the same as that in a full configured carrier? Since the main purpose of the design of a partially configured carrier is for data transmission, it is suggested to prolong the periodicity of SCH appearance in partially configured carrier. There are two reasons.

- Less overhead

With longer periodicity of SCH appearance, we can save the overhead for data transmission.

- No excessive SCH decoding

With longer periodicity of SCH appearance, we can avoid unnecessary excessive computation for SCH decoding. For example, if MS can not receive very well in some partially configured carrier, it may detect the existence of SCH but may not be able to decode it correctly. With the same periodicity as that in fully configured carrier, MS will continue trying to decode the next SCH until the decoding result is correct. However, with longer periodicity, the process can be terminated due to the lack of next SCH. Therefore, this configuration can guarantee MS won’t waste a lot of time on a specific partially configured carrier with poor link quality for unnecessary SCH decoding.

Based on the above two reasons, it is suggested to have longer periodicity of SCH appearance in a partially configured carrier than that in a fully configured one. How long it is can be decided after detailed SCH design becomes more clear.

- References

[1] IEEE 802.16m-08/003r6, IEEE 802.16m System Description Document, Dec. 12, 2008

IV. Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Editor’s Notes: add the following sentence into the “19.3.6.1 SCH” of TGm System Description Document]

The periodicity of the SCH in partially configured carrier is FFS.

-----------------------------------------------------------End of the Text---------------------------------------------------------