| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Codebook-Based Combined Precoding Across Multiple BSs |  |
| Date Submitted | 2009-1-5 |  |
| Source(s) | Yang Song, Xiaolong Zhu, Dong Li Alcatel Shanghai Bell | Voice: + 86-21-50554550-6352; Fax:+ 86-21-50554554 Mail to: Hongwei.yang@alcatel-sbell.com.cn |
| Re: | IEEE 802.16m-08/052 Call for Comments on Project 802.16m System Description Document (SDD) – Section 20.2.2.1.1 |  |
| Abstract | This contribution is to provide a feasible implementation method of codebook-based combined precoding cross multiple BSs with low complexity whenever multiple BSs intend to serve one or more common MSs in the downlink. |  |
| Purpose | Discussed and adopted in SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Codebook-Based Combined Precoding Across Multiple BSs

Yang Song, Xiaolong Zhu, Dong Li

Alcatel Shanghai Bell

# Introduction

Precoding is a promising closed-loop SU-MIMO technique that can be used to considerably enhance the link quality or throughput of a single user. On the other side, there are some scenarios that multiple base stations (BSs) transmit signals to a common mobile station (MS), usually an edge-user, e.g., macro diversity, and multi-cell coordination techniques such as collaborative MIMO, etc. In such cases, two or more BSs transmitting the same data to a MS at the same time interval such that diversity combining can be performed by the MS and meanwhile a large portion of interference can be mitigated. In the cases of multiple BSs transmitting signals to a MS, combined precoding across the coordinating BSs is employed to make good use of combined MIMO channel and further improve the user performance.

This contribution is to provide a feasible implementation method of codebook-based combined precoding cross multiple BSs with low complexity whenever multiple BSs intend to serve one or more common MSs in the downlink.

# Rationale

When multiple BSs are cooperatively transmitting signals to a common MS, e.g., macro diversity, multi-cell MIMO etc., the MS is able to estimate the channel matrix of the combined channel from multiple BSs if appropriate pilot pattern are designed. With the estimated combined channel matrix, the MS can obtain and feed back the preferred precoding matrix index (PMI) to the multiple BSs in coordination which keep the same precoding matrix codebook. With a duplicate of the user data stream, each BS in cooperation shall precode the data with the indicated combined precoding matrix or corresponding sub-matrix block of the combined precoding matrix.

Assume two BSs are serving a common MS at the same time-frequency block, each BS has NT transmit antennas, and the commonly served MS has NR receive antennas. The channel matrices from serving BS to the MS and coordinating BS are represented by H1 and H2. Codebook-based combined precoding across multiple BSs operates in the following procedures:

1. The commonly served MS measures the combined channel HC.

If collided pilots from two BSs are received, the common MS can only estimate the summation of H1 and H2. The combined channel matrix would be HC=H1+H2 with dimension of NR×NT. In the case that separate pilots received from two BSs, the common MS is able to estimate H1 and H2 from its serving BS and coordinating BS respectively. Thus, the combined channel would be a NR×2NT matrix .

What’s more, to reduce the dimension of combined channel matrix or realize another precoding step such as SDMA, an inner precoding as regular single cell SU-MIMO/MU-MIMO precoding within a single BS can be conducted. The term “inner precoding” is used in the context to separate with “combined precoding”. If separate pilots from the two BSs are received, based on H1 and H2 the MS can select the best precoding matrices W1I and W2I for its serving and coordinating BSs respectively. If uplink sounding signal from the common MS can be received by both BSs in cooperation in TDD system, either one BS can calculate the inner precoding matrix by itself. The dimension of W1I and W2I are NT×R1I and NT×R2I, where R1I and R2I are the ranks of inner precoding matrix of serving and coordinating BS. Thus, the combined channel can be written as  with dimension of NR×(R1I+R2I) or  with dimension of NR×RI (RI=R1I=R2I) depending on the pilot pattern from the BSs in cooperation.

2. The common MS selects its preferred combined precoding matrix  and rank RC based on HC. This operation is the same as SU-MIMO precoding matrix selection.

For combined channel HC=H1+H2, the dimension of combined precodng matrix  shall be NT×RC; for ,  shall be a 2NT×RC dimensional matrix. If inner precoding is applied, the dimension of  for  shall be RI×RC, and the dimension for  shall be (R1I+R2I)×RC.

Common or different precoding codebooks can be used by inner precoding and combined precoding.

3. The common MS feeds back the PMI and rank of combined precoding matrix to the BSs in coordinating.

If the MS only has an uplink connection with the serving BS, it feeds back the PMI and rank of combined precoding to the serving BS, by which forwards the PMI and rank to the coordinating BS. By doing this, only a few bits are transferred over backhaul network. If the MS has uplink connections with both BSs, the MS can feed back PMI and rank to both BSs. The PMI and rank of inner precoding may also be fed back at the same time.

4. With the data stream for the common user, either BS in cooperation precodes the data with the indicated combined precoding matrix or corresponding sub-matrix block of the combined precoding matrix, followed by an inner precoding if needed.

For combined channel HC=H1+H2, precoding processing at the serving BS and the coordinating BS can both be represented by WCd, where d is the data vector for the common MS with dimension of RC×1; if inner precoding is applied, i.e., HC=H1W1I+H2W2I, precoding processing at the serving BS can be written as W1IWCd, and precoding at the coordinating BS is W2IWCd. For combined channel HC=[H1,H2], the serving BS and the coordinating BS will apply W1Cd and W2Cd respectively, where W1C is the upper NT-row sub-matrix of WC and W2C is the lower NT-row sub-matrix of WC, i.e., WC=[W1C,W2C]T; if inner precoding is applied, i.e., HC=[H1W1I,H2W2I], precoding at the serving BS and the coordinating BS can be written as W1IW1Cd and W2IW2Cd respectively, also WC=[W1C,W2C]T, whereW1C is the upper R1I-row sub-matrix of WC and W2C is the lower R2I-row sub-matrix of WC.

Figure 1 shows an illustration of above steps for the PMI forwarding case.

Figure 1

The precoding block diagrams of summated and separated combined channels are plotted in Figure 2. If no inner precoding is applied, the inner precoding blocks can be skipped or all the inner precoding matrices can be regarded as identity matrices.

a) Precoding of summated combined channel			b) Precoding of separated combined channel

Figure 2  Precoding block diagrams

Received signal for summated combined channel can be written as r=(H1W1I+H2W2I)WCd+n, where n is the interference plus noise vector. For separated combined channel, received signal is r=[H1W1I,H2W2I]WCd+n=[H1W1I,H2W2I]W1C,W2CTd+n=(H1W1IW1C+H2W2IW2C)d+n.

Codebook-based combined precoding across multiple BSs has the advantages of user performance improvement, low implementation complexity, low requirement on the backhaul. It can be regarded as an enhancement of macro-diversity, multi-cell MIMO and applied to both FDD and TDD systems.

# Proposed text to SDD

[add the following text in the end of Section 20.2.2.1.1 in page 154, Line 18]

When multiple ABSs are cooperatively transmitting signals to a common AMS, e.g., macro diversity, multi-ABS MIMO etc., the AMS is able to estimate the channel matrix of the combined channel from multiple ABSs, select and feed back the preferred PMI to the multiple ABSs. Each ABS in cooperation shall precode the common user data with the indicated combined precoding matrix or corresponding sub-matrix block of the combined precoding matrix.