| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Rank-1 precoder for uplink open-loop SU-MIMO |  |
| Date Submitted | 2009-1-5 |  |
| Source(s) | Xiaolong ZHU, Hongwei Yang, Dong LI, Dong LI,Keying WU Alcatel Shanghai Bell | Voice: + 86-21-50554550; Fax:+ 86-21-50554554 Mail to: Hongwei.yang@alcatel-sbell.com.cn |
| Re: | IEEE 802.16m-08/052 Call for Comments on Project 802.16m System Description Document (SDD): 11.12 UL MIMO Transmission Scheme |  |
| Abstract | This contribution addresses the open issues about the rank-1 precoder for uplink open-loop SU-MIMO. |  |
| Purpose | Adopt the proposed text into SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Rank-1 precoder for Uplink Open-Loop SU-MIMO

Xiaolong ZHU, Hongwei YANG, Dong LI, Keying WU

Alcatel Shanghai Bell

# 1 Introduction

In the current 16m SDD [1], precoder is accepted as an element of uplink open-loop (OL) single-user (SU-) MIMO transmit diversity and spatial multiplexing schemes. More specifically, in Section 11.12.2.1.1 OL SU-MIMO it is indicated that on a given subcarrier k, the precoding matrix P can be defined using the following equation:

P(k) = W(k), Equation 12

W(k) is an NT x Ns matrix, where NT is the number of transmit antennas and Ns is the number of streams. The matrix W(k) is selected from a predefined unitary codebook, and changes every u subcarriers and/or v OFDM symbols. The matrix W(k) can be identity matrix for 2Tx rate-2 and 4Tx rate-4. A codebook is a unitary codebook if each of its matrices consists of columns of a unitary matrix. It is also stated in the current SDD that “The detailed unitary codebook, the parameters u and v are FFS.”

This contribution addresses these open issues about the rank-1 precoder.

2 Rank-1 precoder

For NT = 2, the unitary codebook consists of all or a subset of the following matrices

,,

where D is the 2x2 identity matrix or an implementation-specific diagonal unitary matrix. W1 allows the implementation of cyclic delay diversity (CDD) or various beamforming schemes. W2 and W3 allow the implementation of antenna selection or its variants like switching diversity scheme presented in [2].

For NT = 4, the unitary codebook consists of all or a subset of the following matrices

,,,,

where D is the 4x4 identity matrix or an implementation-specific diagonal unitary matrix. W1 allows the implementation of cyclic delay diversity (CDD) or various beamforming schemes. W2 ~ W5 allow the implementation of antenna selection or its variants like switching diversity scheme presented in [2].

Dedicated pilots are specified for the uplink transmission, which indicates that the pilot subcarriers are precoded using the same precoder as the data subcarriers. To avoid the adverse impact of precoder on channel estimation at the receiver side, the parameters are set as u = L1 x Nsc and v = L2 x Nsym, where L1 and L2 are positive integers, Nsc and Nsym are the number of subcarriers and the number of OFDM symbols inside one uplink tile, respectively. Take the 4-by-6 tile structure as shown in Figure 46 [1] and L1 = L2 = 1 as an example, then u = 4 subcarriers and v = 6 symbols.

To allow implementation-specific precoder in the open-loop case, the OL SU-MIMO may use a different unitary codebook from the CL SU-MIMO.

# 3 Proposed SDD text

------------------------------------------- Start of Proposed SDD Text ------------------------------------------------------

11.12.2.1.1 Open-loop SU-MIMO

On a given subcarrier k, the precoding matrix P can be defined using the following equation:

P(k) = W(k), Equation 12

W(k) is an NT x Ns matrix, where NT is the number of transmit antennas and Ns is the number f streams. The matrix W(k) is selected from a predefined unitary codebook, and changes every u subcarriers and/or v OFDM symbols. The matrix W(k) can be identity matrix for 2Tx rate-2 and 4Tx rate-4. A codebook is a unitary codebook if each of its matrices consists of columns of a unitary matrix. [The detailed unitary codebook, and the parameter u are FFS.]

For NT = 2 and M = 1, the codebook for rank-1 precoder is comprised of all or a subset of the following matrices

,,

where D is the 2x2 identity matrix or an implementation-specific diagonal unitary matrix.

For NT = 4 and M = 1, the codebook of rank-1 precoder is comprised of all or a subset of the following matrices

,,,,

where D is the 4x4 identity matrix or an implementation-specific diagonal unitary matrix.

The parameters are set as u = L1 x Nsc and v = L2 x Nsym, where L1 and L2 are positive integers, Nsc and Nsym are the number of subcarriers and the number of OFDM symbols inside one uplink tile, respectively. Take the 4-by-6 tile and L1 = L2 = 1 as an example, then u = 4 and v = 6.

The OL SU-MIMO and CL SU-MIMO may different unitary codebooks.

------------------------------------------- End of Proposed SDD Text ------------------------------------------------------

References

- IEEE C802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document.”

- IEEE C802.16m-MIMO-08/092, “Comments and proposed text changes on the final SDD draft on the UL MIMO schemes.”