| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on Bit Rearrangement/Constellation Rearrangement |  |
| Date Submitted | 2009-01-05 |  |
| Source(s) | Isamu Yoshii Panasonic Corporation Lei Huang Panasonic Singapore Laboratories | Voice: +81-50-3687-6548 E-mail: yoshii.isamu@jp.panasonic.com |
| Re: | 802.16m-08/052,Call for comments on 802.16m SDD (802.16m-08/003r6), Section 11.13.2.2 |  |
| Abstract | Comment on IEEE 802.16m System Description Document |  |
| Purpose | For discussion and approval by IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on Bit Rearrangement/Constellation Rearrangement

Isamu Yoshii

Panasonic Corporation

Lei Huang

Panasonic Singapore Laboratories

- Introduction

In #58, the following two comments are agreed and the SDD Editor considered that it does not make sense to merge both, and then decided to take cmt 510.

-----------------------------------------------Agreed Text-----------------------------------------------

cmt 510

11.13.2.2 Bit re-arrangement

Bit re-arrangement (BitRe) is supported in 802.16m. Bit re-arrangement includes a bit-level interleaver and an inverter. For each transmitted burst, the BitRe-version is selected by the transmission number of this burst. The specific bit-level interleaver and inverter mechanism is FFS.

cmt 511

11.13.2.2 Constellation re-arrangement

Constellation re-arrangement (Co-Re) is supported in 802.16m. All the QAM symbols in the same HARQ re-transmission use same Co-Re version. The specific selection mechanism is FFS.

---------------------------------------------------------------------------------------------------------------

The description of cmt 510 includes a bit-level interleaver and an inverter across multiple symbols [1]. The scheme which is interleaved across multiple symbols is different from the concept of constellation rearrangement (Co-Re). The Co-Re can be expressed by a bit-level interleaver and an inverter with a QAM symbol, but not across multiple symbols. The scheme across multiple symbols can not get the optimum gain from bit re-arrangement. Especially, in AWGN case, the bit-interleaving across multiple symbols does not bring any gain, but reduce the combining gain by non-optimum bit rearrangement pattern. In case of fading condition, it might bring some diversity gain by a bit interleaver, but its diversity gain can also be brought by a normal symbol interleaver.

- LLS Comparisons

Figure 1 show the performace comparisons between “the normal CoRe scheme[2]” and “the scheme of bit-level interleaving and inversion across multiple symbols[1]” in case of AWGN channel and fading channel, respectively. The detail of simulation parameter is shown at Table 1. (In Fig. 1, CoRe does not have any channel interleaver. In Fig.2, CoRe(with SymIL) uses a symbol interleaver as a channel interleaver.)

From Figures 1 and Figure 2, we can observe that “the normal Co-Re scheme” outperforms “the scheme of bit interleaving and inversion across multiple symbols” in case of AWGN channel. Furthermore, “the normal Co-Re with a symbol interleaver” outperforms “the scheme of bit-level interleaving and inversion across multiple symbols” in case of fading channel. (All interleaver pattern for bit and symbol is random and inversion pattern is also random.)

Table 1 :  Simulation parameter

Figure 1  Performance comparison (AWGN)

Figure 2 Performance Comparison (TU 6path)

- Conclusion

In this contribution, it has been shown that “the normal Co-Re scheme”, i.e., a bit-level interleaver and an inverter within a QAM symbol, outperforms “the scheme of bit-level interleaving and inversion across multiple QAM symbols”.

---------------------------- Begin Proposed Text -------------------------------------------------------------------------

11.13.2.2 Constellation re-arrangement

Constellation re-arrangement (Co-Re) is supported in 802.16m. Constellation re-arrangement can be expressed by a bit-level interleaver and an inverter within a QAM symbol.  All the QAM symbols in the same subpacket use same Co-Re version. The specific selection mechanism is FFS.

---------------------------End of Text Proposal ---------------------------------------------------------------------------

* "Subpacket" is the packet after concatenation/collection of encoded FEC block.

Reference

[1] IEEE C802.16m-08/301r2, Enhancing CTC HARQ performance by bit rearrangement

[2] IEEE C802.16m-08/771r1, Enhanced HARQ scheme with Signal Constellation Rearrangement

[3] IEEE 802.16m-08/003r6, IEEE 802.16m System Description Document [draft]