| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | LDPC-Convolutional Codes for E-MBS FEC |  |
| Date Submitted | 2009-01-05 |  |
| Source(s) | Takaaki KISHIGAMI, Yutaka MURAKAMI, Isamu YOSHII Panasonic Corporation | Voice: +81-46-840-5123 E-mail: kishigami.takaaki@jp.panasonic.com yoshii.isamu@jp.panasonic.com |
| Re: | IEEE802.16m-08/052 – Call for Contributions on Project 802.16m System Description Document(SDD), Section 14.5.2.1 |  |
| Abstract | We propose to support LDPC codes for E-MBS FEC with large FEC block size. |  |
| Purpose | For 802.16m discussion and adoption. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

LDPC- Convolutional Codes for E-MBS FEC

Takaaki KISHIGAMI, Yutaka MURAKAMI, Isamu YOSHII

Panasonic Corporation

# Introduction

LDPC codes enable not only to achieve as good performance as Turbo codes, but also to achieve high-speed decoding, thanks to parallel processing with message-passing decoding. We have already presented the contributions on LDPC-Convolutional Codes (LDPC-CCs) [1][2] for the 16m FEC .

In current SDD [3], FEC with large block size for E-MBS transmission should be supported. So, we compare the performance between CTC and LDPC-CC in the case of the large FEC block size, and show that LDPC-CC performance in higher coding rate is better than CTC. Hence, we proposed to support LDPC codes for E-MBS FEC with large FEC block.

# Proposed LDPC-CC for E-MBS FEC

# 2.1 FEC Encoding based on LDPC-CC

The FEC scheme block diagram is shown in Figure 1.

The FEC encoder is composed of LDPC-CC Encoder, where payload data length is M bits and code bit length is z bits.

M

Information

bits

z

Code bits

to

Modulator

LDPC-CC Encoder

Figure 1 FEC encoder.

Payload data is coded by LDPC-CC encoder. LDPC-CC encoder of Figure 1 is shown in Figure 2Error: Reference source not found.

Constituent

Encoder

M

Systematic

bits

z-M

Parity bits

M

Information

bits

Figure 2 Structure of LDPC Convolutional Encoder

An example of LDPC-CC encoder with a coding rate R=1/2 is shown in Figure 3[4]. The LDPC-CC encoder is a time-varying systematic convolutional encoder. The LDPC-CC encoder consists of shift registers for information bit, shift registers for parity bit, a weight controller and an adder and weight multipliers. The complexity of LDPC-CC encoding is proportional to the number of row weights of the parity check matrix H defined in the next subclause.

Figure 3:  An example structure of LDPC-CC encoder.

2.2 Polynomials for encoding LDPC-CCs

Polynomials of LDPC-CCs with R=(N-1)/N are described in the following.

LDPC-CC Encoder:

- Payload input M bits then outputs are “Systematic Bits (M bits)” and “Parity Bits (also M−z bits)”. Note that Coding rate in LDPC-CCs is (N−1)/N.

Encoding process in LDPC-CCs is as follows:

Information bits (M bits) are streamed to two blocks. One is for systematic bits (M bits) and another is for a constituent encoder input.

A constituent encoder performs coding process for “Information Bits (M bits)”, then outputs “Parity Bits (M−z bits)”. The encoder of LDPC-CCs outputs “Code Bits (a pair bits)”, sequentially {d1,0, d2,0, …, dN-1,0, p0}, {d1,1, d2,1,…, dN-1,1, p1}, {d1,2, d2,2,…, dN-1,2, p2}, ….

LDPC-CCs can be defined by an infinite parity check matrix, however their parity check matrix is periodically time-varying. A parity check matrix is expressed as H and we describe LDPC-CCs by using the transposed form HT which is well-known to syndrome former. An infinite parity check matrix of LDPC-CCs with coding rate R=b/c (b<c) is expressed as:

--- (1)

where HiT (t) (i=0,1,···,ms) are c(c−b) periodically time-varying sub-matrices and ms is a memory. HiT (t)=HiT (t+ Ts) are satisfied for all i and t when Ts is a period.

LDPC-CCs with coding rate R=(n−1)/n based on the check polynomial are used. Information bits of X1, X2,···, XN-1 and parity bits P at time j are expressed as X1,j, X2,j,···, XN-1,j and Pj, respectively, and a vector uj is expressed as uj=(X1,j,X2,j,···,XN-1,j, Pj). In addition, polynomials for information bits of X1, X2,···, XN-1 are expressed as X1(D), X2(D),···, XN-1(D), respectively. A parity check polynomial of equation (2) is considered.

--- (2)

where ap,q (p=1,2,···,N−1 ; q=1,2,···,rp) are natural numbers, and ap,1ap,2···ap,rp is satisfied. Three pieces of different parity check polynomial based on equation (2) for LDPC-CCs with R=(n−1)/n are prepared and expressed as:

--- (3)

where i=0,1,···,m−1. For X1,j,X2,j,···,Xn-1,j and Pj at time j, a parity check polynomial of equation (4) is satisfied.

(k=j mod 3)      --- (4)

Regarding computational complexity, we propose rate compatible LDPC-CCs. Supported code rate in rate compatible LDPC-CCs are shown in Table 1Error: Reference source not found.

Table 1 Supported code Rates of rate compatible LDPC-CCs.

| Supported Code Rate | 1/3, 1/2, 2/3, 3/4, 4/5,5/6 |
| --- | --- |

Note that Xk(D) (k=1,2,…,N−1) represents ”Systematic Bits” and P(D) represents “Parity Bits”. The encoder of LDPC-CCs is time-varying convolutional encoder with period 3 and the maximum memory length approx. 400, which uses equations (5) and (6) depending on time slot.

An encoder of LDPC-CCs can be optionally composed, performing equation (5).

--- (5)

Initial sate of LDPC-CC Encoder is all zero state which means:

,         --- (6)

LDPC-CCs support “variable length M information bits coding” by same encoder composition, and it also supports plural memory length.

In the mean time, we described LDPC-CCs with coding rate R=(N-1)/N (i.e. coding rate R=1/2, 2/3, 3/4, 4/5) in the above. However we support LDPC-CCs with coding rate R=1/3. In this proposal, we achieve coding rate R=1/3 by adopting a shortening scheme for LDPC-CCs with coding rate R=1/2 [5].

Encoding Termination:

Termination via Zero-tailing will lead to a stable LDPC-CC Encoder state.

Zero-tailing can be done by LDPC-CC encoder by adding Qz-t bits with value set to “0”. Since Tail-bits are known at receiver, Tail-bits are excluded from Systematic Bits transmission but Parity Bits resulted from Tail-bit coding can be transmitted.

# Performance Evaluation

An example of performance in the proposed rate compatible LDPC-CCs is shown. Figure 4 shows BLER (Block error rate) performance comparing with CTC [6] under AWGN channels, BPSK modulation. Note that Normalized BP decoding [7] used as the message passing decoding and information bit length is 6144 bits. We can confirm the proposed LDPC-CC performance in lower coding rate is as good as CTC and that in higher coding rate, LDPC-CC performance is better than CTC.

Fig. 4:  Performance comparison between LDPC-CC and CTC.

(Parameters in LDPC-CCs evaluation: Iteration number=50 [except for coding rate R=1/3],

Number of the termination bits Qz-t=400bits)

# Conclusion

We compared the performance between CTC and LDPC-CC in the case of large FEC block size, and showed that the proposed LDPC-CC performance in higher coding rate is better than CTC. Hence, we proposed to support LDPC codes for E-MBS FEC with large FEC block size.

# Text Proposal for SDD

-------------------------------  Text Start  -------------------------------

14.5 E-MBS Transmission on Dedicated Broadcast Carriers

14.5.2 E-MBS Dedicated Carrier

14.5.2.1 Channel coding

Section 14.5.2.1 , page 136, line 6

- Replace the following text

“FEC with large block size should be supported in E-MBS. LDPC code support is FFS.”

with

“FEC with large block size shall be supported in E-MBS. LDPC codes shall be supported.”

-------------------------------  Text End  -------------------------------

# References

[1] IEEE C802.16m-08/74r1, “LDPC-Convolutional Codes for IEEE 802.16m FEC Scheme”.
[2] IEEE C802.16m-HARQ-08/51, “Rate Compatible LDPC-Convolutional Codes for IEEE 802.16m FEC”.
[3] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”
[4]Alberto Jimenez Felström and Kamil Sh. Zigangirov, "Time-Varying Periodic Convolutional Codes with Low - Density Parity - Check Matrix," IEEE Transactions on Information Theory, vol.45, no.6, pp.2181-2191, Sep. 1999. 
[5]U. Dammer, E. Naroska, S. Schemermbeck, and Schwiegelshohn, “A data puncturing IR-scheme for typ-II hybrid ARQ protocols using LDPC codes,” Proc. of IEEE Globecom 2004, pp.3012-3016, Nov. 2004.
[6]3GPP TSG RAN WG1#44bis, R1-061050, “EUTRA FEC Enhancement”.
[7]J. Chen, and M. P. C. Fossorier, “Near optimum universal belief propagation based decoding of low-density parity check codes,” IEEE Transactions on communications, vol.50, no.3, pp.406-414, March 2002.