| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Use of 1 Bit EKS Field |  |
| Date Submitted | 2009-01-05 |  |
| Source(s) | GeneBeck Hahn, KiSeon Ryu, JeongKi Kim and Ronny YongHo Kim LG Electronics LG R&D Complex, 533 Hogye-1dong, Dongan-gu, Anyang, 431-749, Korea | Voice: +82-31-450-7188 E-mail: gbhan@lge.com, ksryu@lge.com, chegal@lge.com and ronnykim@lge.com |
| Re: | IEEE 802.16m-08/005: Call for Contributions on Project 802.16m System Description Document (SDD) Specific topic "Key Update Method for Idle Mode MS" |  |
| Abstract | This contribution describes a use of 1 bit EKS fields in 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Use of 1 Bit EKS Field

Gene Beck Hahn, Ki Seon Ryu, JeongKi Kim and Ronny Yong Ho Kim

LG Electronics

# Problem Definition

According to 802.16m SRD, 802.16m shall include security functions for enhanced key management and the impact of security procedures on the performance of system functions shall be minimized [2].

In legacy systems, 2 bit EKS is included in GMH to identify the index of TEK and IV (Initialization Vector). This is to identify more than two sets of TEK materials. However, MS and BS maintain just two sets of active TEKs per SAID, corresponding two successive generations of keying material. The two generations of TEKs shall have overlapping lifetimes. Each TEK becomes active halfway through the lifetime of its predecessor and expires halfway through the lifetime of its successor. Once a TEK’s lifetime expires, the TEK becomes inactive and shall no longer be used.

In the legacy system, 2 bit EKS is only useful in case the packet encrypted using old key (TEK 1) is arrived at a BS after updating new TEK materials (TEK2, TEK3) at BS. However, it will never happen that the packet encrypted using TEK1 arrives after TEK1 lifetime at BS is expired. This stems from the fact that TEK1 grace time is very long (e.g., in legacy system, minimum (5 min), default (1 hour), max (3.5 days)). Hence, 1 bit EKS is enough to identify two active sets of TEK materials [1].

# Use of 1 bit EKS field

Figure 1 shows the flow of TEK management at AMS and ABS. The right-hand side illustrates the BS’s management of an SA’s TEKs and the left-hand side presents the MS’s management of an SA’s TEK respectively. The key reply messages sent by BS contain TEK parameters for the two active TEKs. As we can see in figure 1, TEK grace time is activated before the current TEK is expired and new TEK is updated. That is, through operation of MS’s TEK state machines, MS shall request a new set of TEK material a configurable amount of time, the TEK grace time, before the MS’s latest TEK is scheduled to expire.

In detail, during the TEK 1 active time, the TEK 1 grace time starts immediately when a new TEK request message is sent from MS to BS. When MS receives the updated keying materials, i.e., the TEK 1, TEK 2 pairs through key reply message, it uses TEK 2. However, in this point, the TEK 1 grace time is not expired. When BS transmits the key reply containing TEK 1, TEK 2 pairs, the TEK 2 is already activated and TEK 1 is no longer used at BS. That is, the active lifetime of TEK 1 is already expired. On the MS side, the active lifetime of TEK 0 is already expired long before key request message for TEK 2 is sent to BS. Hence, both MS and BS can maintain maximally two sets of active TEK materials.

Since the TEK grace time is very long, it will never happen that MS and BS shall manage three sets of TEK due to HARQ retransmission. That is, after new TEKs (TEK 1, TEK 2) are updated at BS, the packet encrypted using TEK 0 will not be transmitted to BS. Hence, 1 bit is enough to identify the two active sets of TEK materials in 802.16m.

Figure 1: TEK Management in ABS and AMS

# References

[1] IEEE 802.16e Rev2/D2, Part 16: Air interface for Broadband Wireless Access Systems, December 2007

[2] IEEE 802.16m-07/002r2150, Draft IEEE 802.16m Requirements, 2007-06-08

Text Proposal for IEEE 802.16m SDD

============================= Start of Proposed Text =============================

# 10.6.3.3 Key Usage

The TEK usage does not differ from ‘Reference System’.

In encryption, used KEY_COUNT shall be identified by the receiver (AMS or ABS). EKS field carries the 2-bit 1-bit key sequence of associated TEK. Alternative EKS design and usage is FFS.

# 10.12.1.1 Generic MAC Header

Figure 22 Generic MAC header format

- HT (Header Type) : Indicates the type of the header. This field is TBD bits long.

- EH (Extended Header Presence Indicator) : When set to ‘1’, this field indicates that an

Extended Header is present following this GMH.

- FlowID (Flow Identifier) : This field indicates the flow that is addressed. This field is 4 bits long.

- EKS (2 1 bits) : Encryption Key Sequence

- Length : Length of the payload. This field is 11 bits long.

- FPI : The indication of FPI is FFS.

============================= End of Proposed Text =============================