| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on SDD text for Management/Multiplexing MAC header |  |
| Date Submitted | 2009-01-05 |  |
| Source(s) | Eun-jong Lee, Doo-hyun Sung and Jeongki Kim LG Electronics | mehappy3@lge.com, dh.sung@lge.com, chegal@lge.com |
| Re: | IEEE 802.16m-08/052: Call for Comments on Project 802.16m System Description Document (SDD), Section 10.12 MAC PDU formats |  |
| Abstract | This contribution proposes modified MAC header formats in IEEE 802.16m SDD |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on SDD text for Management/Multiplexing MAC header

Eun-jong Lee, Doo-hyun Sung and Jeongki Kim

LG Electronics

# Introduction

In legacy system, MAC management message is transmitted with generic MAC header (GMH). However, not all fields in GMH are used to transmit MAC management messages. For example, in case of management message transmission, a functionality of flow id in GMH is only to indicate whether the payload is for management flow or not. Therefore, in management header design, by defining the header type optimized for management messages, the size of MMH can be reduced shorter than GMH. In addition, the format of management MAC header can be reused in multiplexing MAC header because optimized fields of management message header should also be contained in multiplexing MAC header. Therefore, we propose to use the same header format for both management MAC header and multiplexing MAC header. We also propose the multiplexing extended header (MEH) to support multiplexing multiple MAC SDUs from different flows belonging to the same MS.

The proposed management/multiplexing MAC header is designed with following considerations.

- A HT is used to indicate whether the header is for management/multiplexing messages. This field is 2 bits long to distinguish four header types; GMH, CMH, SMH and MMH.

- For MAC management messages, fragmentation/packing extended header may exist to support the packing/fragmentation of management messages.

- For multiplexing MAC header, the EH field should exist to add the multiplexing extended header.

- An EC should support the selective encryption for management/multiplexing MAC PDU.

- The length field in MAC header should be large enough to transmit large payload.

# Text Proposal

============================ Start of Proposed Text ==================================

Insert the following chapter into the chapter 10.12.1

10.12.1 MAC header formats

10.12.1.1 Generic MAC Header

10.12.1.2 Compact Header

10.12.1.3 Multicast/Broadcast MAC header

10.12.1.4 Management/Multiplexing MAC Header

Figure xx Management/Multiplexing MAC Header Format

- HT (Header Type): Indicates the type of the header. This field is 2 bits long.

- EH (Extended Header Presence Indicator): When set to ‘1”, this field indicates that an Extended Header is present following this MAC header. If this MAC header is used for Multiplexing MAC header, this field should be set and the multiplexing extended header will be present following this header.

- EC (Encryption Control): Need to support the selective encryption for management/multiplexing messages.

- EKS (1 bit): Encryption Key Sequence.

- Length: Length of the payload. This field is 11 bits long.

10.12.1.5 Signaling MAC Header

10.12.2 Extended header

The inclusion of extended header is indicated by EH indicator bit in MAC Header. The EH format is shown in Figure xx and will be used unless specified otherwise.

Figure xx Extended Header Format

- Last: When the “Last” bit is set, another extended header will follow the current extended header. If this bit is not set this extended header is the last one.

- Type: indicates the type of extended header. The length is TBD.

- Body Contents: Type-dependent contents.

10.12.2.1 Fragmentation and packing extended header

10.12.2.2 Multiplexing extended header

Multiple MAC SDUs and/or SDU fragments from different unicast flows belonging to the same MS can be multiplexed into a single MAC PDU by multiplexing extended header.

Figure xx Multiplexing Extended Header Format

- FlowID (Flow Identifier): This field indicates the flow that is addressed. This field is 4 bits long.

- EH (Extended Header Presence Indicator): When set to ‘1”, this field indicates that an Extended Header for this FlowID is present. The EHs for multiple FlowIDs will follow the current header in order of FlowIDs presented in multiplexing extended header.

- END: When the “END” bit is set, information of another flow (FlowID, EH, END, Length) will follow current Length field. If this bit is not set, last Length field is omitted.

- Length: Length of payload for each Flow ID. This field is 11 bits long. If END bit is not set, last Length field is omitted.

============================== End of Proposed Text ================================